BOREAS HYD-08 Throughfall Data

Summary

The BOREAS HYD-08 team made measurements of surface hydrological processes at the SSA (1996) and NSA OBS (1994) Tower Flux sites, supporting its research into point hydrological processes and the spatial variation of these processes.

These data were collected during the 1994 and 1996 field campaigns.  Data collected may be useful in characterizing canopy interception, drip, throughfall, moss interception, drainage, evaporation, and capacity during the growing season at daily temporal resolution.  This particular data set contains the measurements of throughfall, which is the amount of precipitation that fell through the canopy. A nested spatial sampling plan was implemented to determine  spatial variations of the measured hydrological processes and ultimately the impact of these variations on modeled carbon and water budgets.  These data are stored in ASCII text files.

Table of Contents

*  1 Data Set Overview

*  2 Investigator(s)

*  3 Theory of Measurements

*  4 Equipment

*  5 Data Acquisition Methods

*  6 Observations

*  7 Data Description

*  8 Data Organization

*  9 Data Manipulations

*  10 Errors

*  11 Notes

*  12 Application of the Data Set

*  13 Future Modifications and Plans

*  14 Software

*  15 Data Access

*  16 Output Products and Availability

*  17 References

*  18 Glossary of Terms

*  19 List of Acronyms

*  20 Document Information

1. Data Set Overview

1.1 Data Set Identification

BOREAS HYD-08 Throughfall Data

1.2 Data Set Introduction

The BOReal Ecosystem-Atmosphere Study Hydrology (HYD)-08 team made measurements of surface hydrological processes at the Southern Study Area (SSA) (1996) and Northern Study Area (NSA) Old Black Spruce (OBS) (1994) Tower Flux sites, supporting its research into point hydrological processes and the spatial variation of these processes.

These data were collected during the 1994 and 1996 field campaigns.  Data collected may be useful in characterizing canopy interception, drip, throughfall, moss interception, drainage, evaporation, and capacity during the growing season at daily temporal resolution.  This particular data set contains the measurements of throughfall, which is the amount of precipitation that fell through the canopy. A nested spatial sampling plan was implemented to determine spatial variations of the measured hydrological processes and ultimately the impact of these variations on modeled carbon and water budgets.  These data are stored in American Standard Code for Information Interchange (ASCII) text files.

1.3 Objective/Purpose

The objective of these data sets was to quantify the magnitude and spatial variation of storages and fluxes of water at the moss surface and during precipitation events in selected Picea Mariana stands.  The following parameters were measured to permit future parameterization of flux models: throughfall, stemflow, moss water storage, and gross precipitation.  A nested spatial sampling plan was implemented to characterize the length scales of variations of the measured parameters for future use in modeling studies and for comparison with measurements at the black spruce flux towers located in the study sites, SSA-OBS and NSA-OBS.

1.4 Summary of Parameters

Precipitation that has fallen through the canopy (after storm events)

1.5 Discussion

Hydrological processes such as canopy evaporation and moss storage and evaporation may play a significant role in controlling water fluxes during the growing season in boreal wetlands. Canopy interception and moss storages and evaporation were measured using mass balance methods (throughfall catch buckets and lysimeters) to give a quantitative estimate of these processes for sparse black spruce stands.  More importantly, the spatial sampling scheme allowed quantification of the expected variation of these processes within the footprint of a colocated flux measurement tower.  This will allow consideration of the sub-tower-footprint controls on vapor fluxes that the tower is measuring.  In addition, these data sets will be useful in parameterizing flux models for the site targeted as well as determining the typical variation in fine scale processes that the models may have to account for when scaling to watershed and regional extents.

1.6 Related Data Sets

BOREAS HYD-08 1996 Gravimetric Moss Moisture Data

BOREAS HYD-08 1994 Gravimetric Moss Moisture Data

BOREAS HYD-08 Gross Precipitation Data

2. Investigator(s)

2.1 Investigator(s) Name and Title

Dr. Lawrence Band

University of Toronto

Department of Geography

Toronto, Ontario

2.2 Title of Investigation

Simulation of Boreal Ecosystem Carbon and Water Budgets: Scaling from Local to Regional Extents

2.3 Contact Information

Contact 1

---------

Richard Fernandes

University of Toronto

Toronto, Ontario

(416) 978-5070

(416) 978-6729 (fax)

fernande@geog.utoronto.ca

Contact 2

------------

Xuewen Wang

University of Toronto

Toronto, Ontario

(416) 978-5070

(416) 978-6729 (fax)

wangx@geog.utoronto.ca

Contact 3

---------

David Knapp

Raytheon STX Corporation

NASA GSFC

Greenbelt, MD

(301) 286-1424

(301) 286-0239 (fax)

David.Knapp@gsfc.nasa.gov

3. Theory of Measurements

Throughfall is the amount of precipitation that falls through a tree canopy.  Throughfall catch buckets were placed at random locations and beside each turf lysimeter in each nested sampling plot.  In addition, four intensive throughfall plots were located with only throughfall catch buckets.  In all plots, the catch buckets were placed so that the lip of the bucket was just above the level of the live moss layer.  Throughfall was measured using the same protocol that was used for gross precipitation, with care taken to extract any litter in the buckets.

4. Equipment

4.1 Sensor/Instrument Description

Throughfall Catch Buckets

Clear plastic buckets with an 11 cm diameter orifice, 10 cm diameter base, and 10 cm depth were used.

4.1.1 Collection Environment

None given.

4.1.2 Source/Platform

Throughfall Gauges - Placed in the live moss surface layer flush with the live moss surface.

4.1.3 Source/Platform Mission Objectives

The objective was to measure daily changes in water storages or turf weights.

4.1.4 Key Variables

Throughfall Gauge - Throughfall (the amount of precipitation that falls through the forest canopy to the ground.)

4.1.5 Principles of Operation

The gauges were designed to hold an amount of water that fell through the canopy, that was weighed.  The weights were used to determine the water equivalent depth.

4.1.6 Sensor/Instrument Measurement Geometry

All throughfall gauges were repositioned using a bubble level to ensure that they were upright.

4.1.7 Manufacturer of Sensor/Instrument

Gauges - Darryl Carlysle Moses and Kira Dunham

(University of Toronto, Dept. of Geography)

Weigh Scales - (2) MARS MS3000W Series

4.2 Calibration

The weigh scales were calibrated to within the manufacturer's specifications immediately before the measurement campaign and at the University of Toronto after the campaign.  The effect of the weigh scales being off level were also tested, with no appreciable difference for tilt angles less than 20 degrees (which were defined by the first indent in the bubble level gauge used in the field).

4.2.1 Specifications

Weight Scales

Weight < 1 kg: accurate to +/- 0.1 g

Weight > 1 kg: accurate to +/- 1.0 g

4.2.1.1 Tolerance

None given.

4.2.2 Frequency of Calibration

The weigh scales were calibrated to within the manufacturer's specifications immediately before the measurement campaign and at the University of Toronto after the campaign.

4.2.3 Other Calibration Information

None.

5. Data Acquisition Methods

Each gauge and lysimeter was placed at a randomly selected location in each stratified plot or in clearings in the case of throughfall gauges.  The locations were not changed during the field campaign.  Measurements were made at each plot for all gauges and lysimeters before moving to another plot.  The measurements were made by weighing the amount of water in the gauge.  These weights were converted to water depths based on the orifice area of the gauge.

6. Observations

6.1 Data Notes

None given.

6.2 Field Notes

None given.

7. Data Description

7.1 Spatial Characteristics

All 1996 measurement plots were located within 500 m of the SSA-OBS flux tower  along a single transect leading radially outwards from the tower.  The goal was to place the plots on a perceived wetness gradient while keeping them within the flux tower footprint.  In addition, each plot was located so that it was separated from the others to characterize the typical spatial variability of surface hydrological processes.  Unfortunately, no plot was located in a region dominated by Sphagnum bogs, so turf lysimeters with sphagnum were located in isolated sphagnum patches within 10 m of each plot.

The 1994 throughfall data were collected near Joey Lake and near the NSA-OBS tower.

7.1.1 Spatial Coverage

In 1996, seven plots were located along a transect in the vicinity of the SSA-OBS flux tower.  The locations of the plot and of the actual measurements are given in the following figures.  Each plot had five live turf lysimeters and five throughfall gauges (one throughfall gauge near each live turf lysimeter).  In addition, two throughfall plots with over 20 throughfall gauges were located independently.

At the Joey Lake site in 1994, these data were collected at four different plots.

At the NSA-OBS site in 1994, the data were taken from two plots located near the flux tower. The plots were about 30 meters apart, about 10 m by 10 m square each. The samples within each plot were about 5 meters apart.

The approximate locations of the various plots are as follows:

BOREAS Grid

Site                 Longitude   Latitude      X          Y_____

NSA (Joey Lake)      98.15026W   55.46676N   807.025    571.969

NSA-OBS (Flux Twr.)  98.48139W   55.88007N   778.216    613.516

SSA-OBS (Flux Twr.) 105.11779W   53.98717N   385.012    348.646

7.1.2 Spatial Coverage Map

None.

7.1.3 Spatial Resolution

Each measurement site is located on the figures above.

Each throughfall gauge had an 11 cm diameter orifice, 10 cm diameter base, and 10 cm depth.  The throughfall gauges may catch precipitation from a region larger than their orifices.  The "fetch" of a throughfall or rain gauge depends on wind speed, precipitation intensity, and the cover over the gauge.

7.1.4 Projection

These plots are at point locations.  A map projection is not applicable.

7.1.5 Grid Description

Not applicable.

7.2 Temporal Characteristics

7.2.1 Temporal Coverage

The data at the SSA-OBS were collected from July to August 1996 with some small gaps.

The data at the NSA-OBS were collected from 23-Aug-1994 to 15-Sep-1994 with some gaps.

The data at Joey Lake (near NSA) were collected from 24-Jun-1994 to 06-Sep-1994 with some gaps.

7.2.2 Temporal Coverage Map

None.

7.2.3 Temporal Resolution

In 1996, data were collected daily and after each rain event, where possible. After the time was recorded, data collection proceeded for 1.5 to 2 hours, in order to visit all of the gauges.  Therefore, the actual data collection time is accurate to within 1.5 to 2.0 hours.  The amount of precipitation recorded is the amount that fell since the gauge was last checked.

The temporal resolution of the 1994 data is assumed to be similar to that of the 1996 data.

7.3 Data Characteristics

Data characteristics are defined in the companion data definition file (h8thrfld.def).

7.4 Sample Data Record

Sample data format shown in the companion data definition file (h8thrfld.def).

8. Data Organization

8.1 Data Granularity

All of the BOREAS HYD-08 Throughfall Data are contained in one dataset.

8.2 Data Format(s)

A data record consists of a series of numerical and character fields of American Standard Code for Information Interchange (ASCII) characters of varying length.  The fields are separated by commas with the character fields enclosed in single apostrophe marks.  Sample data records are shown in the companion data definition file (h8thrfld.def).

9. Data Manipulations

9.1 Formulae

See Section 9.1.1.

9.1.1 Derivation Techniques and Algorithms

The computation of water equivalent depth for throughfall gauges was performed using:

d (mm) = 1000 (mm/m) * mass_water(g) / ( 1000kg/m3 * area_gauge_bottom(m2) )

9.2 Data Processing Sequence

9.2.1 Processing Steps

1. Set up necessary equipment.

2. Performed daily weighings and emptied weighed gauges.

3. Performed the necessary data manipulations.

4. Added the necessary column headings.

5. Transferred the information to the BOREAS Information System (BORIS).

6. BORIS staff loaded the data into the relational data base.

9.2.2 Processing Changes

None given.

9.3 Calculations

9.3.1 Special Corrections/Adjustments

None.

9.3.2 Calculated Variables

See Section 9.1.1.

9.4 Graphs and Plots

None given.

10. Errors

10.1 Sources of Error

Quantifiable Errors

1. Location errors - The plots were located with reference to the flux tower using dead reckoning.  Errors on the order of +/-10 m can be expected for Figure 1 (location of plot origins) and +/- 0.1 m for Figure 2 (location of measurement sites within plots).

2. Dimensional measurements - Measurements of radii, length, and width dimensions were made using a metric hand ruler.  An error of +/-0.5 mm in precision is possible.  This will result in negligible errors in computed surface areas of catch gauges or turf trays.

3. Throughfall gauge weighing error - Tests were performed to detect the weight of water drops present on the sides of throughfall gauges.  These weights were not measurable.  The average weight of all throughfall gauges was used to compute the net weight of water in the gauge.  The error in using the average weight is less than +/- 0.1 g and can be considered negligible.

4.  Weigh scale errors - The weigh scale errors assuming no contamination of the weighing surface (e.g., water drops on it) are given by the manufacturer as a precision error of +/- 0.1 g for weights less than 1,000 g and +/- 1.0 g for weights greater than 1,000 g.

Unquantifiable Errors

Throughfall Gauges

- Splashing of drops outside the gauge.  This error suggests a negative accuracy error for throughfall gauges.  However, given that splash is larger for larger storms (the water level in the container is higher and the drop size and velocity is likely higher) the accuracy bias should be small relative to the

measured value.

- Lack of leveling because of wind or animal disturbance.  Gauges that were substantially tilted were recorded but flagged and have not been included in the submitted data set.

- Drip not completed at observation time.  It is possible that the drip process had not completed after the observation, especially for nighttime rains.  No attempt was made to reconcile this possible negative accuracy error; however,

substantial drip would be recorded in the subsequent measurement.

Gross Precipitation Gauges

- Errors caused by wind turbulence around the gauge, evaporation from the collector funnel or condensation on the funnel are possible.  It is likely that precipitation is underestimated because of evaporation from the funnel.

10.2 Quality Assessment

10.2.1 Data Validation by Source

These data are very preliminary.  General trends in the data are reliable; however, individual measurements may be completely in error.

10.2.2 Confidence Level/Accuracy Judgment

Mean values or plots and gross precipitation accuracy is estimated to be approximately 2 out of 5,  individual measurements at 1 out of 5.

10.2.3 Measurement Error for Parameters

Estimates of errors of each measurement variable are given below.

Time: +/- 2 hours

Throughfall:  The accuracy of the throughfall measurements is thought to be approximately 5%.  The precision of each measurement is directly related to the precision of the scale that weighed the gauge.

10.2.4 Additional Quality Assessments

Data quality assessment is ongoing by the investigator.

10.2.5 Data Verification by Data Center

The data were loaded into the relational data base and checked to ensure that the values were loaded correctly.

11. Notes

11.1 Limitations of the Data

Isolated data points may be in complete error because of improper recording or reformatting during documentation.  Revision of data is continuing.

11.2 Known Problems with the Data

None given.

11.3 Usage Guidance

Moss water fluxes are conservative; any strong jumps in time series should be flagged as potential measurement or recording errors unless explained by commensurate inputs.

11.4 Other Relevant Information

None.

12. Application of the Data Set

The HYD-08 data sets can be used for:

1. Quantifying rough canopy interception rates for given storm size at the SSA-OBS site.

2. Quantifying daily moisture fluxes in moss layers.

3. Possibly inferring relationships between stand parameters and measured fluxes.

4. Parameterizing flux models (especially hydrological models at stand to local scale).

13. Future Modifications and Plans

Data quality assessment is continuing by the investigators.

14. Software

14.1 Software Description

Not applicable.

14.2 Software Access

Not applicable.

15. Data Access

15.1 Contact Information

Ms. Beth Nelson

BOREAS Data Manager

NASA GSFC

Greenbelt, MD

(301) 286-4005

(301) 286-0239 (fax)

Elizabeth.Nelson@gsfc.nasa.gov

15.2 Data Center Identification

See Section 15.1.

15.3 Procedures for Obtaining Data

Users may place requests by telephone, fax, or electronic mail.

15.4 Data Center Status/Plans

The HYD-08 throughfall data are available from the Earth Observing System Data and Information System (EOSDIS) Oak Ridge National Laboratory (ORNL) Distributed Active Archive Center (DAAC).

The BOREAS contact at ORNL is:

ORNL DAAC User Services

Oak Ridge National Laboratory

(865) 241-3952

ornldaac@ornl.gov

ornl@eos.nasa.gov

16. Output Products and Availability

16.1 Tape Products

None.

16.2 Film Products

None.

16.3 Other Products

The ASCII data files.

17. References

None given.

17.1 Platform/Sensor/Instrument/Data Processing Documentation

None.

17.2 Journal Articles and Study Reports

Haddeland, I. and D.P. Lettenmaier. 1995. Hydrologic Modeling of Boreal

Forest Ecosystems. Water Resources Series Technical Report No. 143.

University of Washington, 123 pp.

Price, A.G., K. Dunham, T. Carleton, and L.E. Band. 1997. Variability of

water fluxes through the Black Spruce (Picea Mariana) canopy and Feather

Moss (Pleurozium Schreberi) carpet in the Boreal Forest of Northern Manitoba.

Journal of Hydrology, 196, 310-323.

Sellers, P. and F. Hall. 1994. Boreal Ecosystem-Atmosphere Study: Experiment Plan. Version 1994-3.0, NASA BOREAS Report (EXPLAN 94).

Sellers, P. and F. Hall. 1996. Boreal Ecosystem-Atmosphere Study: Experiment Plan. Version 1996-2.0, NASA BOREAS Report (EXPLAN 96).

Sellers, P., F. Hall, and K.F. Huemmrich. 1996. Boreal Ecosystem-Atmosphere Study: 1994 Operations. NASA BOREAS Report (OPS DOC 94).

Sellers, P., F. Hall, and K.F. Huemmrich. 1997. Boreal Ecosystem-Atmosphere Study: 1996 Operations. NASA BOREAS Report (OPS DOC 96).

Sellers, P., F. Hall, H. Margolis, B. Kelly, D. Baldocchi, G. den Hartog, J. Cihlar, M.G. Ryan, B. Goodison, P. Crill, K.J. Ranson, D. Lettenmaier, and D.E. Wickland. 1995. The boreal ecosystem-atmosphere study (BOREAS): an overview and early results from the 1994 field year. Bulletin of the American Meteorological Society, 76(9), 1549-1577.

Sellers, P.J., F.G. Hall, R.D. Kelly, A. Black, D. Baldocchi, J. Berry, M. Ryan, K.J. Ranson, P.M. Crill, D.P. Lettenmaier, H. Margolis, J. Cihlar,

J. Newcomer, D. Fitzjarrald, P.G. Jarvis, S.T. Gower, D. Halliwell, D. Williams, B. Goodison, D.E. Wickland, and F.E. Guertin. 1997. BOREAS in 1997:

Experiment Overview, Scientific Results and Future Directions. Journal of Geophysical Research, BOREAS Special Issue, 102(D24), Dec. 1997, pp.

28731-28770.

17.3 Archive/DBMS Usage Documentation

None given.

18. Glossary of Terms

None given.

19. List of Acronyms

ASCII   - American Standard Code for Information Interchange

BOREAS  - BOReal Ecosystem-Atmosphere Study

BORIS   - BOREAS Information System

CD-ROM  - Compact Disk (optical), Read-Only Memory

DAAC    - Distributed Active Archive Center

EOS     - Earth Observing System

EOSDIS  - EOS Data and Information System

FFC-T   - Focused Field Campaign - Thaw

GMT     - Greenwich Mean Time

GSFC    - Goddard Space Flight Center

HYD     - Hydrology

IFC     - Intensive Field Campaign

NASA    - National Aeronautics and Space Administration

NSA     - Northern Study Area

OBS     - Old Black Spruce

ORNL    - Oak Ridge National Laboratory

PANP    - Prince Albert National Park

SSA     - Southern Study Area

URL     - Uniform Resource Locator

20. Document Information

20.1 Document Revision Date

Written: 20-Nov-1996

Updated: 04-Aug-1998

20.2 Document Review Date(s)

BORIS Review: 24-Jul-1998

Science Review:

20.3 Document ID

20.4 Citation

This data product was collected and processed by the BOREAS Science Team HYD-08, led by Prof. Lawrence Band at the University of Toronto.  Please contact the principal investigator, Dr. Lawrence Band, before publishing results that are based on these data.

20.5 Document Curator

20.6 Document URL

KEYWORDS

PRECIPITATION

MOSS