BOREAS Level-3p Landsat TM Imagery: Geocoded and Scaled At-sensor Radiance

Summary:

For BOREAS, the level-3p Landsat TM data were used to supplement the level-3s Landsat TM products.  Along with the other remotely sensed images, the Landsat TM images were collected in order to provide spatially extensive information over the primary study areas.  This information includes radiant energy, detailed land cover, and biophysical parameter maps such as FPAR and LAI.  Although very similar to the level-3s Landsat TM products, the level-3p images were processed with ground control information which improved the accuracy of the geographic coordinates provided.  Geographically, the level-3p images cover the BOREAS NSA and SSA.  Temporally, the four images cover the period of 20-Aug-1988 to 07-Jun-1994.  Except for the 07-Jun-1994 image which contains 7 bands, the other three only contain 3 bands.

Table of Contents

*   1 Data Set Overview

*   2 Investigator(s)

*   3 Theory of Measurements

*   4 Equipment

*   5 Data Acquisition Methods

*   6 Observations

*   7 Data Description

*   8 Data Organization

*   9 Data Manipulations

*  10 Errors

*  11 Notes

*  12 Application of the Data Set

*  13 Future Modifications and Plans

*  14 Software

*  15 Data Access

*  16 Output Products and Availability

*  17 References

*  18 Glossary of Terms

*  19 List of Acronyms

*  20 Document Information

1. Data Set Overview

1.1 Data Set Identification

BOREAS Level-3p Landsat TM Imagery: Geocoded and Scaled At-sensor Radiance

1.2 Data Set Introduction

The BOReal Ecosystem-Atmosphere Study (BOREAS) Staff Science effort covered those activities that were BOREAS community-level activities, or required uniform data collection procedures across sites and time. These activities included the acquisition of the relevant satellite data. Data from the Landsat Thematic Mapper (TM) instruments on the Landsat satellites were acquired by the Canada Centre for Remote Sensing (CCRS) and provided for use by BOREAS researchers.

1.3 Objective/Purpose

For BOREAS, the Landsat TM imagery, along with the other remotely sensed images, was collected in order to provide spatially extensive information over the primary study areas.  This information includes detailed land cover and biophysical parameter maps such as biomass, Fraction of Photosynthetically Active Radiation (FPAR), and Leaf Area Index (LAI).

1.4 Summary of Parameters

Landsat TM level-3p data in the BORIS contains the following parameters:

Original image header information, image coordinates, gains and offsets for each band for at-sensor radiance derivations, image bands 3, 4, 5 or 1 - 7 processed in a precision geocorrected form.

1.5 Discussion

Use and distribution of the level-3p Landsat TM images are subject to copyright restrictions.  CCRS and Radarsat International (RSI) granted permission to BOREAS to place a subset of the level-3a Landsat TM images on the BOREAS CD-ROM series; however, none of the level-3p images are included.  The level-3p images may not be available for public access.  Please see Sections 15 and 16 for further details.

BORIS staff processed the Landsat TM level-3p imagery by:

1) Extracting pertinent header information from the level-3p image product and

placing it in an American Standard Code for Information Interchange (ASCII)

file on disk,

2) Reading the information in the ASCII disk file and loading the on-line data

base with pertinent information.

1.6 Related Data Sets

BOREAS Level-3a Landsat TM Imagery: Scaled At-sensor Radiance in BSQ Format

BOREAS Level-3b Landsat TM Imagery: At-sensor Radiance in BSQ Format

BOREAS Level-3s Landsat TM Imagery: Scaled At-sensor Radiance in LGSOWG Format

BOREAS Level-3s SPOT Imagery: Scaled At-sensor Radiance in LGSOWG Format

2. Investigator(s)

2.1 Investigator(s) Name and Title

BOREAS Staff Science

2.2 Title of Investigation

BOREAS Staff Science Satellite Data Acquisition Program

2.3 Contact Information

Contact 1

----------

Jeffrey A. Newcomer

NASA Goddard Space Flight Center (GSFC)

Greenbelt, MD

(301) 286-785

(301) 286-0239

Jeffrey.Newcomer@gsfc.nasa.gov

Contact 2

----------

Josef Cihlar

Canada Centre for Remote Sensing

Ottawa, Ontario

Canada

(613) 947-1265

Josef.Cihlar@geocan.emr.ca

3. Theory of Measurements

The Landsat series of satellites began with the Earth Resources Technology Satellite (ERTS) launched in July 1972. This satellite was renamed Landsat 1 in 1975 to reflect its primary use as a land resource observatory. Through its onboard instruments, Landsat monitors Earth's mountain ranges, deserts, forests, and crops by measuring the light waves they reflect.

The second generation of Landsat satellites (4 and 5) marked a significant advance in remote sensing through the addition of the more sophisticated TM sensor, with higher spectral and spatial resolution, and faster data processing at a highly automated data processing facility at the National Aeronautics and Space Administration (NASA) Goddard Space Flight Center (GSFC) in Greenbelt, MD.  For BOREAS, the CCRS receiving station in Prince Albert, Saskatchewan collected the raw data.  Processing of the raw data to the level-3p images was performed with the Geocoded Image Correction System (GICS; Friedel, 1992) at the CCRS facility in Ottawa.

As Landsat's instrument mirrors scan Earth's surface, light enters the instrument optics, where it is focused on specially calibrated detector arrays. Onboard electronics encode the detector voltage as binary digits or bits. These digital image data are then relayed back to Earth to be processed into film and Computer-Compatible Tape (CCT) products which are subsequently used for Earth resources analysis.

4. Equipment

4.1 Sensor/Instrument Description

The TM sensor system records radiation from seven bands in the electromagnetic spectrum. It has a telescope that directs the incoming radiant flux obtained along a scan line through a scan line collector to the visible and near-infrared focal plane, or to the mid-infrared and thermal-infrared cooled focal plane. The detectors for the visible and near-infrared bands (1 to 4) are four staggered linear arrays, each containing 16 silicon detectors. The two mid-infrared detectors are 16 indium-antimonide cells in a staggered linear array, and the thermal-infrared detector is a four-element array of mercury-cadmium-telluride cells. The spectral regions, band widths, and primary use of each channel are given in the following table:

Channel  Wavelength (µm)                Primary Use

-------  ---------------  ------------------------------------------

1        0.451 -  0.521      Coastal water mapping, soil vegetation

differentiation, deciduous/coniferous

differentiation.

2        0.526 -  0.615      Green reflectance by healthy vegetation.

3        0.622 -  0.699      Chlorophyll absorption for plant species

differentiation.

4        0.771 -  0.905      Biomass surveys, water body delineation.

5        1.564 -  1.790      Vegetation moisture measurement, snow and cloud

differentiation.

6       10.450 - 12.460      Plant heat stress measurement, other thermal

mapping.

7        2.083 -  2.351      Hydrothermal mapping.

4.1.1 Collection Environment

The BOREAS Landsat TM level-3p images were acquired through the CCRS.  Radiometric corrections and systematic geometric corrections are applied to produce the images in a path-oriented and precision-corrected (level-3p) form.  A full TM image contains 6920 pixels in each of 5728 lines.  Before any geometric corrections, the ground resolution is 30 m for bands 1, 2, 3, 4, 5, and 7 and 120 m for band 6 at nadir. The pixel values of the images can range from 0 to 255. This allows each pixel to be stored in a single-byte field.  The level-3p images were processed through the CCRS GICS.  The Landsat satellite orbits Earth at an altitude of 705 km.

4.1.2 Source/Platform

Although the majority of the BOREAS Landsat TM imagery was acquired by the instrument onboard Landsat-5, some imagery was obtained with the TM sensor on the Landsat 4 platform.

4.1.3 Source/Platform Mission Objectives

The Landsat TM is designed to respond to and measure both reflected and emitted Earth surface radiation to enable the investigation, survey, inventory, and mapping of Earth's natural resources.

4.1.4 Key Variables

Reflected radiation, emitted radiation, temperature.

4.1.5 Principles of Operation

The TM is a scanning optical sensor operating in the visible and infrared wavelengths. It contains a scan mirror assembly that directly projects the reflected Earth radiation onto detectors arrayed in two focal planes. The TM achieves better image resolution, sharper color separation, and greater in-flight geometric and radiometric accuracy for seven spectral bands simultaneously than the previous generation sensor, the MultiSpectral Scanner (MSS). Data collected by the sensor are beamed back to ground receiving stations for processing.

4.1.6 Sensor/Instrument Measurement Geometry

The TM sensor depends on the forward motion of the spacecraft for the along-track scan and uses moving mirror assembly to scan in the cross-track direction (perpendicular to the spacecraft).  The instantaneous field-of-view (IFOV) for each detector from bands 1 - 5 and band 7 is equivalent to a 30-m square when projected to the ground at nadir; band 6 (the thermal infrared band) has an IFOV equivalent to a 120-m square at nadir.

4.1.7 Manufacturer of Sensor/Instrument

NASA GSFC

Greenbelt, MD 20771

Hughes Santa Barbara Remote Sensing (SBRS)

Goleta, CA

4.2 Calibration

The internal calibrator, a flex-pivot-mounted shutter assembly, is synchronized with the scan mirror, oscillating at the same 7-Hz frequency. During the turnaround period of the scan mirror, the shutter introduces the calibration source energy and a black direct-current restoration surface into the 100-detector field-of-view (FOV).

The calibration signals for bands 1 - 5 and 7 are derived from three regulated tungsten-filament lamps. The calibration source for band 6 is a blackbody with three temperature selections, commanded from the ground. The method for transmitting radiation to the moving calibration shutter allows the tungsten lamps to provide radiation independently and to contribute proportionately to the illumination of all detectors.

4.2.1 Specifications

Radiometric

Band              Sensitivity [NE(dP)]*

----              --------------------

1                     0.8%

2                     0.5%

3                     0.5%

4                     0.5%

5                     1.0%

6                     0.5 K [NE(dT)]

7                     2.4%

Ground IFOV             30 m (Bands 1-5, 7)

120 m (Band 6)

Avg. altitude          699.6 Km

Data rate                 85 Mbps

Quantization levels      256

Orbit angle             8.15 degrees

Orbital nodal period   98.88 minutes

Scan width               185 km

Scan angle              14.9 degrees

Image overlap            7.6%

Note:  The radiometric sensitivities are the noise-equivalent (NE) reflectance differences for the reflective channels expressed as percentages [NE(dP)] and temperature differences for the thermal-infrared bands [NE(dT)] in Kelvins.

4.2.1.1 Tolerance

The TM channels were designed for a NE differential represented by the radiometric sensitivity shown in Section 4.2.1.

4.2.2 Frequency of Calibration

The absolute radiometric calibration between bands on the TM sensor is maintained by using internal calibrators located between the telescope and the detectors that are sampled at the end of a scan.

4.2.3 Other Calibration Information

Relative within-band radiometric calibration, to reduce "striping," is provided by a scene-based procedure called histogram equalization. Because of the absolute accuracy and relative precision of this calibration scheme, it is assumed that any changes in the optics of the primary telescope or the "effective radiance" from the internal calibrator lamps are insignificant in comparison to the changes in detector sensitivity and electronic gain and bias with time and that the scene-dependent sampling is sufficiently precise for the required within-scan destriping from histogram equalization.

Each TM reflective band and the internal calibrator lamps were calibrated prior to launch using lamps in integrating spheres that were in turn calibrated against lamps traceable to calibrated National Bureau of Standards lamps. The absolute radiometric calibration constants in the "short-term" and "long-term" parameter files used for ground processing were modified after launch if there was an inconsistency within or between bands, a change in the inherent dynamic range of the sensors, or a desire to make quantized and calibrated values from one sensor match those from another.

5. Data Acquisition Methods

The BOREAS Landsat TM level-3s and -3p images were acquired through the CCRS.  Radiometric corrections and systematic or precision geometric corrections are applied to produce the images in a path-oriented form.  A full TM image contains 6920 pixels in each of 5728 lines (see Section 11.2).  Before any geometric corrections, the ground resolution is 30 m for bands 1-5 and 7 and 120 m for band 6 at nadir. The pixel values of the images can range from 0 to 255. This allows each pixel to be stored in a single-byte field.

6. Observations

6.1 Data Notes

None.

6.2 Field Notes

Not applicable.

7. Data Description

7.1 Spatial Characteristics

7.1.1 Spatial Coverage

The BOREAS level-3p Landsat TM images cover the NSA and SSA.  The SSA and the NSA are located in the southwest and northeast portions of the overall region.  A full TM scene covers approximately 31,000 square kilometers.

The North American Datum of 1983 (NAD83) corner coordinates of the SSA are:

Latitude     Longitude

--------     ---------

Northwest    54.321 N     106.228 W

Northeast    54.225 N     104.237 W

Southwest    53.515 N     106.321 W

Southeast    53.420 N     104.368 W

The NAD83 corner coordinates of the NSA are:

Latitude     Longitude

--------     ---------

Northwest    56.249 N      98.825 W

Northeast    56.083 N      97.234 W

Southwest    55.542 N      99.045 W

Southeast    55.379 N      97.489 W

7.1.2 Spatial Coverage Map

Not available.

7.1.3 Spatial Resolution

Before any geometric corrections, the spatial resolution is 30 m for bands 1-5,  and 7 and 120 m for band 6 at nadir.  These values increase with scan angle away from the nadir path.  The level-3p Landsat TM images have had geometric corrections applied so that the spatial resolution for all pixels is 30 m in all bands.  These level-3p images have a high level of internal spatial integrity and have had ground control added to improve the accuracy of the geographic coordinates provided on the tape.  The accuracy of these geographic coordinates is unknown at the time of writing this document.

7.1.4 Projection

The level-3p Landsat TM images are placed in a Universal Transverse Mercator (UTM) projection based on NAD83. Detailed projection parameter information for the individual images is contained in the leader file(s).

7.1.5 Grid Description

The pixel/grid spacing for each pixel in the level-3p Landsat TM images is 30 m in the UTM projection.  Detailed grid parameter information for the individual images is contained in the leader file(s).

7.2 Temporal Characteristics

7.2.1 Temporal Coverage

Imagery acquired before the BOREAS field campaigns were conducted is included in the BOREAS archive with imagery collected during the project.  Historical Landsat data have been acquired by CCRS routinely since the launch of Landsat 1 and are kept in the CCRS archive.

Since the mid-1980s, CCRS has been acquiring and archiving all Landsat data over Canada during the growing season; however during the winter, only requested data were obtained. For BOREAS, this policy was modified to obtain data throughout the year over the BOREAS region. The acquired data are archived by CCRS and can be interrogated to ascertain which scenes were archived and their characteristics.

The BOREAS level-3p Landsat TM acquisitions cover 20-Aug-1988 to 07-Jun-1994.

three-band scenes were ordered specifically to compare precision coordinate information in the header with coordinates derived from the enhanced geographic

There are a limited number of level-3p TM images in BORIS.  A few level-3p information BORIS is providing with the level-3a TM product.  One seven-band level-3p TM scene is archived in BORIS, only because the level-3s was not available.

7.2.2 Temporal Coverage Map

The following table lists the available level-3p Landsat TM images:

Date        Study Area

-----------     ----------

20-Aug-1988        NSA

14-Aug-1989        NSA

09-Aug-1991        SSA

07-Jul-1994        SSA

7.2.3 Temporal Resolution

The Landsat TM satellite revisit frequency is 16 days for each path/row, however in the BOREAS region the overlap between scene paths is about 50%.

7.3 Data Characteristics

Data characteristics are defined in the companion data definition file

(ltm_ii3p.def)

7.3.1 Parameter/Variable

The main parameter contained in the image data files is scaled at-sensor radiance.

7.3.2 Variable Description/Definition

For the image data files:

Scaled at-sensor radiance - The scaled value representing the radiant energy

incident on the sensor aperture at the time of data

collection in the specific TM wavelength regions.

7.3.3 Unit of Measurement

The units for the scaled at-sensor radiance values vary by band.  To obtain at-sensor radiance values in Watts/(m2 * sr * µm) use the formula:

At-sensor Radiance = Scaled Value * Gain + Offset

where the gain and Offset values are contained in the ASCII header file.

7.3.4 Data Source

The data contained in the level-3p Landsat TM data files come from various portions of the Landsat satellite, the TM instrument, and the ground processing components.  The level-3p Landsat TM images were supplied to the BOREAS by the CCRS.

7.3.5 Data Range

The maximum range of scaled at-sensor radiance values in each level-3p Landsat TM image band is limited from 0 (zero) to 255 so that the values can be stored in a single 8-bit (byte) field.

7.4 Sample Data Record

Sample data format shown in the companion data definition file (ltm_ii3p.def).

8. Data Organization

8.1 Data Granularity

The smallest unit of data for level-3p Landsat TM imagery is a full TM scene.  The individual level-3p Landsat TM images from CCRS are stored in either a band sequential (BSQ) or band interleaved by line (BIL) form.  General information on these two formats is provided in Section 8.2.  Detailed information on these formats can be obtained from the CCRS document referenced in Section 17.1.  Most of the BOREAS level-3p Landsat TM image products contain only three of the seven bands.  BORIS used these three-band images to evaluate the processing of the level-3s images to level-3a products.  As such, the number and size of the BIL and BSQ files for the three-band images are less than the full seven-band image format described below.

8.2 Data Format(s)

The data files consists numerical and character fields of varying length separated by commas. The character fields are enclosed with a single apostrophe marks. There are no spaces between the fields. Sample data format shown in the companion data definition file (ltm_ii3p.def).

8.2.1 Band Sequential Format

The files associated with a BSQ TM scene are as follows:

File 1   volume directory

File 2   leader file band 1

File 3   TM band 1

File 4   trailer file band 1

File 5   leader file band 2

File 6   TM band 2

File 7   trailer file band 2

File 8   leader file band 2

File 9   TM band 2

File 10  trailer file band 2

.         .

and so on...

.         .

File 21  TM band 7

File 22  trailer file band 7

File 23  null volume file

If there are multiple scenes on a tape, the next scene would occupy files 24-46, 23 files exactly as above.  Up to four TM scenes (92 files) are contained on one 8 mm tape.  Multiple volume directory files are contained on one tape media because the 8 mm tapes were generated from copying the original 9-track tapes, and each one of those had its own volume directory.  Each image file in BSQ format contains image data for one spectral band.

A three-band level-3p TM scene would contain a total of 11 files as described above; however the bands included in the three-band subset are TM bands 3, 4, and 5.

8.2.1.1 BSQ Leader Files

The contents of leader files which have been defined in detail by the Landsat Ground Station Operations Working Group (LGSOWG) Technical Working Group (LTWG) are as follows:

File descriptor record;

Scene header record;

Map projection (scene-related) ancillary record;

Radiometric transformation ancillary record.

All leader files contain fixed-length records of 4320 bytes and contain both ASCII and binary data.  For specific details, see the CCRS documentation referenced in Section 17.1.

8.2.1.2 BSQ Imagery File

The BSQ image files have 5729 records; each record contains 7020 bytes.  The first record in this file is a header record, followed by 5728 image records.

The contents of the scene header record are specified by LTWG standards and include information relating to the mission, sensor parameters, processing options, and geometric parameters for the sensor.

Each image record contains 32 bytes of prefix data, 6920 bytes of image data, and 68 bytes of suffix data (32 + 6920 + 68 = 7020).  Each image is oriented so that pixel 1, line 1 is in the upper left-hand corner (i.e., northwest) of the screen display.  Pixels and lines progress left to right and top to bottom so that pixel n, line n is in the lower right-hand corner.

8.2.1.3 BSQ Trailer File

The trailer file contains information associated with the image data that was not always available before writing the image data, such as data and recording quality and data summaries.  Each trailer file contains a file descriptor record and trailer records for all bands of imagery in the associated imagery file.  All trailer files contain fixed-length records of 4320 bytes and contain both ASCII and binary data.  For specific details see the CCRS documentation referenced in Section 17.1.

8.2.2 Band Interleaved by Line Format

At this time, no level-3p Landsat TM data were supplied in BIL format.

9. Data Manipulations

9.1 Formulae

9.1.1 Derivation Techniques and Algorithms

Not applicable.

9.2 Data Processing Sequence

9.2.1 Processing Steps

BORIS staff processes a level-3p Landsat TM image by :

1) Extracting pertinent header information from the level-3p image product and

writing it to a disk file,

2) Reading the information in the disk file and loading the on-line data base

with needed information.

Some cloud cover and image quality assessment information is generated when BORIS staff processed the level-3p images to level-3a products.  This information is entered into the BORIS data base but is not included with the images on tape.  To obtain this information, see the inventory file included on the CD-ROMs or see Section 15.1.

9.2.2 Processing Changes

None.

9.3 Calculations

9.3.1 Special Corrections/Adjustments

None.

9.3.2 Calculated Variables

None.

9.4 Graphs and Plots

None.

10. Errors

10.1 Sources of Error

Errors could arise in the acquired imagery from location inaccuracy, distortion of lengths, anisomorphism, the instrument's local coherence, and multispectral registrability. Other errors could arise from inherent radiometric imperfections of the sensors.

10.2 Quality Assessment

10.2.1 Data Validation by Source

Whatever the processing level, the geometric quality of the image depends on the accuracy of the viewing geometry.

Spectral errors could arise from image-wide signal-to-noise ratio, saturation, cross-talk, spikes, and response normalization caused by change in gain.

10.2.2 Confidence Level/Accuracy Judgment

Assessment of accuracy of the absolute radiometric constants is difficult. The uncertainties in prelaunch and postlaunch updates of the absolute calibration constants are nominally specified to be less than 10%. A root mean square (rms) summing of known errors in the prelaunch calibration suggests that this may be a reasonable estimate of overall uncertainty in the prelaunch calibration.

There are also known, but as yet uncorrected, effects associated with temperature-dependence of the TM internal calibrator that may be contributing to apparent discontinuous changes at launch and to the continuous changes of gain while in orbit. Additional uncertainties for exoatmospheric reflectances are probably less than 2% in the visible/near-infrared and less than 5% in the shortwave infrared portion of the spectrum as judged by the current differences in estimates of the solar irradiance.

The level-3p Landsat TM images have had geometric corrections applied so that the spatial resolution for all pixels is 30 m in all bands.  These level-3p images have a high level of internal spatial integrity and have had ground control applied to give a high level of accuracy to the supplied geographic coordinates.

10.2.3 Measurement Error for Parameters

None given.

10.2.4 Additional Quality Assessments

The reproducibility of ground measurements at White Sands, NM, at times of Landsat TM overpass to about 5% for 5 dates for Bands 1-4 suggests a potential for monitoring sensor change for the whole system with time.

Images are screened for level of cloud cover before BORIS processing.

10.2.5 Data Verification by Data Center

BORIS used developed software to extract information for logging the data into a relational data base.  In addition, the software read through the records of the files checking for proper record sizes.

11. Notes

11.1 Limitations of the Data

None.

11.2 Known Problems with the Data

To date, the following discrepancies/problems have been noted in the data:

o Some header files refer to Level-1 rather than Level-3 or to L1P rather than

L3P since they were created by software prior to BORIS finalization of data

categories.

11.3 Usage Guidance

None.

11.4 Other Relevant Information

o None

12. Application of the Data Set

The level-3a Landsat TM images are useful for anyone interested in high spatial resolution imagery over the entire NSA or SSA.

13. Future Modifications and Plans

None.

14. Software

14.1 Software Description

BORIS developed software and command procedures to :

1) Extract header information from level-3p Landsat TM images on tape and

writing it to ASCII files on disk,

2) Read the ASCII disk file and logging the level-3 Landsat TM image products

into the Oracle data base tables.

3) Convert between the geographic systems of (latitude, longitude), UTM

(northing, easting), and BOREAS (x,y) grid locations.

The software mentioned under items 1 to 5 is written in C and is operational on VAX 6410 and MicroVAX 3100 systems at GSFC.  The primary dependencies in the software are the tape input/output (I/O) library and the Oracle data base utility routines.

The geographic coordinate conversion utility (BOR_CORD) has been tested and used on Macintosh, IBM PC, VAX, Silicon Graphics, and Sun workstations.

14.2 Software Access

All of the described software is available upon request.  BORIS staff would appreciate knowing of any problems discovered with the software, but cannot promise to fix them.

15. Data Access

15.1 Contact Information

Ms. Beth Nelson

NASA GSFC

Greenbelt, MD

(301) 286-4005

(301) 286-0239 (fax)

beth@ltpmail.gsfc.nasa.gov

15.2 Data Center Identification

See 15.1

15.3 Procedures for Obtaining Data

Users may place requests by telephone, electronic mail, or FAX.

15.4 Data Center Status/Plans

The level-3p Landsat TM image data are available from the Earth Observing System Data and Information System (EOS-DIS) Oak Ridge National Laboratory (ORNL) Distributed Active Archive Center (DAAC).  The BOREAS contact at ORNL is:

ORNL DAAC User Services

Oak Ridge National Laboratory

(865) 241-3952

ornldaac@ornl.gov

ornl@eos.nasa.gov

Although the BOREAS level-3p Landsat TM images are being held in a public archive, copyright restrictions limit the distribution and use of the data.  The BOREAS CD-ROM series is publicly available and contain some of the level-3a Landsat TM images.  However, other Landsat TM image products in the collection are only available to official BOREAS project personnel.  Please contact the ORNL DAAC User Services office to get the most recent information.

16. Output Products and Availability

16.1 Tape Products

The level-3a Landsat TM data can be made available on 8-mm, Digital Archive Tape (DAT), or 9-track tapes at 6250 or 1600 Bytes per Inch (BPI).

Although the BOREAS level-3p Landsat TM images are being held in a public archive, copyright restrictions limit the distribution and use of the data.  The BOREAS CD-ROM series is publicly available and contain some of the level-3a Landsat TM images.  However, other Landsat TM image products in the collection are only available to official BOREAS project personnel.  Please contact the ORNL DAAC User Services office (see Section 15.4) to get the most recent information.

16.2 Film Products

None.

16.3 Other Products

Although the image inventory is contained on the BOREAS CD-ROM set, the actual level-3p Landsat TM images are not.  See section 15 for information about how to obtain the data.

17. References

17.1 Platform/Sensor/Instrument/Data Processing Documentation

Multispectral Scanner System for ERTS. 1972. HS324-5214. Hughes Aircraft Corporation. Santa Barbara, CA.

Standard Landsat 4, 5 and 6 TM CCT Format Specification, DMD-TM #82-249E. 1991. Canada Centre for Remote Sensing (CCRS), Surveys, Mapping and Remote Sensing Sector, Energy, Mines and Resources, Canada.

User's Guide for Landsat Thematic Mapper Computer-Compatible Tapes. 1985. Earth Observation Satellite Company. Lanham, MD.

17.2 Journal Articles and Study Reports

Byrne, G.F., P.F. Crapper, and K.K. Mayo. 1980. Monitoring land-cover change by principal component analysis of multitemporal Landsat data. Remote Sens. Environ. 10:175-184.

Chavez, P.C., S.C. Guptill, and J.A. Bowell. 1984. Image processing techniques for Thematic Mapper data. Technical Papers. 50th Annual Meeting of the Amer. Soc. of Photogr. 2:728-743.

Crist, E.P. and R.C. Cicone. 1984. Application of the Tasseled Cap concept to simulated Thematic Mapper data. Photogr. Engr. & Rem. Sens. 50:343-352.

Engel, J.L. and O. Weinstein. 1983. The Thematic Mapper: An Overview. IEEE Transactions on Geoscience and Remote Sensing. GE-21:258-265.

Friedel, J. 1992. System description of the Geocoded Image Correction System. Report GC-MA-50-3915, MacDonald Detwiller and Associates, Richmond, B.C.

Holmes, R.A. 1984. Advanced sensor systems: Thematic Mapper and beyond. Remote Sens. Environ. 15:213-221.

Kanemasu, E.T., J.L. Heilman, J.O. Bagley, and W.L. Powers. 1977. Using Landsat data to estimate evapotranspiration of winter wheat. Environmental Management. 1:515-520.

Lulla, K. 1983. The Landsat satellites and selected aspects of Physical Geography. Progress in Phy. Geogr. 7:1-45.

Malila, W.A. 1985. Comparison of the Information Contents of Landsat TM and MSS Data. Photogrammetric Engineering and Remote Sensing. 51:1449-1457.

Pollock, R.B. and E.T. Kanemasu. 1979. Estimating leaf-area index of wheat with Landsat data. Remote Sens. Environ. 8:307-312.

Robinov, C.J. 1982. Computation with physical values from Landsat digital data. Photogr. Engr. & Rem. Sens. 48:781-784.

Salomonson, V.V. 1984. Landsat 4 and 5 status and results from Thematic Mapper data analysis. Proceedings. Machine Processing of Remotely Sensed Data Symposium. Lab. for the Applications of Remote Sensing. West Lafayette, IN. p 13-18.

Satterwhite, M.B. 1984. Discriminating vegetation and soils using Landsat MSS and Thematic Mapper bands and band ratios. Technical Papers. 50th Annual Meeting of the Amer. Soc. of Photogr. 2:479-485.

Sellers, P. F. Hall. 1994. Boreal Ecosystem-Atmosphere Study: Experiment Plan. Version 1994-3.0, NASA BOREAS Report (EXPLAN 94).

Sellers, P. F. Hall. 1996. Boreal Ecosystem-Atmosphere Study: Experiment Plan. Version 1996-2.0, NASA BOREAS Report (EXPLAN 96).

Sellers, P. J., F. G. Hall, R. D. Kelly, A. Black, D. Baldocchi, J. Berry, M. Ryan, K. J. Ranson, P. M. Crill, D. P.  Lettenmaier, H. Margolis, J. Cihlar, J. Newcomer, D. Fitzjarrald, P. G. Jarvis, S. T. Gower, D. Halliwell, D. Williams, B. Goodison, D. E. Wickland, and F. E. Guertin. 1997. BOREAS in 1997: Experiment Overview, Scientific Results and Future Directions. Journal of Geophysical Research 102 (D24): 28, 731-28,770.

Sellers, P., F. Hall, and K.F. Huemmrich. 1996. Boreal Ecosystem-Atmosphere Study: 1994 Operations. NASA BOREAS Report (OPS DOC 94).

Sellers, P., F. Hall, and K.F. Huemmrich. 1997. Boreal Ecosystem-Atmosphere Study: 1996 Operations. NASA BOREAS Report (OPS DOC 96).

Sellers, P., F. Hall, H. Margolis, B. Kelly, D. Baldocchi, G. den Hartog, J. Cihlar, M.G. Ryan, B. Goodison, P. Crill, K.J. Ranson, D. Lettenmaier, and D.E. Wickland. 1995. The boreal ecosystem-atmosphere study (BOREAS): an overview and early results from the 1994 field year. Bulletin of the American Meteorological Society. 76(9):1549-1577.

Singn, A.N. and R.S. Dwived, 1986. The Utility of Landsat Imagery as an Integral Part of the Data Base for Small Scale Soil Mapping. Int. J. Remote Sensing. 7:1099-1108.

Taranik, J.V. 1978. Characteristics of the Landsat Multispectral Data System. U.S. Dept. of the Interior. Open File Report File Report 78-187. Sioux Falls, SD.

Thompson, D.R., and O.A. Wehmanen. 1980. Using Landsat digital data to detect moisture stress in corn-soybean growing region. Photogr. Engr. & Rem. Sens. 46:1082-1089.

Williams, D.L., J.R. Irons, B.L. Markham, R.F. Nelson, D.L. Toll, R.S. Latty, and M.L. Stauffer. 1984. A statistical evaluation of the advantages of Landsat Thematic Mapper data in comparison to Multi-spectral Scanner data. IEEE Transactions on Geoscience and Remote Sensing. GE-22.

17.3 Archive/DBMS Usage Documentation

None.

18. Glossary of Terms

None.

19. List of Acronyms

ASCII   - American Standard Code for Information Interchange

BIL     - Band Interleaved By Line

BOREAS  - BOReal Ecosystem-Atmosphere Study

BORIS   - BOREAS Information System

BPI     - Bytes per inch

BSQ     - Band sequential

CCRS    - Canada Centre for Remote Sensing

CCT     - Computer Compatible Tape

CD-ROM  - Compact Disk-Read-Only Memory

DAAC    - Distributed Active Archive Center

DAT     - Digital Archive Tape

DN      - Digital Number

EDC     - EROS Data Center

EOS     - Earth Observing System

EOSDIS  - EOS Data and Information System

EROS    - Earth Resources Observation System

ERTS    - Earth Resources Technology Satellite

FOLD    - Federally Owned Landsat Database

FOV     - Field of View

FPAR    - Fraction of Photosynthetically Active Radiation

GICS    - Geocoded Image Correction System

GSFC    - Goddard Space Flight Center

IFOV    - Instantaneous Field-of-View

I/O     - Input/Output

LAI     - Leaf Area Index

LGSOWG  - Landsat Ground Station Operations Working Group

LTWG    - LGSOWG Technical Working Group

MSS     - Multispectral Scanner

NAD27   - North American Datum of 1927

NAD83   - North American Datum of 1983

NASA    - National Aeronautics and Space Administration

NE      - Noise Equivalent

NSA     - Northern Study Area

ORNL    - Oak Ridge National Laboratory

PANP    - Prince Albert National Park

rms     - root-mean-square

SBRC    - Santa Barbara Research Center

SSA     - Southern Study Area

TIPS    - Thematic Mapper Image Processing System

TM      - Thematic Mapper

URL     - Uniform Resource Locator

UTM     - Universal Transverse Mercator

20. Document Information

20.1 Document Revision Dates

Written:         12-Apr-1995

Last Updated:    19-Mar-1998

20.2 Document Review Dates

BORIS Review:   20-Feb-1998

Science Review: 27-Feb-1998

20.3 Document ID

20.4 Citation

The Landsat TM level-3p images were acquired by CCRS and processed by RADARSAT International under an agreement with CCRS.

20.5 Document Curator

20.6 Document URL

Keywords

LANDSAT

LANDSAT THEMATIC MAPPER

EMITTED RADIATION

REFLECTED RADIATION