BOREAS TE-18 Landsat TM Physical Classification Image of the NSA

Summary

The objective of this classification is to provide the BOREAS investigators with

a data product that characterizes the land cover of the NSA.  A Landsat-5 TM image from 21-Jun-1995 was used to derive the classification.  A technique was implemented that uses reflectances of various land cover types along with a geometric optical canopy model to produce spectral trajectories.  These trajectories are used as training data to classify the image into the different land cover classes.  The data are provided in a binary, image file format.

Table of Contents

*  1 Data Set Overview

*  2 Investigator(s)

*  3 Theory of Measurements

*  4 Equipment

*  5 Data Acquisition Methods

*  6 Observations

*  7 Data Description

*  8 Data Organization

*  9 Data Manipulations

*  10 Errors

*  11 Notes

*  12 Application of the Data Set

*  13 Future Modifications and Plans

*  14 Software

*  15 Data Access

*  16 Output Products and Availability

*  17 References

*  18 Glossary of Terms

*  19 List of Acronyms

*  20 Document Information

1.  Data Set Overview

1.1  Data Set Identification

BOREAS TE-18 Landsat TM Physical Classification Image of the NSA

1.2  Data Set Introduction

This data set classifies the BOReal Ecosystem-Atmosphere Study (BOREAS) Northern Study Area (NSA) into 13 land cover classes.  These classes include wet conifer, dry conifer, deciduous, mixed (deciduous and conifer), fen, and various regeneration and other classes. The pixel resolution of this data set is 30 meters and the data set is georeferenced in the Albers Equal Area Conic (AEAC) projection.

1.3   Objective/Purpose

The objective of this data set is to provide BOREAS investigators with a land cover product for use in modeling activities.  The technique that was used to produced this data set can also be used to determine the amount of canopy cover within the given class and makes it possible to derive other biophysical parameters from the imagery.

1.4   Summary of Parameters and Variables

In a joint meeting of the BOREAS Terrestrial Ecosystem (TE) modelers and

the Remote Sensing Science (RSS) algorithm developers in Columbia,

MD, July 1992, several land cover classes were identified as

necessary inputs to the TE models.  One exception to this is the fire-

blackened class which is a consequence of spectral distinctness.  The

classification was performed using bands 3, 4, and 5 of the Landsat-5 Thematic Mapper (TM) scene.  The radiometric status of this scene was acceptable.

The parameter that is being described in this data set is the land cover class

for each 30 meter pixel.  The classes that are used in this data set are:

Image Value      Class

------------------------------

1            Conifer (Wet)

2            Conifer (Dry)

3            Mixed (Coniferous and Deciduous)

4            Deciduous

5            Fen

6            Water

7            Disturbed

8            Fire Blackened

9            New Regeneration Conifer

10           Medium-Age Regeneration Conifer

11           New Regeneration Deciduous

12           Medium-Age Regeneration-Deciduous

13           Grass

1.5  Discussion

The objective of this classification is to provide the BOREAS investigators with

a data product that characterizes the land cover of the NSA.  This data set can be used for modeling purposes.  The technique that was used to produce this classification is based on the work of Dr. Forrest Hall.  This technique involves the use of reflectances of various land cover types along with a geometric optical canopy model to model the amount of shadow.  The reflectance data and the model are used to produce spectral trajectories of the various land cover classes.  The trajectories are used in a way that is similar to training data. Each image pixel is compared to the various points of each trajectory.  The pixel is assigned to the class of the point to which it is closest in red/near-infrared reflectance space.

1.6  Related Data Sets

BOREAS Forest Cover Data Layers of the NSA-MSA in Raster Format

BOREAS TE-18 Landsat TM Physical Classification Image of the SSA

2.    Investigator(s)

2.1   Investigator(s) Name and Title

Dr. Forrest G. Hall

NASA Goddard Space Flight Center (GSFC)

2.2   Title of Investigation

TE-18 Regional Scale Carbon Flux from Modeling and Remote Sensing

2.3   Contacts

Contact 1

----------------

Dr. Forrest G. Hall

NASA Goddard Space Flight Center

Greenbelt, MD

Tel.: (301) 286-2974

FAX: (301) 286-0239

Forrest.G.Hall@gsfc.nasa.gov

Contact 2

----------------

David Knapp

NASA Goddard Space Flight Center

Greenbelt, MD

Tel: (301) 286-1424

FAX: (301) 286-0239

David.Knapp@gsfc.nasa.gov

3.  Theory of Measurements

The Landsat-5 TM sensor collects imagery of Earth in seven spectral bands ranging from the blue to the thermal infrared portion of the electromagnetic spectrum.

This image was classified from Landsat-5 TM imagery using a technique described by Dr. Forrest Hall (Hall, et al., in press).  In this technique, end member

reflectances of canopy, background, and shadow are used with a geometric canopy

model to compute simulated pixel reflectances for increasing amounts of canopy

cover.  These simulated reflectances can be plotted as a continuous trajectory

for each class (e.g., wet conifer, deciduous, etc.) from 0% to 100% canopy cover.

The imagery pixels were classified based on their proximity to the trajectories,

with the pixel being assigned to the class of the closest trajectory.

4.  Equipment

4.1  Instrument Description.

The Landsat-5 TM sensor system records radiation from the seven bands

described in Section 4.2.1. It has a telescope that directs the incoming radiant flux obtained along a scan line through a scan line collector to the visible and near-infrared focal plane, or to the mid-infrared and thermal-infrared cooled focal plane.  The detectors for the visible and near-infrared bands (1 - 4) are four staggered linear arrays, each containing 16 silicon detectors. The two mid-infrared detectors are 16 indium-antimonide cells in a staggered linear array, and the thermal-infrared detector is a four-element array of mercury-cadmium-telluride cells.

4.1.1  Collection Environment

The data that were used to produce this classification were collected by the

Landsat-5 Thematic Mapper on 21-Jun-1995.  Landsat-5 orbits Earth at an

altitude of approximately 705 kilometers.

4.1.2  Source/Platform

Landsat-5 satellite

4.1.3  Source/Platform Mission Objectives

The mission of the Landsat-5 satellite is to measure reflected radiation from

Earth’s surface at a spatial resolution of 30 meters and to measure the

temperature of Earth’s surface at a spatial resolution of 120 meters.

4.1.4  Key Variables

Reflected radiation.

Emitted radiation.

Temperature.

4.1.5  Principles of Operation

The TM is a scanning optical sensor operating in the visible and infrared wavelengths. It contains a scan mirror assembly that directly projects the reflected Earth radiation onto detectors arrayed in two focal planes. The TM achieves better imagery resolution, sharper color separation, and greater inflight geometric and radiometric accuracy for seven spectral bands simultaneously than the previous Multispectral Scanner (MSS). Data collected by the sensor are transmitted to Earth-receiving stations for processing.

4.1.6  Sensor/Instrument Measurement Geometry

The TM depends on the forward motion of the spacecraft for the along-track scan and uses moving mirror assembly to scan in the cross-track direction (perpendicular to the spacecraft).  The Instantaneous Field of View (IFOV) for each detector from bands 1-5 and band 7 is equivalent to a 30-m square when projected to the ground; band 6 (the thermal-infrared band) has an IFOV equivalent to a 120-meter square.

4.1.7  Manufacturer of Sensor/Instrument

NASA Goddard Space Flight Center

Greenbelt, MD 20771

Hughes Aircraft Corporation

Santa Barbara, CA.

4.2  Calibration.

The internal calibrator, a flex-pivot-mounted shutter assembly, is

synchronized with the scan mirror, oscillating at the same 7-Hz

frequency. During the turnaround period of the scan mirror, the

shutter introduces the calibration source energy and a black

direct-current restoration surface into the 100 detector fields of

view.

The calibration signals for bands 1 - 5 and band 7 are derived from three regulated tungsten-filament lamps. The calibration source for band 6 is a blackbody with three temperature selections, commanded from the ground. The method for transmitting radiation to the moving calibration shutter allows the

tungsten lamps to provide radiation independently and to contribute

proportionately to the illumination of all detectors.

4.2.1  Specifications

The following spectral bands are collected by the TM sensor:

Channel    Wavelength (um)                   Primary Use

-------    ---------------   -----------------------------------------

1        0.45 - 0.52        Coastal water mapping, soil vegetation

differentiation, deciduous/coniferous

differentiation.

2        0.52 - 0.60        Green reflectance by healthy vegetation.

3        0.63 - 0.69        Chlorophyll absorption for plant species

differentiation.

4        0.76 - 0.90        Biomass surveys, water body delineation.

5        1.55 - 1.72        Vegetation moisture measurement, snow

cloud differentiation.

6        10.4 - 12.5        Plant heat stress measurement, other

thermal mapping.

7        2.08 - 2.35        Hydrothermal mapping.

Band             Radiometric

Sensitivity [NE(dP)]*

----           --------------------

1                  0.8%

2                  0.5%

3                  0.5%

4                  0.5%

5                  1.0%

6                  0.5 K [NE(dT)]

7                  2.4%

Ground IFOV             30 m (bands 1-5, 7)

120 m (band 6)

Avg. Altitude          699.6 km

Data Rate                 85 Mbps

Quantization levels      256

Orbit angle             8.15 degrees

Orbital Nodal Period   98.88 minutes

Scan width               185 km

Scan angle              14.9 degrees

Image overlap            7.6 %

*  N.B. The radiometric sensitivities are the noise-equivalent

reflectance differences for the reflective channels expressed as

percentages [NE(dP)] and temperature differences for the thermal

infrared bands [NE(dT)].

4.2.1.1  Tolerance

The TM channels were designed for a noise equivalent differential represented by the radiometric sensitivity shown in Section 4.2.1.

4.2.2  Frequency of Calibration

The absolute radiometric calibration between bands on both sensors is maintained

by using internal calibrators that are physically located between the telescope

and the detectors and are sampled at the end of a scan.

4.2.3  Other Calibration Information

Relative within-band radiometric calibration, to reduce "striping", is provided

by a scene-based procedure called  histogram equalization. The absolute accuracy

and relative  precision of this calibration scheme assumes that any change  in

the optics of the primary telescope or the "effective radiance" from the

internal calibrator lamps is insignificant in  comparison to the changes in

detector sensitivity and electronic gain and bias with time and that the scene-

dependent sampling is  sufficiently precise for the required within-scan

destriping from  histogram equalization.  Each TM reflective band and the

internal calibrator lamps were calibrated prior to launch using lamps in

integrating spheres that were in turn calibrated against lamps traceable to

calibrated National Bureau of Standards lamps. Sometimes the absolute

radiometric calibration constants in the "short term" and "long-term parameters"

files used for ground processing have been modified after launch because

of inconsistency within or between bands, changes in the inherent dynamic range

of the sensors, or a desire to make quantized and calibrated values from one

sensor match those from another.

5. Data Acquisition Methods

These data were acquired from the Landsat-5 TM sensor and received from the

Canadian Centre for Remote Sensing (CCRS) who purchased it from the Earth

Observation Satellite Company (EOSAT). As received from CCRS, the image had been

processed from raw telemetry to a systematically corrected product within the

CCRS MOSAICS system.  After original delivery to the BOREAS data system, CCRS

reprocessed these data which produced minor differences in the pixel values.

The data that were used to produce this data product are from the original data

delivery, not the TM image product that currently exists in the BOREAS data set.

6.  Observations

6.1  Data Notes

This imagery was collected on 21-Jun-1995.  This scene is Path 33, Row 21 in the

Landsat Worldwide Reference System (WRS).  The solar elevation angle at the time

of image acquisition was 40.1 degrees.  The solar azimuth angle was 146 degrees.

The radiometric quality of this imagery was acceptable.

The TM image from which this classification was produced was atmospherically

corrected using aerosol optical thickness data measured by sunphotometers in

the study area.  These optical thickness data were used in the Second Simulation of the Satellite Signal in the Solar Spectrum (6S) program to determine the spherical albedo, path radiance, gaseous transmission, and scattering transmission.  These parameters were used to determine surface reflectance based on equations 4a and 4b of Markham, et al. (1992).

6.2  Field Notes

Not applicable.

7. Data Description

7.1  Spatial Characteristics

7.1.1  Spatial Coverage

The classified image covers an area that is approximately 129 km by 86 km and includes areas just west of Thompson, Manitoba.  The corners of the data set are as follows.  These coordinates are in the BOREAS Grid projection.

BOREAS Grid             NAD83

Corner         X       Y         Long.     Lat.

---------------------------------------------------

Northwest    740.000   650.000     98.983W  56.262N

Northeast    850.010   650.000     97.240W  56.081N

Southwest    740.000   569.990     99.202W  55.555N

Southeast    850.010   569.990     97.489W  55.377N

7.1.2 Spatial Coverage Map

Not available.

7.1.3  Spatial Resolution

Each pixel represents a 30-meter by 30-meter area on the ground.

7.1.4 Projection

The area mapped is projected in the BOREAS Grid projection which is based on the ellipsoidal version of the Albers Equal Area Conic (AEAC) projection.  The projection has the following parameters:

Datum:  NAD83

Ellipsoid:  Geodetic Reference System of 1980 (GRS80) or Worldwide Geodetic

System of 1984 (WGS84)

Origin:   111.000°W   51.000°N

Standard Parallels:   52°  30' 00"N

58°  30' 00"N

Units of Measure:  kilometers

7.1.5 Grid Description

The data are referenced to the projection described in section 7.1.4.

7.2  Temporal Characteristics

7.2.1  Temporal Coverage

This original spectral imagery was collected on 21-Jun-1995.  The scene is from Path 33, Row 21 in the Landsat WRS.  The solar elevation angle at the time of image acquisition was 38.2 degrees.  The solar azimuth angle was 136.3 degrees.  The radiometric quality of this imagery was acceptable.

7.2.2 Temporal Coverage Map

Not applicable.

7.2.3  Temporal Resolution

This data set represents the land cover as it appeared on 21-Jun-1995.

7.3 Data Characteristics

7.3.1 Parameter/Variable

Land cover type.

7.3.2  Variable Description/Definition

Each pixel in the classification image contains a number between 0 and 13.  This number represents one of the following land cover classes:

Class Descriptions

0     No Data

This area is not covered in the classification.  This area is most likely blank fill on the edges of the image frame.

1	Conifer (Wet)

Primarily black spruce and jack pine on three major different soil substrates: (i) moderately well drained soils with feather moss over clay, (ii) poorly drained soils with sphagnum on clay, and (iii)  sparsely treed fens with a very deep moss layer. Overstory biomass density varies considerably within this class.

2	Conifer (Dry)

Dry Conifer is an area that contains coniferous trees (primarily jack pine) with a lichen (cladina) background.  These areas have sandy soils that are well drained.  Areas of permafrost supporting conifers with a lichen background are also included in this class.

3	Mixed Deciduous and Coniferous

Mixed deciduous and coniferous contains coniferous and aspen/birch (populus tremuloides/betula papyrifera) trees.  The composition of this class contains less than 80% of the dominant species.

4	Deciduous

The deciduous class contains primarily aspen/birch.  The composition of this class is generally greater than 80% deciduous trees.

5	Fen

The Fen/Bog class is characterized by areas with a water table very near or at the surface.  Fens experience lateral water transport, whereas bogs are enclosed landforms experiencing only vertical transport.  Fens typically contain sedges, moss, and bog birch associated with sparse to medium dense tamarack (larix laricina) stands.  Bogs are usually

6	Water

Water bodies such as ponds, lakes, and streams.

7	Disturbed

The disturbed class consists of areas that are dominated by bare soil, recently logged areas, or rock outcrops.  This class also includes roads, airports, and urban areas.

8	Fire Blackened

Areas that have been burned in the last 5 or 6 years.  Distinguishable for their charred sphagnum background they are usually areas of very intense burn where little or no vegetation survived.

9      New Regeneration Conifer

This class consists primarily of conifers that are regrowing after a burn.  It may also include conifer stands where there are a few remaining

trees after a low- to medium-intensity burn.

10      Medium-Age Regeneration Conifer

Areas that are predominantly young jack pine or young black spruce. This class typically occurs in stands that were cleared or burned and have been growing back for approximately 10 years.

11      New Regeneration deciduous

This class consists of aspen that is starting to regrow after a recent clearing.  This class is younger than the young aspen class.  The aspen in this class may also include grasses or other herbaceous vegetation.

12      Medium-Age Regeneration deciduous

The class consists of areas that were cleared or burned and have been growing back as aspen.  These stands typically contain 10 year old aspen where the background is almost completely obscured and thinning has not yet taken place.

13      Grass

This class consists primarily of grasses, agricultural fields that have been planted, or shrub-like vegetation.

7.3.3 Unit of Measurement

Unitless but coded value.

7.3.4 Data Source

Landsat-5 TM scene on 21-Jun-1995 from the CCRS

7.3.5  Data Range

Land cover type: 13 different land cover classes (pixel values from 0 to 13).

7.4  Sample Data Record

Not applicable for image data.

8. Data Organization

8.1  Data Granularity

The smallest amount of data that can be ordered is the entire data set.

8.2  Data Format

8.2.1 Uncompressed Data Files

The NSA classification product contains two files as follows:

File 1: (80-byte American Standard Code for Information Interchange (ASCII) text

records)

Text file listing the files on tape.

File 2: (2,667 records of 3,667 bytes each)

(1 byte per pixel)

Classified image with values from 0 to 13.

8.2.2 Compressed CD-ROM Files

On the BOREAS CD-ROMs, file 1 listed above is stored as ASCII text; however, file 2 has been compressed with the Gzip compression program (file name *.gz).  These data have been compressed using gzip version 1.2.4 and the high compression (-9) option (Copyright (C) 1992-1993 Jean-loup Gailly).  Gzip (GNU zip) uses the Lempel-Ziv algorithm (Welch, 1994) used in the zip and PKZIP programs. The compressed files may be uncompressed using gzip (-d option) or gunzip.  Gzip is available from many websites (for example, ftp site prep.ai.mit.edu/pub/gnu/gzip-*.*) for a variety of operating systems in both executable and source code form.  Versions of the decompression software for various systems are included on the CD-ROMs.

9. Data Manipulations

9.1 Formulae

Not applicable.

9.1.1  Derivation Techniques and Algorithms

The techniques that were used to classify this image are described in sections

1.5, 3, and 6.1.

9.2 Data Processing Sequence

9.2.1  Processing Steps

1) The imagery was converted to surface reflectance before the classification

was performed.  Atmospheric correction coefficients were computed using

optical depths from a sunphotometer in conjunction with 6S (Markham et al.,

1992).

2) End member reflectances were the same as those used for the Southern Study

Area (SSA) classification.

3) Trajectories were computed based on end member reflectances, solar geometry,

tree height to width ratio, and tree form (i.e. cone or cylinder).

4) Additional trajectories for regeneration classes were added using data from

regeneration areas of the SSA.  No end member reflectances were used to

characterize the regeneration and water classes (classes 6 - 13).

5) The trajectories were used as input to the image classifier.

6) Post-processing techniques to classify any remaining null-classed pixels were

applied.

7) The classified image was mapped into the AEAC projection using nearest

neighbor resampling.

8) The classification image was written to tape.

9) Copy the ASCII and compress the binary files for release on CD-ROM.

9.2.2  Processing Changes

None.

9.3 Calculations

9.3.1  Special Corrections/Adjustments

None.

9.4 Graphs and Plots

None.

10. Errors

10.1  Sources of Error

The sources of error in this classification can be attributed to several factors.  In many cases, the reflectance of one feature could be similar to the reflectance of another feature, resulting in confusion.  The similarity in reflectances could be the result of similar background components and variations in tree density.  Error could also be a result of spectral mixing of various features that fall within a 30 meter pixel.

10.2   Quality Assessment

10.2.1  Data Validation by Source

The imagery was spot checked at various locations and the image

class was compared to the forest cover map.

An error assessment was performed on the classification.  The auxiliary sites and a few randomly selected sites were used as ground truth.  The location of each ground truth site was identified on the georeferenced image as a 3 by 3 pixel area.  Each of the 9 pixels in these areas represents a test point.  Some classes were not represented by auxiliary sites or randomly selected sites.

10.2.2  Confidence Level/Accuracy Judgment

Although efforts have been made to make this classification as accurate as possible, there is bound to be some confusion between classes.  In some areas, new regeneration conifer can be confused with fen because of differences in canopy density.  Also, many of the age classes within the deciduous or conifer classes can be confused because of minor variations in background.

10.2.3  Measurement Error for Parameters

The following tables and statistics were derived to assess the accuracy of the classification:

Confusion Matrix

Classification

Class   1    2    3    4    5    6   7   8    9   10   11   12   13

Truth

----------------------------------------------------------------------------

Wet Conifer(1)  93   0    0    8    0    0   0   0    0    0    9    0    0

Dry Conifer(2)   0   0    0    0    1    0   0   0    7    0    0    0    0

Mixed      (3)   0   0   21    5    0    0   0   0    0    0    2    2    0

Deciduous  (4)   0   0    0   85    0    0   0   0    1    0    0    0    0

Fen        (5)   0   0    6   22   80    0   0   0    0    0    9    0    0

Water      (6)   0   0    0    0    0   51   0   0    0    0    0    0    0

Bare Soil  (7)   0   1    0    0    0    0  18   0    0    0    0    0    0

Fire Black.(8)   0   0    0    0    0    0   0   0    0    0    0    0    0

New Regen.

Conifer    (9)   0   0    0    3    1    0   0   0   67    0    0    0    0

Med. Age

Regen. Con.(10)  0   0    7    0    0    0   0   0    0   47    0    0    0

New Regen.

Deciduous  (11)  0   0    0    0    9    0   0   0    0    0   39    0    0

Med. Age

Regen.

Deciduous  (12)  0   0    0    0    0    0   0   0    0    0    0   21    0

Grass      (13)  0   0    0    0    0    0   0   0    0    0    0    0    0

Class           %

Correct

Wet Conifer      84 %

Dry Conifer       0 %

Mixed            70 %

Deciduous        99 %

Fen              68 %

Water           100 %

Bare Soil        95 %

Fire Blackened  Not represented in NSA

New Regen.

Conifer          94 %

Med. Age

Regen. Con.      87 %

New Regen.

Decid.           81 %

Med. Age

Regen.

Decid.          100 %

Grass           Not represented in NSA

Overall          85 %

Kappa = 0.83 or 83 % better than chance agreement (Campbell, 1987).

10.2.4  Additional Quality Assessments

None.

10.2.5 Data Verification by Data Center

The imagery was spot checked at various locations and the image class was

compared to the forest cover maps from Manitoba Natural Resources.

11. Notes

11.1  Limitations of the Data

This data set is based on an image that was collected on 21-Jun-1995 and only

represents the land cover as it existed on that day.

Please see Section 10.2.1 to determine how the amount of error in this product

may affect your results from using it.

11.2   Known Problems With the Data

Clouds in this classification show up in the disturbed class, and cloud shadows show up in the water class.  The scene is mostly clear, so this problem has a very limited impact.

11.3 Usage Guidance

Before uncompressing the Gzip files on CD-ROM, be sure that you have enough disk space to hold the uncompressed data files.  Then use the appropriate decompression program provided on the CD-ROM for your specific system.

11.4 Other Relevant Information

None.

12. Application of the Data Set

This data set may be used for modeling purposes.  It can also be used to analyze measurements from aircraft to determine the land cover that was under the aircraft at locations along the aircraft’s path.

13. Future Modifications and Plans

None.

14.  Software

14.1 Software Description

Programs written at NASA GSFC to run under EASI/PACE image processing software

from PCI, Inc. were used to classify the image.  The trajectories were computed

using Microsoft Excel (Version 4.0).

Questions related to the specific details of the software written to process

this data set should be addressed to David Knapp (see Section 2.3).

Microsoft Excel (Version 4.0) is a spreadsheet program.

Gzip (GNU zip) uses the Lempel-Ziv algorithm (Welch, 1994) used in the zip and PKZIP commands.

14.2 Software Access

EASI/PACE is a proprietary software package developed by PCI, Inc.  Contact PCI

for details.

PCI, Inc.

50 West Wilmot St.

Richmond Hill

Ontario, Canada    L4B 1M5

Phone: (905) 764-0614

FAX: (905) 764-9604

Microsoft Excel is a proprietary software package that is widely available in

the commercial software market.

Gzip is available from many websites across the net (for example) ftp site prep.ai.mit.edu/pub/gnu/gzip-*.*) for a variety of operating systems in both executable and source code form. Versions of the decompression software for various systems are included on the CD-ROMs.

15. Data Access

15.1 Contact Information

Ms. Beth Nelson

BOREAS Data Manager

NASA GSFC

Greenbelt, MD

(301) 286-4005

(301) 286-0239 (fax)

Elizabeth.Nelson@gsfc.nasa.gov

15.2 Data Center Identification

See Section 15.1

15.3 Procedures for Obtaining Data

Users may place data requests by telephone, electronic mail, or fax.

15.4 Data Center Status/Plans

The NSA physical classification is available from the EOSDIS ORNL DAAC (Earth Observing System Data and Information System) (Oak Ridge National Laboratory) (Distributed Active Archive Center).

The BOREAS contact at ORNL is:

ORNL DAAC User Services

Oak Ridge National Laboratory

Oak Ridge, TN

(423) 241-3952

ornldaac@ornl.gov

ornl@eos.nasa.gov

16. Output Products and Availability

16.1 Tape Products

These data can be made available on 8mm, DAT, or 9-track tapes.

16.2 Film Products

None.

16.3 Other Products

These data are available on the BOREAS CD-ROM series.

17. References

17.1 Platform/Sensor/Instrument/Data Processing Documentation

Hall, F.G., D.E. Knapp and K.F. Huemmrich. Physically-Based Classification and Satellite Mapping of Biophysical Characteristics in the Southern Boreal Forest.JGR.  BOREAS Special Issue (in press).

Hall, F.G., Y.E. Shimabukuro and K.F. Huemmrich. 1995.  Remote sensing of forest  biophysical structure using mixture decomposition and geometric reflectance models.  Ecological Applications 5(4):993-1013.

Markham, B.L., R.N. Halthornea and S.J. Goetz. 1992. Surface reflectance retrieval from satellite and aircraft sensors: Results of sensor and algorithm comparisons during FIFE.  FIFE Special Issue. American Geophysical Union. 18785-18795.

PACE Image Analysis Kernal Version 5.2. 1993. PCI Inc. Richmond Hill, Ontario.

Richards, J. A. 1986. Remote Sensing Digital Image Analysis: An Introduction. Springer Verlag.

Welch, T.A. 1984, A Technique for High Performance Data Compression, IEEE Computer, Vol. 17, No. 6, pp. 8 - 19.

17.2 Journal Articles and Study Reports

Campbell, J. B. 1987. Introduction to Remote Sensing. Guilford Press. p.349.

Sellers, P. and F. Hall. 1994. Boreal Ecosystem-Atmosphere Study: Experiment Plan. Version 1994-3.0, NASA BOREAS Report (EXPLAN 94).

Sellers, P. and F. Hall. 1996. Boreal Ecosystem-Atmosphere Study: Experiment Plan. Version 1996-2.0, NASA BOREAS Report (EXPLAN 96).

Sellers, P. J., F. G. Hall, R. D. Kelly, A. Black, D. Baldocchi, J. Berry, M. Ryan, K. J. Ranson, P. M. Crill, D. P.  Lettenmaier, H. Margolis, J. Cihlar, J. Newcomer, D. Fitzjarrald, P. G. Jarvis, S. T. Gower, D. Halliwell, D. Williams, B. Goodison, D. E. Wickland, and F. E. Guertin. 1997. BOREAS in 1997: Experiment Overview, Scientific Results and Future Directions. Journal of Geophysical Research 102 (D24): 28, 731-28,770.

Sellers, P., F. Hall, and K.F. Huemmrich. 1996. Boreal Ecosystem-Atmosphere Study: 1994 Operations. NASA BOREAS Report (OPS DOC 94).

Sellers, P., F. Hall, and K.F. Huemmrich. 1997. Boreal Ecosystem-Atmosphere Study: 1996 Operations. NASA BOREAS Report (OPS DOC 96).

Sellers, P., F. Hall, H. Margolis, B. Kelly, D. Baldocchi, G. den Hartog, J. Cihlar, M.G. Ryan, B. Goodison, P. Crill, K.J. Ranson, D. Lettenmaier, and D.E. Wickland. 1995. The boreal ecosystem-atmosphere study (BOREAS): an overview and early results from the 1994 field year. Bulletin of the American Meteorological Society. 76(9):1549-1577.

17.3 Archive/DBMS Usage Documentation

None.

18. Glossary of Terms

None.

19. List of Acronyms

AEAC      - Albers Equal-Area Conic

ASCII     - American Standard Code for Information Interchange

BOREAS    - BOReal Ecosystem-Atmosphere Study

BORIS     - BOREAS Information System

BPI       - Bytes per inch

CCRS      - Canadian Centre for Remote Sensing

CD-ROM    - Compact Disk-Read-Only Memory

DAAC      - Distributed Active Archive Center

DAT       - Digital Archive Tape

DEM       - Digital Elevation Model

EOS       - Earth Observing System

EOSAT     - Earth Observation Satellite Company

EOSDIS    - EOS Data and Information System

GMT       - Greenwich Mean Time

GRS80     - Geodetic Reference System of 1980

GSFC      - Goddard Space Flight Center

IFOV      - Instantaneous Field of View

MSA       - Modeling Sub-Area

MSS       - Multispectral Scanner

NAD27     - North American Datum of 1927

NAD83     - North American Datum of 1983

NASA      - National Aeronautics and Space Administration

NSA       - Northern Study Area

PANP      - Prince Albert National Park

ORNL      - Oak Ridge National Laboratory

SSA       - Southern Study Area

RSS       - Remote Sensing Science

6S        - Second Simulation of the Satellite Signal in the Solar Spectrum

TE        - Terrestrial Ecology

TM        - Thematic Mapper

URL       - Uniform Resource Locator

UTM       - Universal Transverse Mercator

WGS84     - World Geodetic System of 1984

WRS       - Worldwide Reference System

WWW       - World Wide Web

20. Document Information

20.1 Document Revision Dates

Written:         06-Apr-1995

Last Updated:    30-Jul-1998

20.2 Document Review Dates

BORIS Review:    09-Jan-1998

Science Review:

20.3 Document ID

20.4 Citation

This classification image was produced for the BOREAS project as part of the research of Dr. Forrest Hall of NASA Goddard Space Flight Center.

Please contact Dr. Hall or David Knapp before using these data in a publication.

20.5 Document Curator

20.6 Document URL

Keywords

LAND COVER

LANDSAT

TM

CLASSIFICATION