BOREAS TGB-01 NSA CH4 and CO2 Chamber Flux Data

Summary

The BOREAS TGB-01 team made methane (CH4) and carbon dioxide (CO2) dark chamber flux measurements at the NSA-OJP, NSA-OBS, NSA-BP, and NSA-YJP sites from 16-May-1994 through 13-Sep-1994. Gas samples were extracted approximately every 7 days from dark chambers and analyzed at the NSA lab facility.  The data are provided in tabular ASCII files.

Table of Contents

*  1 Data Set Overview

*  2 Investigator(s)

*  3 Theory of Measurements

*  4 Equipment

*  5 Data Acquisition Methods

*  6 Observations

*  7 Data Description

*  8 Data Organization

*  9 Data Manipulations

*  10 Errors

*  11 Notes

*  12 Application of the Data Set

*  13 Future Modifications and Plans

*  14 Software

*  15 Data Access

*  16 Output Products and Availability

*  17 References

*  18 Glossary of Terms

*  19 List of Acronyms

*  20 Document Information

1. Data Set Overview

1.1 Data Set Identification

BOREAS TGB-01 NSA CH4 and CO2 Chamber Flux Data

1.2 Data Set Introduction

Chamber flux measurements were taken at the Northern Study Area (NSA) Old Jack Pine (OJP), Young Jack Pine (YJP), Old Black Spruce (OBS), and Beaver Pond (BP) sites during the summer of 1994 by the BOReal Ecosystem-Atmosphere Study (BOREAS) Trace Gas Biogeochemistry (TGB)-01 team.  The values represent the average/mean flux from five values taken at 20-minute intervals.

1.3 Objective/Purpose

The purpose of these measurements was to examine the trace gas exchange between the atmosphere and the boreal soils.

1.4 Summary of Parameters

CH4 and CO2 chamber fluxes were measured at the NSA OJP, YJP, OBS, and BP sites.  Temperatures of the chamber, 1-cm soil depth, and 10-cm soil depth were recorded at the OJP, YJP, and OBS (Patrick Crill) sites.  At the OBS (Dean Moosavi) and BP sites, temperatures at 5-, 10-, and 20-cm soil depths were recorded.

1.5 Discussion

The chamber flux data are provided in American Standard Code for Information Interchange (ASCII) comma delimited text files.  The data are a compilation of measurements from the aluminum chamber and collar sites at OJP, YJP and OBS (Patrick Crill) and the plastic chamber and collar sites at OBS (Dean Moosavi) and BP sites.  Another part of the data is a diurnal study completed 08-Jul-94 and 09-Jul-1994 at the BP plastic collar and chamber sites.

1.6 Related Data Sets

BOREAS TGB01 CH4 Tower flux data over NSA

BOREAS TGB01 Soil CH4 and CO2 concentration data over the NSA.

2. Investigator(s)

2.1 Investigator(s) Name and Title

Dr. Patrick M. Crill

Research Associate Professor

University of New Hampshire

2.2 Title of Investigation

Magnitude and Control of Trace Gas Exchange in Boreal Ecosystems

2.3 Contact Information

Contact 1

Dr. Patrick M. Crill

Institute for the Study of Earth, Oceans, and Space

Complex Systems Research Center

University of New Hampshire

Durham, NH

(603) 862-3519

(603) 862-0188 (fax)

Contact 2

Sadredin C. Moosavi

Graduate Student

Institute for the Study of Earth, Oceans, and Space

Complex Systems Research Center

University of New Hampshire

Durham, NH

(603) 862-2927

(603) 862-0188 (fax)

Contact 3

Ruth K. Varner

Research Scientist

Institute for the Study of Earth, Oceans, and Space

Complex Systems Research Center

University of New Hampshire

Durham, NH

(603) 862-2939

(603) 862-0188 (fax)

Contact 4

Sara K. Golightley

Raytheon STX Corporation

NASA GSFC

Greenbelt, MD

301-286-2624

sgdight@pop900.gsfc.nasa.gov

3. Theory of Measurements

Chamber fluxes measure the changes in the mixing ratio of trace gases (CO2 and CH4) in a closed headspace over a period of time.  This headspace is isolated from the atmosphere; therefore, we can quantify the exchange of material between the covered soil and the headspace can be quantified.

4. Equipment

4.1 Sensor/Instrument Description

CH4 was quantified with a Shimadzu GC-14A Gas Chromatograph (GC) or a Shimadzu GC-MINI2 with a flame ionization detector (FID) operated at 125 degrees C after separation on a HayeSepQ column at 40 degrees C using ultrapure (99.999%) N2 as a carrier gas flowing at 30 mL/min.  CO2 was quantified with a Shimadzu GC-14A with a thermal conductivity detector (TCD) operated at 70 degrees C after separation on a HayeSepQ column at 40 degrees C using ultrapure (99.999%) He as a carrier gas flowing at 30 mL/min.  The oven temperature during sample analysis was run at 130 degrees C and the detector at 300 degrees C. Analog signals (0-1 V) from the detectors were digitized at 10 Hz with a Hewlett Packard (HP) 35000D A/D board and quantified and logged using HP ChemStation software.  Chamber fluxes were accomplished with aluminum chambers manufactured at the University of New Hampshire and designed by Patrick Crill.

4.1.1 Collection Environment

The chamber fluxes were collected under ambient conditions.  The GC analysis was completed at the Hayes Road Lab in Thompson, Manitoba.

4.1.2 Source/Platform

Ground.

4.1.3 Source/Platform Mission Objectives

This mission objective was to determine the soil-surface exchange rates of CH4 and CO2 at the NSA OJP, YJP, OBS, and BP sites.

4.1.4 Key Variables

The key variables measured during the sampling period were CH4 and CO2 fluxes.  The OBS site was divided into two different sets of data.  In the SITE_COLLAR, a C denotes the OBS site where Patrick Crill measured fluxes with aluminum chambers and collars.  An M denotes the OBS site where Dean Moosavi measured fluxes with plastic chambers and collars.  Plastic chambers and collars were also used at the BP site.  Soil profile temperatures were also recorded.

4.1.5 Principles of Operation

The Shimadzu GC-14A GC is equipped with a hydrogen FID and a TCD.  The FID is used to detect CH4, while the TCD is used to detect CO2.  The FID uses a hydrogen flame in an air atmosphere to burn components as they exit the column.  In the flame, carbon-carbon bonds are fragmented so that various organic ions and free electrons exist.  Application of a voltage across a collector electrode over the flame causes an ion current to flow, which is amplified and then measured as the output signal.

The TCD detects CO2 by passing a sample, in a helium carrier gas, past metallic filaments with current flowing through them.  The sample components with lower thermal conductivity than the helium carrier gas raise the filament temperature when they pass through.  The signal output from the TCD is a measurement of the change in filament resistance caused by the temperature rise.  The signal output from both the FID and TCD is for a data processor, integrator, recorder, or computer (Instruction Manual: GC-14A; Shimadzu Corporation, Kyoto, Japan).  The GC-MINI2 was equipped with a FID and operated in the same manner as the GC-14A FID.

4.1.6 Sensor/Instrument Measurement Geometry

Not applicable.

4.1.7 Manufacturer of Sensor/Instrument

Manufacturer of GC-14A FID/TCD, GC-MINI2, and GC-8A ECD

Shimadzu Scientific Instruments, Inc.

7102 Riverwood Drive

Columbia, MD 21046

(410) 381-1227

4.2 Calibration

4.2.1 Specifications

None given.

4.2.1.1 Tolerance

The sensitivity of the TCD is approximately 6000 mV ml/mg.  The FID's maximum sensitivity is 3 x 10-12 g/s for diphenyl.

4.2.2 Frequency of Calibration

The instrument is calibrated on a daily basis.  Standards are run generally before and after samples on a given day of analysis.

4.2.3 Other Calibration Information

Signal peaks from the detectors were quantified with working standards calibrated against Canadian Atmospheric Environment Services (AES) certified primary standards acquired by the BOREAS project and a CO2/CH4 standard of Niwot Ridge air prepared by the National Oceanic and Atmospheric Administration (NOAA) Climate Monitoring and Diagnostics Laboratory (CMDL).  Uncertainty in the standards’ analyses on a given day ranged from 0.1 to 0.2%.

5. Data Acquisition Methods

The chamber fluxes are determined by analysis of concentrations of methane (CH4) and carbon dioxide (CO2) in a time series of grab samples of headspace over the ground surface enclosed by a dark aluminum chamber.  The volumes of the two aluminum chambers were 0.071 and 0.120 m3 over an area of 0.397 m2.  The aluminum chamber was placed on the trough of an aluminum collar embedded in the ground.  Water was added to the trough of the collar to create an airtight seal.  The volume of the plastic chambers was 0.028 m3 over an area of 0.078 m2. The seal between chamber and collar was made using weather stripping or water as appropriate.  Five 60-mL samples were removed from the headspace with polypropylene syringes and polycarbonate/nylon stopcocks at 4-minute intervals for 20 minutes (five samples). Samples were returned to the Hayes Road lab and analyzed for CO2 and CH4 using gas chromatography within 12 hours after collection.

6. Observations

6.1 Data Notes

None given.

6.2 Field Notes

For data collected by Patrick Crill:

25-May-94- Fans in chambers not working at OBS.  Can’t use the data.

29-May-94- Fan in chamber not working after 20 minutes in OJPA2 flux; no fan for the OJPM2 flux; no fan for the OJP1 and 2 fluxes.

01-Jun-94- No fan for OBS12 flux.

20-Jun-94- Thermometer probe outside of chamber for collar OBS1.

10-Jul-94- One chamber had been filled with water and therefore the fan and temperature probe aren’t working.  Used it with collars YJP1, OJPA1, OJPL2, OJPM1, and OJPM2.

16-Jul-94- No temperature still in one chamber at OJP and YJP.

23-Jul-94- No temperature still in one chamber at OJP and YJP; Collar YJP2 syringe number 4 lost its stopcock.

30-Jul-94- No temperature still in one chamber at OJP and YJP.

01-Aug-94- Very hot, using silver mylar to reflect sun on half of the flux chambers to see if it effects temperature change in chamber...it doesn’t heat up as much as chambers without cover by about 10 oC.

07-Aug-94- Fixed chamber thermometer for first few fluxes; then it went bad again.

08-Aug-94- No fan for collar OBS10 flux; no temperatures for OBS8, OBS6, OBS11, OBS4, and OBS2

12-Aug-94- No temperature still in one chamber at OJP and YJP.

19-Aug-94- No temperature still in one chamber at OJP and YJP.

For data taken by Dean Moosavi:

19-May-94- Experiments halted due to chamber hardware problems.

20-May-94- Screw missing in chamber for flux at collar OBS1; therefore, there was a hole in chamber during the flux.  Fixed with duct tape.

26-May-94- Problems with collar at collar BP29.

04-Jun-94- Water levels in the fen have dropped 5-10 cm from season beginning at OBS.

10-Jun-94- Carex and other vegetation have sprouted.  Larch are out.

20-Jun-94- Temperature probe dead!

21-Jun-94- Fans did not work for collars 19 and 20 at BP.

30-Jun-94- BP- Bear attack on equipment!  Collar 23 pulled up and moved 5 yards.  Minor tooth damage.  Collar 18 destroyed.  Lichen thermocouple uprooted and destroyed.  Pond and dam level have dropped 10 cm leaving collars above the water level.

02-Aug-94- BP- Moose stepped in collar 24!

21-Aug-94- Senescence beginning.

24-Aug-94- Due to problems with yesterday’s samples. BP and dam were resampled off collar in vicinity of site.

31-Aug-94- BP and dam fluxes taken next to (NOT on) collars.

13-Sep-94- BP collars 25-32 fluxes taken near (not on) collars.

14-Sep-94- All collars and chambers removed!

7. Data Description

7.1 Spatial Characteristics

7.1.1 Spatial Coverage

The collars at the OJP tower site and profile sampling sites were located as follows:  collars located on moss plots were approximately northwest of the tower, while the profile corral was located a few meters south of the collars (55o55’43.2” N, 98o37’29.4” W).

The collars at the OJP site located on lichen plots were due west of the tower; the profile trench was located about 1 m southeast of the collars (55o55’40.9” N, 98o37’29.3” W).

The collars at the YJP tower site were located southeast of the tower;  the gas profile corral was located approximately 1 m west of the collars (55o53’42.8” N, 98o17’12.7” W).

The collars at the OBS tower site were located along a low to high moisture gradient from the lichen to the feathermoss, the fen rim and ending at the fen site.  The lichen and the feather moss collars were located along the boardwalk approximately 200 m northeast of the tower.  The fen rim and fen site collars were located due east of the lichen and feather moss sites about 150 m and 200 m respectively.

For data collected by Dean Moosavi:

The collars and profile sampling sites near the OBS tower were located as follows:  the OBS sampling collars were located along a moisture gradient from feathermoss and lichen sites to the fen rim site and ending with the fen sites.  The transect ran approximately east to west and was located about 200 m northeast of the tower at the OBS site.

The collars and profile sampling sites at the BP tower site were located as follows:  the BP sampling sites were located along a low to high moisture gradient that began with the upland lichen site, then the sphagnum moss site, the mire site, and ending at the pond site.  The transect ran approximately north to south and was about 60 m from the BP tower.

7.1.2 Spatial Coverage Map

Not available.

7.1.3 Spatial Resolution

Four collars were placed in the ground for each biome type.  They were approximately 1 to 2 m apart depending on the site specifics.  In Section 7.3.3, the cover type refers to the predominant vegetation in the collar.  These are point source measurements at the locations given in Section 7.1.1.

7.1.4 Projection

Not applicable.

7.1.5 Grid Description

Not applicable.

7.2 Temporal Characteristics

7.2.1 Temporal Coverage

Chamber fluxes were measured from 16-May-1994 through 13-Sep-1994.  One diurnal study was conducted at eight collars on 8- and 9-Jul-1994 at the BP site..

7.2.2 Temporal Coverage Map

Not available.

7.2.3 Temporal Resolution

The chamber fluxes were measured approximately every 7 days from 23-May-1994 to 20-Sep-1994.

7.3 Data Characteristics

Data characteristics are defined in the companion data definition file (tgb1ccfd.def).

7.4 Sample Data Record

Sample data format shown in the companion data definition file (tgb1ccfd.def).

8. Data Organization

8.1 Data Granularity

All of the NSA CH4 and CO2 Chamber Flux Data are contained in one dataset.

8.2 Data Format(s)

The data files contain numerical and character fields of varying length separated by commas. The character fields are enclosed with  a single apostrophe marks. There are no spaces between the fields.  Sample data records are shown in the companion data definition files (tgb1ccfd.def).

9. Data Manipulations

9.1 Formulae

Rf = Cstd/Astd

Cs = Rf * As

Rf = Response factor

Astd = Average of 10 standard peak areas

Cstd = Concentration of the standard

Cs = Concentration of the sample

As = Peak area of sample

CH4 and CO2 concentrations were calculated from the average of 10 peak areas of known CH4 and CO2 standards.  The response factor was calculated as the concentration of the known standard divided by the average of 10 standard peak areas.  The peak area of the unknown sample was multiplied by the response factor.  Fitting a regression curve to the time series of CH4 and CO2 concentrations made the flux calculations.  The flux rate of a gas is calculated using the following equation:

F= ppmv/min*(P/R*MW)*1/T*Vc/Ac*C

Where:

F = flux (mg/m2/d)

P = Chamber pressure (atm)

R = 8.2054 x 10-5 m3 atm mol-1K-1

MW = Molecular weight of the analyte

T = Chamber temperature (K)

Vc = Chamber volume (m3)

Ac = Chamber area (m2)

C = (1000 mg/g*1440 min/d)

Gases:

CH4 = 16 g/mol

CO2 = 44 g/mol

At sites where oxidation of CH4 occurs, the flux starts at or near ambient CH4 concentrations and is depleted below ambient concentrations. If the correlation coefficients were not significantly high, the flux was determined to be below the detection limit and was regarded as a zero flux rate.  If there was an efflux of CH4 was measured at any of these sites, those data were eliminated.

For sites where CH4 and CO2 were known to have a positive flux (from the soil into the atmosphere) the regressions of the time series were expected to have an r2 of greater than 0.85.

If -888 is present in the data set, it indicates that a measurement was taken but discarded for some reason.  If -999 is present, then no data were taken.

9.1.1 Derivation Techniques and Algorithms

Not applicable.

9.2 Data Processing Sequence

Not applicable.

9.2.1 Processing Steps

The peak areas were taken directly from the HP ChemStation reports from the GC.  They were entered into spreadsheets and the concentrations were calculated by the formulas in Section 9.1.  The spreadsheets then automatically calculate the flux using the formulas.

9.2.2 Processing Changes

None given.

9.3 Calculations

Not applicable.

9.3.1 Special Corrections/Adjustments

Not applicable.

9.3.2 Calculated Variables

Not applicable.

9.4 Graphs and Plots

None.

10. Errors

10.1 Sources of Error

Field sampling error could account for some error in the concentration of the syringe samples:

1.  Not flushing the sampling line from the chamber before sampling could cause dilution of the sample with air from the last sampling time.

2.  Not completely closing the syringes or allowing them to come open during transport will cause dilution from ambient air.

3.  Placing the chamber down with much force can change the pressure inside the chamber to other than ambient and can affect the mechanisms and processes producing/taking up CH4 and CO2.

(Errors such as these would have been written down in the lab/field books, and those data therefore would have been edited out.)

The analytical precision of the GC's is: 0.2% for CH4 and 1% for CO2.

10.2 Quality Assessment

10.2.1 Data Validation by Source

None given.

10.2.2 Confidence Level/Accuracy Judgment

None given.

10.2.3 Measurement Error for Parameters

The analytical precision of the GC's is: 0.2% for CH4 and 1% for CO2.

10.2.4 Additional Quality Assessments

Not applicable.

10.2.5 Data Verification by Data Center

Data were examined for general consistency and clarity.

11. Notes

11.1 Limitations of the Data

None given.

11.2 Known Problems with the Data

See notes under Section 6.

11.3 Usage Guidance

None given.

11.4 Other Relevant Information

None given.

12. Application of the Data Set

The chamber flux data can be used in connection with the tower flux data to determine the CH4 and CO2 exchange between the atmosphere and the boreal soils.  The soil profile data can be used in comparison with the flux data to determine controls on the fluxes for a certain biome.

13. Future Modifications and Plans

None.

14. Software

Not applicable.

14.1 Software Description

HP ChemStation.

14.2 Software Access

Purchase from Hewlett Packard/

15. Data Access

15.1 Contact Information

Ms. Beth Nelson

BOREAS Data Manager

NASA GSFC

Greenbelt, MD

(301) 286-4005

(301) 286-0239 (fax)

beth@ltpmail.gsfc.nasa.gov

15.2 Data Center Identification

See Section 15.1.

15.3 Procedures for Obtaining Data

Users may place requests by telephone, electronic mail, or fax.

15.4 Data Center Status/Plans

The TGB-01 CO2 and CH4 chanmber flux data are available from the Earth Observing System Data and Information System (EOSDIS), Oak Ridge National Laboratory (ORNL) Distributed Active Archive Center (DAAC). The BOREAS contact at ORNL is:

ORNL DAAC User Services

Oak Ridge National Laboratory

(865) 241-3952

ornldaac@ornl.gov

ornl@eos.nasa.gov

16. Output Products and Availability

16.1 Tape Products

None.

16.2 Film Products

None.

16.3 Other Products

Tabular ASCII file.

17. References

17.1 Platform/Sensor/Instrument/Data Processing Documentation

Instruction Manual:  GC-14A. Shimadzu Corporation, Kyoto, Japan.

17.2 Journal Articles and Study Reports

Sellers, P. and  F. Hall. 1994. Boreal Ecosystem-Atmosphere Study: Experiment Plan. Version 1994-3.0, NASA BOREAS Report

(EXPLAN 94).

Sellers, P. and F. Hall. 1996. Boreal Ecosystem-Atmosphere Study: Experiment Plan. Version 1996-2.0, NASA BOREAS Report

(EXPLAN 96).

Sellers, P., F. Hall. 1997. BOREAS Overview Paper. JGR Special Issue (1997).

Sellers, P., F. Hall, and K.F. Huemmrich. 1996. Boreal Ecosystem-Atmosphere Study: 1994 Operations. NASA BOREAS Report (OPS

DOC 94).

Sellers, P., F. Hall, and K.F. Huemmrich. 1997. Boreal Ecosystem-Atmosphere Study: 1996 Operations. NASA BOREAS Report (OPS

DOC 96).

Sellers, P., F. Hall, H. Margolis, B. Kelly, D. Baldocchi, G. den Hartog, J. Cihlar, M.G. Ryan, B. Goodison, P. Crill, K.J. Ranson, D. Lettenmaier, and D.E. Wickland. 1995. The boreal ecosystem-atmosphere study (BOREAS): an overview and early results from the 1994 field year. Bulletin of the American Meteorological Society. 76(9):1549-1577.

17.3 Archive/DBMS Usage Documentation

None.

18. Glossary of Terms

None given.

19. List of Acronyms

AES     - Atmospheric Environment Services

ASCII   - American Standard Codes for Information

BOREAS  - BOReal Ecosystem-Atmosphere Study

BORIS   - BOREAS Information System

BP      - Beaver Pond site, NSA

CGR     - Certofoed bu Group

CMDL    - Climate Monitoring and Diagnostics Laboratory

CPI     - Certified by PI

CPI-??? - Certified but questionable

DAAC    - Distributed Active Archive Center

ECD     - Electron Capture Detector

EOS     - Earth Observing System

EOSDIS  - EOS Data and Information System

FID     - Flame Ionization Detector

GC      - Gas Chromatograph

GMT     - Greewich Mean Time

GSFC    - Goddard Space Flight Center

HP      - Hewlett Packard

NASA    - National Aeronautics and Space Administration

NOAA    - National Oceanic and Atmospheric Administration

NSA     - Northern Study Area

OBS     - Old Black Spruce site

OJP     - Old Jack Pine

ORNL    - Oak Ridge National Laboratory

PANP    - Prince Albert National Park

c

SSA     - Southern Study Area

TCD     - Thermal Conductivity Detector

TGB     - Trace Gas Biogeochemistry

URL     - Uniform Resource Locator

YJP     - Young Jack Pine site, NSA

20. Document Information

20.1 Document Revision Dates

Written:

Last Updated:  20-Jul-98

20.2 Document Review Dates

BORIS Review:  21-Oct-97

Science Review:

20.3 Document ID

20.4 Citation

20.5 Document Curator

20.6 Document URL

Keywords

Carbon Dioxide

Methane