BOREAS TGB-08 Monoterpene Concentration Data over the SSA-OBS and the SSA-OJP

Summary:

The TGB-08 team collected data to investigate the controls over non-methane hydrocarbon (NMHC) fluxes from boreal forest tree species.  This data set contains measurements of monoterpene concentrations in collected foliar gas emissions and foliar samples.  The data were collected at the Old Jack Pine (OJP) and Old Black Spruce(OBS) tower-flux sites in the SSA and were the locus for the monoterpene emission measurements.  These areas contained mature stands of jack pine and black spruce and were the focal sites in the BOREAS program for studies of biosphere/atmosphere exchange from these two habitat types.  The OBS site is situated in a black spruce/sphagnum bog with the largest trees 155 years old and 10-15 m. tall.  The OJP site is in a jack pine forest, 80 to 120 years old, which lies on a sandy bench of glacial outwash with the largest tree standing 15 m. tall.  Temporally, the data cover the period of 24-May-94 to 19-Sep-94.  The data are stored in tabular ASCII files.

Table of Contents

*  1 Data Set Overview

*  2 Investigator(s)

*  3 Theory of Measurements

*  4 Equipment

*  5 Data Acquisition Methods

*  6 Observations

*  7 Data Description

*  8 Data Organization

*  9 Data Manipulations

*  10 Errors

*  11 Notes

*  12 Application of the Data Set

*  13 Future Modifications and Plans

*  14 Software

*  15 Data Access

*  16 Output Products and Availability

*  17 References

*  18 Glossary of Terms

*  19 List of Acronyms

*  20 Document Information

1. Data Set Overview

1.1 Data Set Identification

BOREAS TGB-08 Monoterpene Concentration Data over the SSA-OBS and the SSA-OJP

1.2 Data Set Introduction

1.3 Objective/Purpose

The objective of this data set was to measure the Monoterpene Concentration in 1st fully expanded (2nd year needles) from the SSA-OBS and SSA-OJP sites.

1.4 Summary of Parameters

Monterpene concentration and Monoterpene emission.

1.5 Discussion

The research was ordered around three general questions: (1) To what extent are leaf carbon balance and isoprene synthase activity (the enzyme responsible for isoprene emission predictors of NMHC flux, (2) How do leaf carbon balance and isoprene synthase activity depend on nitrogen/water availability and carbon source/sink parameters, and (3) How do we modify the FORES-BGC ecosystem model based on question 1 and 2, to predict canopy-level NMHC fluxes.  Studies included seasonal monitoring of NMHC emissions and its relationship to plant phenology, photosynthesis, respiration, isoprene synthase activity, and leaf starch concentrations.

1.6 Related Data Sets

BOREAS TGB-08 Photosynthetic Rate Data over the SSA-OBS and the SSA-OJP

BOREAS TGB-08 Starch Concentration Data over the SSA-OBS and the SSA-OJP

BOREAS TGB-09 Tower Non Methane Hydrocarbons Mixing Ratio

BOREAS TGB-10 Tower Non Methane Hydrocarbons Mixing Ratio

BOREAS TGB-10 Oxidant Concentration

BOREAS TGB-10 Oxidant Flux

2. Investigator(s)

2.1 Investigator(s) Name and Title

Manuel Lerdau

2.2 Title of Investigation

The Relationship Between Non-Methane Hydrocarbon Emission and Leaf Carbon Balance in the Boreal Forest:  An Approach for Mechanistic Ecosystem Modeling.

2.3 Contact Information

Contact 1

---------

Dr. Manuel Lerdau

Ecology and Evolution

SUNY

Stony Brook, NY

(516)632-6633

(516)632-7626 (fax)

mlerdau@life.bio.sunysb.edu

Contact 2

---------

Sara K. Conrad

Raytheon STX Corporation

NASA/GSFC

Greenbelt, MD

(301)286-2624

Sara.Golightly@gsfc.nasa.gov

3. Theory of Measurements

Sample selection

For the photosynthesis/hydrocarbon measurements, ten trees of each species were chosen that had sunlit leaves accessible within 3 m of the ground.  All measurements were conducted on sunlit leaves that had developed the previous year.  Tissue chemistry and gas exchange sampling on sunlit leaves from branches much higher in the canopy showed that there was no significant effect of branch height on photosynthetic rate or on tissue composition (ANOVA, p>0.05, data not shown).  The black spruce trees used in the bog transect/tissue chemistry sampling were chosen on the basis of having sunlit leaves accessible within 2m of the ground.  All hydrocarbon measurements were made on fully expanded needles that had developed during the previous growing season.  Needles were placed in the cuvette so that only those needles that expanded during the previous growing season were included.  Needle age in all cases was determined unambiguously by marking needle cohorts before leaf expansion in the spring and by examining the branches for twig color change and bud scarring associated with each year's growth.  The bog transect measurements were also all made on last year's fully expanded needles.

Sample procedure

Hydrocarbon emissions:  Samples were collected by enclosing branches in a temperature- and light-controlled cuvette connected to plant gas exchange system (Campbell MPH 1000, Campbell Scientific, Logan, UT) and flowing hydrocarbon-free  air over the needles.  Temperature was controlled by use of thermoelectric coolers provided by Campbell Scientific, and light intensity was controlled by mounting a projector bulb at a right angle to the top of the glass-topped cuvette.  The light was then reflected off of a cold mirror (458 cold mirror, 15-33233, OCLI, Santa Rosa, CA) mounted at a 458 angle to cuvette.  The mirror transmitted light at wavelengths >720nm and reflected light of shorter wavelengths. Hydrocarbon-free air was produced by pumping ambient air through a clean-air generator (Aadco 5L, AADCO Instruments, Silver Springs, FL) and adding CO2 back to the entering air stream.  All flows and environmental conditions were monitored by the sensors and mass flow controllers of the Campbell MPH 1000.Hydrocarbon emission samples were collected by diverting a fraction of the air exiting the leaf cuvette through a sampling tube packed with a solid sorbent.  Supelco Carbotrap 300 cartridges (Bellafonte, PA) with dimensions of 7  in length and 1/4  in outer diameter were used in this work.  The sampling tubes were conditioned before each use via a Tekmar Thermo Trap unit (Cincinnati, OH) at 220 8C for a minimum of four hours at a flow of approximately 10 mL/min of ultrahigh purity nitrogen.  A blank from each set of conditioned cartridges was analyzed to ensure that they had been properly cleaned.  The flow rate and volume of air passing over the sampling tube was controlled with a low flow pump (SKC Model-222, SKC, Inc. Eighty-Four, PA).  This sample volume was variable but kept well below the typical breakthrough volumes for terpenes on this sampling tube (Anon., 1986).

Foliar chemistry:  After the emissions samples were collected, we separated the  branchlet and needles from the main branch.  Total needle biomass was measured on fresh needles.  We then separated the total needle biomass into two parts, half of which we placed in a 60 8C drying oven and weighed daily  until no further change in weight was observed.  These dried needles were hen stored for nitrogen analysis.  The remaining needles were ground in liquid nitrogen and stored in 20 mL scintillation vials filled with pentane until they were analyzed for monoterpene concentrations.  The pentane storage was never less than seven days and was more than sufficient to allow for complete solvent extraction of the monoterpenes (Lerdau et al. 1995).  The fresh weight / dry weight ratio from the needles used in the nitrogen analyses was applied to each monoterpene sample to provide an estimated dry weight for the sample.  All analyses are reported on a dry-weight basis.

Analysis

Hydrocarbon emissions:  Samples were analyzed by thermal desorption followed by gas chromatography/mass spectrometry (GC/MS) using a modified form of the EPA TO-1 method (Anon., 1984).  A Tekmar AeroTRAP unit (Cincinnati, OH), Tekmar Cryofocusing unit (Cincinnati, OH), and ultrahigh purity helium purge gas were used to thermally desorb hydrocarbons off the cartridges via a multi-step process.  Cartridges were heated to 220 8C to desorb the sample onto an internal glass-bead-packed trap cooled with liquid nitrogen to -165 8C.  Next, the sample was desorbed off the internal trap at 220 8C, passed through a moisture control system to remove water, and re-focused onto the head of the column which was held at -165 8C using liquid nitrogen.  Finally, the sample was injected onto the column by flash heating the head of the column to 220 8C.A Finnigan model ITS40 gas chromatograph/ion trap mass spectrometer/data system (San Jose, CA) was used to analyze the hydrocarbons in the samples.  A 25 m, 0.25 mm ID DB-5ms column from J&W Scientific (Folsom, CA) was used to separate the terpenes.  The temperature of the column was programmed to separate the terpenes of interest.  This entailed holding the column at 408C for 5 minutes, increasing the temperature at 88 C/min to 2208 C, and then holding the temperature at 2208 C for 4 minutes.  The transfer line to the ion trap mass spectrometer was held at 2508 C, and the ion trap manifold was set to 1008 C.  The ion trap mass spectrometer was run through a daily auto-tune sequence prior to data acquisition on samples, standards, and blanks.  This auto-tune sequence involved calibrating the mass scale and setting emission current, multiplier voltage, automatic gain control target value to proper values.  Mass spectra were acquired under electron ionization (EI) conditions using a scan range of m/z 50 to 200 and a scan rate of 1 scan/second.  Terpenes were identified using both retention time confirmation and matching experimental mass spectra against a custom library of mass spectra derived from a series of terpene standards.  Terpenes were quantified against external standards, in which the most intense ion for the individual terpene  (quantitation ion) was compared to the same ion from analyses of known amounts of neat terpene standards in hexane.  Method blanks were used to correct for any response for terpenes on the conditioned cartridges.  All emission rates are reported on a dry weight basis.

Foliar chemistry:   Monoterpenes were separated on a HP 5890 Series II gas chromatograph equipped with a split/splitless injector, flame ionization detector (FID) and a 30 m DB-1 capillary column (0.32 mm i.d., 1 µm film thickness, J & W Scientific, Folsom CA), 2 L of sample was injected in the split mode (80:1 split) using a HP 7673 Auto Sampler, He as the carrier gas at a flow rate of 2.2 mL min-1.  Column temperature was programmed to stay at 508 C for five minutes, then increase 68 C/min to 2508 C and held for 5 minutes.  Detector temperature was 3008 C and injector temperature was2758 C.  Individual monoterpenes were identified by comparison of their retention times with authentic monoterpene standards (purchased from Sigma/Aldrich Chemical CO, St. Louis, MO) that were analyzed under conditions identical to those used for the unknown samples.  Monoterpenes were quantified using fenchone as an internal standard.  Fenchone was added to the extract 24 hours prior to analysis.  The results are expressed as mg terpene per gram of needle dry weight.  Needle nitrogen concentration was measured as total Kjeldahl nitrogen (TKN; calculated as a percentage of needle dry mass) using the digestion and measurement protocol described by Jaeger and Monson (1992).  Needles were ground and digested in sulfuric acid with a copper sulfate catalyst at3608 C for three hours and then analyzed colorometrically using a flow injection analyzer (LACHAT Inst., Mequon, WI).

4. Equipment:

4.1 Sensor/Instrument Description

4.1.1 Collection Environment

Samples were collected under ambient environmental conditions.

4.1.2 Source/Platform

Trees.

4.1.3 Source/Platform Mission Objectives

The purpose of the trees and branches for this experiment were to support the measurement equipment.

4.1.4 Key Variables

Monoterpene concentration and Monoterpene Emission.

4.1.5 Principles of Operation

None given.

4.1.6 Sensor/Instrument Measurement Geometry

None given.

4.1.7 Manufacturer of Sensor/Instrument

Campbell Scientific

AADCO Instruments

Silver Springs, FL

Supelco

Bellafonte, PA

Tekmar

Cincinnati, OH

SKC, Inc.

Eighty Four, PA

Finnigan

San Jose, CA

J&W Scientific

Folsom, CA

Hewlett Packard

LACHAT Inst.

Mequon, WI

4.2 Calibration

4.2.1 Specifications

Terpenes were identified using both retention time confirmation and matching experimental mass spectra against a custom library of mass spectra derived from a series of terpene standards.  Terpenes were quantified against external standards, in which the most intense ion for the individual terpene  (quantitation ion) was compared to the same ion from analyses of known amounts of neat terpene standards in hexane.  Method blanks were used to correct for any response for terpenes on the conditioned cartridges. Monoterpenes were quantified using fenchone as an internal standard.

4.2.1.1 Tolerance

None Given.

4.2.2 Frequency of Calibration

The ion trap mass spectrometer was run through a daily auto-tune sequence prior to data acquisition on samples, standards, and blanks.  This auto-tune sequence involved calibrating the mass scale and setting emission current, multiplier voltage, automatic gain control target value to proper values.

4.2.3 Other Calibration Information

None Given.

5. Data Acquisition Methods

Samples were collected by enclosing branches in a temperature- and light-controlled cuvette connected to plant gas exchange system (Campbell MPH 1000, Campbell Scientific, Logan, UT) and flowing hydrocarbon-free air over the needles.  Temperature was controlled by use of thermoelectric coolers provided by Campbell Scientific, and light intensity was controlled by mounting a projector bulb at a right angle to the top of the glass-topped cuvette.  The light was then reflected off of a cold mirror (458 cold mirror, 15-33233, OCLI, Santa Rosa, CA) mounted at a 458 angle to cuvette.  The mirror transmitted light at wavelengths >720nm and reflected light of shorter wavelengths. Hydrocarbon-free air was produced by pumping ambient air through a clean-air generator (Aadco 5L, AADCO Instruments, Silver Springs, FL) and adding CO2 back to the entering air stream.  All flows and environmental conditions were monitored by the sensors and mass flow controllers of the Campbell MPH 1000.Hydrocarbon emission samples were collected by diverting a fraction of the air exiting the leaf cuvette through a sampling tube packed with a solid sorbent.  Supelco Carbotrap 300 cartridges (Bellafonte , PA) with  dimensions of 7  in length and 1/4  in outer diameter were used in this work.  The sampling tubes were conditioned before each use via a Tekmar Thermo Trap unit (Cincinnati, OH) at 220 8C for a minimum of four hours at a flow of approximately 10 mL/min of ultrahigh purity nitrogen.  A blank from each set of conditioned cartridges was analyzed to ensure that they had been properly cleaned.  The flow rate and volume of air passing over the sampling tube was controlled with a low flow pump (SKC Model-222, SKC, Inc. Eighty Four, PA).  This sample volume was variable but kept well below the typical breakthrough volumes for terpenes on this sampling tube (Anon., 1986).

After the emissions samples were collected, we separated the branchlet and needles from the main branch.  Total needle biomass was measured on fresh needles.  We then separated the total needle biomass into two parts, half of which we placed in a 60 8C drying oven and weighed daily until no further change in weight was observed.  These dried needles were hen stored for nitrogen analysis.  The remaining needles were ground in liquid nitrogen and stored in 20 mL scintillation vials filled with pentane until they were analyzed for monoterpene concentrations.  The pentane storage was never less than seven days and was more than sufficient to allow for complete solvent extraction of the monoterpenes (Lerdau et al. 1995).  The fresh weight / dry weight ratio from the needles used in the nitrogen analyses was applied to each monoterpene sample to provide an estimated dry weight for the sample.  All analyses are reported on a dry-weight basis.

6. Observations

6.1 Data Notes

None Given.

6.2 Field Notes

None Given.

7. Data Description

7.1 Spatial Characteristics

7.1.1 Spatial Coverage

The North American Datum 1983 (NAD83) coordinates for the sites are:

SSA-OBS  53.98717°N, 105.11779°W

SSA-OJP  53.91634°N, 104.69203°W

7.1.2 Spatial Coverage Map

None Given.

7.1.3 Spatial Resolution

These data are point source measurements taken near the given coordinates.

7.1.4 Projection

Not Applicable.

7.1.5 Grid Description

Not applicable.

7.2 Temporal Characteristics

7.2.1 Temporal Coverage

The data were collected from 24-May-94 to 19-Sep-94.

7.2.2 Temporal Coverage Map

None given.

7.2.3 Temporal Resolution

Monthly averages of the data were calculated for 24-May-94 to 19-Sep-94.

7.3 Data Characteristics

Data characteristics are defined in the companion data definition file (tgb8mono.def).

7.4 Sample Data Record

Sample data format shown in the companion data definition file (tgb8mono.def).

8. Data Organization

8.1 Data Granularity

All of the Monoterpene Concentration Data over the SSA-OBS and the SSA-OJP are contained in one dataset.

8.2 Data Format(s)

The data files contain numerical and character fields of varying length separated by commas. The character fields are enclosed with a single apostrophe marks. There are no spaces between the fields. Sample data records are shown in the companion data definition file (tgb8mono.def).

9. Data Manipulations

9.1 Formulae

9.1.1 Derivation Techniques and Algorithms

None given.

9.2 Data Processing Sequence

9.2.1 Processing Steps

None given.

9.2.2 Processing Changes

None given.

9.3 Calculations

9.3.1 Special Corrections/Adjustments

None given.

9.3.2 Calculated Variables

None.

9.4 Graphs and Plots

None.

10. Errors

10.1 Sources of Error

None given.

10.2 Quality Assessment

10.2.1 Data Validation by Source

None given.

10.2.2 Confidence Level/Accuracy Judgement

None given.

10.2.3 Measurement Error for Parameters

None given.

10.2.4 Additional Quality Assessments

None given.

10.2.5 Data Verification by Data Center

The data were examined for general consistency and clarity.

11. Notes

11.1 Limitations of the Data

None given.

11.2 Known Problems with the Data

None given.

11.3 Usage Guidance

None.

11.4 Other Relevant Information

None.

12. Application of the Data Set

None given.

13. Future Modifications and Plans

None given.

14. Software

14.1 Software Description

None given.

14.2 Software Access

None given.

15. Data Access

15.1 Contact Information

MS. Beth Nelson

BOREAS Data Manager

NASA GSFC

Greenbelt, MD

(301) 286-4005

(301) 286-0239 (fax)

Elizabeth.Nelson@gsfc.nasa.gov

15.2 Data Center Identification

See 15.1.

15.3 Procedures for Obtaining Data

Users may place requests by telephone, electronic mail, or FAX.

15.4 Data Center Status/Plans

The TGB-08 monoterpene data are available from the EOSDIS ORNL DAAC (Earth Observing System Data and Information System) (Oak Ridge National Laboratory) (Distributed Active Archive Center). The BOREAS contact at ORNL is:

ORNL DAAC User Services

Oak Ridge National Laboratory

(865) 241-3952

ornldaac@ornl.gov

ornl@eos.nasa.gov

16. Output Products and Availability

16.1 Tape Products

None.

16.2 Film Products

None.

16.3 Other Products

Comma delimited ACSII Data Files

17. References

17.1 Platform/Sensor/Instrument/Data Processing Documentation

None.

17.2 Journal Articles and Study Reports

Sellers, P., F. Hall. 1994. Boreal Ecosystem-Atmosphere Study: Experiment Plan. Version 1994-3.0, NASA BOREAS Report (EXPLAN 94).

Sellers, P., F. Hall, H. Margolis, B. Kelly, D. Baldocchi, G. den Hartog, J. Cihlar, M.G. Ryan, B. Goodison, P. Crill, K.J. Ranson, D. Lettenmaier, and D.E. Wickland. 1995. The boreal ecosystem-atmosphere study (BOREAS): an overview and early results from the 1994 field year. Bulletin of the American Meteorological Society. 76(9):1549-1577.

Sellers, P., F. Hall, K.F. Huemmrich. 1996. Boreal Ecosystem-Atmosphere Study: 1994 Operations. NASA BOREAS Report (OPS DOC 94).

Sellers, P., F. Hall. 1996. Boreal Ecosystem-Atmosphere Study: Experiment Plan. Version 1996-2.0, NASA BOREAS Report (EXPLAN 96).

Sellers, P., F. Hall, K.F. Huemmrich. 1997. Boreal Ecosystem-Atmosphere Study: 1996 Operations. NASA BOREAS Report (OPS DOC 96).

Sellers, P. J., F. G.  Hall, R. D.  Kelly, A.  Black, D.  Baldocchi, J.  Berry, M.  Ryan, K. J.  Ranson, P. M.  Crill, D. P.  Lettenmaier, H.  Margolis, J.  Cihlar, J. Newcomer, D.  Fitzjarrald, P. G.  Jarvis, S. T.  Gower, D.  Halliwell, D.  Williams, B.  Goodison, D. E.   Wickland, and F. E. Guertin. 1997. BOREAS in 1997: Experiment Overview, Scientific Results and Future Directions. Journal of Geophysical Research 102 (D24): 28,731-28,770.

17.3 Archive/DBMS Usage Documentation

None.

18. Glossary of Terms

None given.

19. List of Acronyms

BOREAS  - BOReal Ecosystem-Atmosphere Study

BORIS   - BOREAS Information System

DAAC    - Distributed Active Archive Center

EOS     - Earth Observing System

EOSDIS  - EOS Data and Information System

GSFC    - Goddard Space Flight Center

NASA    - National Aeronautics and Space Administration

ORNL    - Oak Ridge National Laboratory

URL     - Uniform Resource Locator

20. Document Information

20.1 Document Revision Date

Written:  28-Jul-1997

Last updated:  14-Sep-1998

20.2 Document Review Date(s)

BORIS Review:   28-Aug-1998

Science Review:

20.3 Document ID

20.4 Citation

Manuel Lerdau: Department of Ecology and Evolution, State University of

New York, Stony Brook, NY  11794-5245

Marcy Litvak and Russell Monson: Department of Environmental, population and Organismic Biology, University of Colorado, Boulder, CO 80309-033

20.5 Document Curator

20.6 Document URL

Keywords:

Monoterpene, Terpene, NMHC, Photosynthesis, Starch.