1.  TITLE

1.1  Data Set Identification.

Daily Stream Flow Amounts.

1.2  FIS Data Base Table Name.

STREAM_FLOW_DAILY_DATA.

1.3  CD-ROM File Name.

\DATA\HYDROLGY\STRM_DAY\GRIDxxxx\yyyygrid.SDY

Where xxxx is the four digit code for the location within the FIFE site

grid. Note: capital letters indicate fixed values that appear on the

CD-ROM exactly as shown here, lower case indicates characters (values)

that change for each path and file.

The format used for the filenames is: yyyygrid.sfx, where yyyy are the

four digits of the century and year (e.g., 1987 or 1989). The filename

extension (.sfx), identifies the data set content for the file (see

Section 8.2) and is equal to .SDY for this data set.

1.4  Revision Date Of This Document.

April 26, 1994.

This document has been reviewed to eliminate technical and editorial

inaccuracies by the FIFE Information Scientist. Previous versions of this

document have been reviewed by the Principal Investigator, the person who

transmitted the data to FIS, a FIS staff member, or a FIFE scientist

generally familiar with the data. It is believed that the document

accurately describes the data as collected and as archived on the FIFE

CD-ROM series.

2.  INVESTIGATOR(S)

2.1  Investigator(s) Name And Title.

Staff Science.

2.2  Title Of Investigation.

Staff Science Soil Moisture and Hydrology Data Acquisition Program.

2.3  Contacts (For Data Production Information).

_________________________________________

|              |      Contact 1          |

|______________|_________________________|

|2.3.1 Name    |Dr. John Briggs          |

|2.3.2 Address |Ackert Hall              |

|              |Division of Biology      |

|              |Kansas State University  |

|      City/St.|Manhattan, KS            |

|      Zip Code|66506                    |

|2.3.3 Tel.    |(913) 532-6629           |

|2.3.4 Email   |jmb@andro.konza.ksu.edu  |

|______________|_________________________|

2.4  Requested Form of Acknowledgment.

The daily stream flow amounts were provided by the staff of the Long-Term

Ecological Research (LTER) site at Kansas State University (KSU) and the

U.S. Geological Survey, Kansas. The data collected by the LTER staff were

collected under the Konza Prairie LTER grant BSR 8514327 awarded to KSU

by the National Science Foundation.

3.  INTRODUCTION

3.1  Objective/Purpose.

The purpose of this data set was to provide accurate measurements of

the stream-flow from tributaries within the FIFE study area, thereby

enabling the study of the hydrology of streams that drain from a tallgrass

prairie.

3.2  Summary of Parameters.

Daily minimum, maximum and mean stream flow, and total volume of water

flow in the stream.

3.3  Discussion.

Five stream-flow gauges were placed across creeks in the Long-Term

Ecological Research (LTER) section of the FIFE study area. Four of these

five stations were maintained and monitored by the LTER staff while the

fifth was part of the USGS network of stream flow gauges.

Daily measurements of stream flow are available for the 4 LTER stations

and for the USGS stream-flow station located on tributaries to Kings

Creek. This data sets contains measurements from April 1979 to Sept. 1988

for the USGS station, and from June 1985 to December 1987 for the 4 LTER

stations.

4.  THEORY OF MEASUREMENTS

The V-throated flume and standpipes used at the LTER weirs operated on the

principle that the height of the water level in the a standpipe at a specific

location within a weir of known dimensions can be converted to volume of water

in the stream. The change of this instantaneous volume with time could then be

used to compute volumetric stream flow. See Replogle 1978 for a more in-depth

discussion of the theory associated with this type of weir.

The stilling pipe installation at the USGS stations operates on the principle

that the height of the water level in the a standpipe at a specific location

within a streambed can be converted to volume of water in the stream. The

tracking of the change in stream height with time then enables the calculation

of stream flow.

5.  EQUIPMENT

5.1  Instrument Description.

At the LTER sites the weir used created a V-throated flume. There was a

standpipe associated with the weir through which the gauge height was

measured.

5.1.1  Platform.

The LTER wier and stilling pipe intake is built into the streambed.

The stilling pipe intake at the USGS station is built into the

bottom of the streambed.

5.1.2  Mission Objectives.

To measure the volume of stream flow.

5.1.3  Key Parameters.

Average, minimum and maximum daily stream-flow and daily total

volume of stream-flow.

5.1.4  Principles of Operation.

The LTER wiers were constructed in the streambed with a specific

configuration so that the height of the water as it flows through

the weir could be measured using a pressure transducer. The gauge

height is measured as the pressure of the water in a standpipe

associated with the v-throated wier. This height is then converted

to a volumetric flow knowing the geometry of the wier.

For a description of the operating principles for the USGS station

see this section in the document entitled Fifteen-Minute Stream

Flow Rates from USGS on FIFE CD-ROM Volume 1.

5.1.5  Instrument Measurement Geometry .

The LTER weirs were V-throated flumes.

A weir was not installed into Kings Creek at the USGS station.

5.1.6  Manufacturer of Instrument.

The LTER wiers were built by the staff of the Konza LTER site using

a design described in Replogle et al., 1978.

Pressure transducers:  Druck, Model No. PDCR 10/D

Address:  P.O. Box 551

Logan, Utah 84321

(801) 753-2342

Data logger: Campbell Scientific Co., Model No. CR-21X

P. O. Box 551,

Logan, UT 84321.

Data recorder: Campbell Scientific Co. Model PC201

For a description of the instruments used at the USGS station

see this section in the document entitled Fifteen-Minute Stream

Flow Rates form USGS on FIFE CD-ROM Volume 1.

5.2  Calibration.

During normal events the LTER weirs are calibrated by making direct

measurements of actual gauge height which are measured at predetermined

reference points within each weir. These actual heights are used to

correct the pressure measured gauge heights to the actual values on the

data logger. This change is noted in the logbook which is kept by the

LTER site at KSU.

See this section in the document entitled Fifteen-Minute Stream Flow

Rates from USGS on FIFE CD-ROM Volume 1 for a description of the

calibration methodology for the USGS station.

5.2.1  Specifications.

The LTER weirs were calibrated at different discharge volumes.

These calibrations indicate that the conversion from gauge height

to volumetric flow employs different geometric relationships at

gauge heights less than 18.25 cm and at heights above 18.25 cm.

This information is not available at this revision for the USGS

station.

5.2.1.1  Tolerance.

The tolerance of the LTER sites is not available at this

revision.

Accuracy of the stilling well is about (+/-) 0.3048

E-3 meters.

5.2.2  Frequency of Calibration.

Gauge height for the LTER weirs is calibrated at least weekly by

making actual measurements of the gauge height.

Discharge measurements for the USGS station are checked monthly.

Stage height for the USGS station is checked when the station is

visited by an engineer or at a minimum every 2-3 yrs.

5.2.3  Other Calibration Information.

None available at this revision.

6.  PROCEDURE

6.1  Data Acquisition Methods.

The stream gauge height on the LTER weirs is automatically measured every

1 minute with the Druck pressure transducers. These one minute values are

recorded and averaged every five minutes into 5 minute averages. The data

is then dumped from the data logger to a cassette tape. These data are

then dumped from tape to an IBM PC where they are then converted to

volumetric stream flow.

The USGS data were obtained from the USGS by the staff of LTER site at

Kansas State University.

6.2  Spatial Characteristics.

The FIFE site with areal extent of 15 km by 15 km, is located south of

the Tuttle Reservoir and Kansas River, and about 10 km from Manhattan,

Kansas, USA. The northwest corner of the area has UTM coordinates of

4,334,000 Northing and 705,000 Easting in UTM zone 14.

6.2.1  Spatial Coverage.

Data were collected from four LTER stations on watersheds N4D, NUB,

N1B, and N2B in the Konza Prairie Natural Research Area in the

northwest quadrant of the FIFE study area, and from one USGS

station 2.9 miles upstream from the mouth of Kings Creek.

Sitegrid ID  Station ID    Watershed   Northing  Easting

-----------  ----------  ------------  --------  -------

1715-STG     650         KINGS CREEK   4330650   707973

2422-STV     651             N1B       4329110   709380

2421-STV     652             N2B       4329140   709260

2323-STV     653             N4D       4329380   709560

2423-STV     654             NUB       4329230   709620

Latitude  Longitude

--------  ---------

39 06 07  -96 35 42

39 05 16  -96 34 45

39 05 17  -96 34 50

39 05 25  -96 34 37

39 05 20  -96 34 35

6.2.2  Spatial Resolution.

These data were collected at point locations, however, they

represent the integrated value of the runoff into the creek and

stream above the location of the wier or stilling gauge.

6.3  Temporal Characteristics.

6.3.1  Temporal Coverage.

Generally data are available from April, 1979 through December,

1987.

Watershed   Sitegrid ID  Station ID   Start       Stop

------------  -----------  ---------- ---------   ---------

KINGS CREEK   1715-STG        650     01-APR-79   02-SEP-88

N1B       2422-STV        651     14-JUN-85   31-DEC-87

N2B       2421-STV        652     14-JUN-85   31-DEC-87

N4D       2323-STV        653     14-JUN-85   31-DEC-87

NUB       2423-STV        654     10-JUN-86   31-DEC-87

6.3.2  Temporal Resolution.

Data are available every day.

7.  OBSERVATIONS

7.1  Field Notes.

None available at this revision.

8.  DATA DESCRIPTION

8.1  Table Definition With Comments.

The SQL definition for this table is found in the STRM_DAY.TDF file

found on FIFE CD-ROM Volume 1.

8.2  Type of Data.

--------------------------------------------------------------------------------

|                 8.2.1                  |               |          |          |

|Parameter/Variable Name                 |               |          |          |

--------------------------------------------------------------------------------

|    |               8.2.2               |     8.2.3     |  8.2.4   |  8.2.5   |

|    |Parameter/Variable Description     |Range          |Units     |Source    |

--------------------------------------------------------------------------------

|SITEGRID_ID                             |               |          |          |

|    |This is a FIS grid location code.  |               |          |FIS       |

|    |Site grid codes (SSEE-III) give    |               |          |          |

|    |the south (SS) and east (EE) cell  |               |          |          |

|    |number in a 100x100 array of 200m  |               |          |          |

|    |square cells.  The last 3          |               |          |          |

|    |characters (III) are an instrument |               |          |          |

|    |identifier.                        |               |          |          |

--------------------------------------------------------------------------------

|STATION_ID                              |               |          |          |

|    |The station ID designating the     |min = 650,     |          |KANSAS STA|

|    |location of the observations.      |max = 654      |          |TE UNIVERS|

|    |                                   |               |          |ITY       |

--------------------------------------------------------------------------------

|OBS_DATE                                |               |          |          |

|    |The date of the observations.      |min = 01-APR-79|          |KANSAS STA|

|    |                                   |,              |          |TE UNIVERS|

|    |                                   |max = 02-SEP-88|          |ITY       |

--------------------------------------------------------------------------------

|AVG_FLOW                                |               |          |          |

|    |The average water flow for the day.|min = 0,       |[meters^3]|WIER      |

|    |                                   |max = 6.11644, |[sec^-1]  |          |

|    |                                   |missing = -9.99|          |          |

|    |                                   |9              |          |          |

--------------------------------------------------------------------------------

|MAX_FLOW                                |               |          |          |

|    |The maximum water flow for the day.|min = 0,       |[meters^3]|WIER      |

|    |                                   |max = 2.6,     |[sec^-1]  |          |

|    |                                   |missing = -9.99|          |          |

|    |                                   |9              |          |          |

--------------------------------------------------------------------------------

|MAX_FLOW_TIME                           |               |          |          |

|    |The time of the maximum water      |min = 0,       |[GMT]     |WIER      |

|    |flow, modified from CDT.           |max = 2855,    |          |          |

|    |                                   |missing = 9999 |          |          |

--------------------------------------------------------------------------------

|MIN_FLOW                                |               |          |          |

|    |The minimum water flow for the day.|min = 0,       |[meters^3]|WIER      |

|    |                                   |max = .1,      |[sec^-1]  |          |

|    |                                   |missing = -9.99|          |          |

|    |                                   |9              |          |          |

--------------------------------------------------------------------------------

|MIN_FLOW_TIME                           |               |          |          |

|    |The time of the minimum water      |min = 0,       |[GMT]     |KANSAS STA|

|    |flow, modified from CDT.           |max = 2855,    |          |TE UNIVERS|

|    |                                   |missing = 9999 |          |ITY       |

--------------------------------------------------------------------------------

|TOTAL_VOL                               |               |          |          |

|    |The total volume of water which    |min = -10,     |[meters^3]|WIER      |

|    |flowed through the observation     |max = 23020    |          |          |

|    |point during this day.             |               |          |          |

--------------------------------------------------------------------------------

|FIFE_DATA_CRTFCN_CODE                   |       *       |          |          |

|    |The FIFE Certification Code for    |CPI=checked by |          |FIS       |

|    |the data, in the following format: |principal inves|          |          |

|    |CPI (Certified by PI), CPI-???     |tigator,       |          |          |

|    |(CPI - questionable data).         |CPI-MRG=merged |          |          |

|    |                                   |data           |          |          |

--------------------------------------------------------------------------------

|LAST_REVISION_DATE                      |               |          |          |

|    |The last revision date for the     |min = 22-JUN-88|          |FIS       |

|    |data, in the format (DD-MMM-YY).   |,              |          |          |

|    |                                   |max = 17-OCT-89|          |          |

--------------------------------------------------------------------------------

Footnote:

* Valid levels

The primary certification codes are:

EXM - Example or Test data (not for release)

PRE - Preliminary (unchecked, use at your own risk)

CPI - Checked by Principal Investigator (reviewed for quality)

CGR - Checked by a group and reconciled (data comparisons and

cross checks)

The certification code modifiers are:

PRE-NFP  - Preliminary - Not for publication, at the request of

investigator.

CPI-MRG  - PAMS data which is "merged" from two separate receiving

stations to eliminate transmission errors.

CPI-???  - Investigator thinks data item may be questionable.

8.3  Sample Data Base Data Record.

SITEGRID_ID STATION_ID OBS_DATE    AVG_FLOW  MAX_FLOW MAX_FLOW_TIME

----------- ---------- ---------- --------- --------- -------------

1715-STG           650 01-APR-79    .339800

1715-STG           650 02-APR-79    .368120

1715-STG           650 03-APR-79    .339800

1715-STG           650 02-MAY-79    .127430

MIN_FLOW MIN_FLOW_TIME     TOT_VOL FIFE_DATA_CRTFCN_CODE

--------- ------------- ----------- ---------------------

CPI

CPI

CPI

CPI

LAST_REVISION_DATE

------------------

22-JUN-88

22-JUN-88

22-JUN-88

22-JUN-88

8.4  Data Format.

The CD-ROM file format consists of numerical and character fields of

varying length separated by commas. The character fields are enclosed with

a single apostrophe. There are no spaces between the fields.  Each file

begins with five header records. Header records contain the following

information:

Record 1    Name of this file, its table name, number of records in

this file, path and name of the document that describes

the data in this file, and name of principal investigator

for these data.

Record 2    Path and filename of the previous data set, and path and

filename of the next data set. (Path and filenames for files

that contain another set of data taken at the same site on

the same day.)

Record 3    Path and filename of the previous site, and path and filename

of the next site. (Path and filenames for files of the same

data set taken on the same day for the previous and next

sites (sequentially numbered by SITEGRID_ID)).

Record 4    Path and filename of the previous date, and path and filename

of the next date. (Path and filenames for files of the same

data set taken at the same site for the previous and next

date.)

Record 5    Column names for the data within the file, delimited by

commas.

Record 6    Data records begin.

Each field represents one of the attributes listed in the chart in

Section 8.2 and described in detail in the TDF file (see Section 8.1).

These fields are in the same order as in the chart.

8.5  Related Data Sets.

Storm Event Stream Flow.

Fifteen-Minute Stream Flow Rates from USGS.

9.  DATA MANIPULATIONS

9.1  Formulas.

Gauge height measurements made by the LTER staff at SITEGRID IDs 2422-STV,

2421-STV, 2323-STV, 2423-STV are converted to stream discharge using the

following rating curves (Replogle et. al., 1978);

Use the following equation when gauge height is > 18.25 cm:

= 4.64(E^-5)(s^2.587)

Use the following equation when gauge height is between 0-18.25 cm:

= 4.49(E^-5)(s^2.4714)

Where Q = discharge in cubic meters per second

s = gauge height in cm

9.1.1  Derivation Techniques/Algorithms.

Not available at this revision for the LTER stations.

For a description of the formulas and derivation techniques used at

the USGS station see this section in the document entitled

Fifteen-Minute Stream Flow Rates from USGS on FIFE CD-ROM Volume 1.

9.2  Data Processing Sequence.

The data collected at the 4 LTER stations are processed using the

following steps:

1.  The raw data from each watershed is read from the cassette tape

for that watershed and placed on-line.

2.  These raw data are reformatted into a condensed more easily read

form.

3.  These data are then compared to the log sheets for the data

loggers. The data processor is looking for indications that the

data loggers have drifted and the amount of that drift. The data

loggers are routinely monitored for drift. An observer

periodically measures the ACTUAL stage height of the stream and

compares this value with that recorded by the data logger. When

a drift is detected the observer changes the recorded value on

the data logger to the measured value and notes the time and

change in the logbook.

4.  Recorded stage height values are then corrected for the detected

drift in one of 4 ways

a)  When all values are off by the same amount a value is added

or subtracted from all entries.

b)  When values are wrong on both ends of a time period or at one

end and not the other, a linear regression derived from the

actual stage height recorded on the log sheets is performed.

c)  When there is no flow observed yet there is a indication of

flow on the data logger all values are changed to 0.

d)  When there is no drift, the data is not altered.

5.  These data are then converted to discharge using the formulas in

Section 9.1 above.

See Section 9.2 in the Fifteen-Minute Stream Flow Rates from USGS document

on FIFE CD-ROM Volume 1 for a description of the data processing sequence

for data from the USGS station.

9.2.1  Processing Steps and Data Sets.

See Section 9.2 above.

9.2.2  Processing Changes.

None known at this revision.

9.3  Calculations.

See Section 9.1 above.

9.3.1  Special Corrections/Adjustments.

None known at this revision.

9.4  Graphs and Plots.

None.

10.  ERRORS

10.1  Sources of Error.

Errors in the data collected by the LTER site staff may arise from

several sources:

1)  The transducers which detect the stage height have a tendency to

drift. Errors can result with this drift is not detected.

2)  Power outages occur occasionally thereby preventing the recording

of data to the data loggers.

3)  Occasionally the data loggers themselves malfunction.

4)  Data can be lost or corrupted when it is transferred from the data

loggers to the cassette tapes.

Errors in the data from the USGS station originate from the instability

of the relationship between stage height and discharge. This

relationship varies temporally. It is effected by changes in the

streambed that result from the flow of water over the bed, such as

scour and fill, aquatic growth, ice, debris, or bed roughness.

The intakes to the stilling well at the USGS station can become plugged

with debris or overgrown with plant or animal material, and the float can

malfunction.

10.2  Quality Assessment.

10.2.1  Data Validation by Source.

The LTER site staff make spot checks of the stage height during

storm events. These spot checks "calibrations" are used to

create a calibration for the stage height during the storm event.

The discharge curves for the USGS station are routinely checked

for accuracy. The stage height within the stilling well is

checked against that measured using a manual gauge.

10.2.2  Confidence Level/Accuracy Judgment.

The quality of the data collected by the staff of the LTER site

is thought to range from poor to excellent, generally it is

considered to be excellent data. No formal quality assurance

is done, however, missing values are inserted when the data is

known to be very poor. A coarse check is routinely made of

these data.

See this section in the document titled Fifteen-Minute Stream

Flow Rates from USGS for accuracy information for the USGS

station.

10.2.3  Measurement Error for Parameters and Variables.

The measurement error for the data collected by the staff of

the LTER site is unknown.

See this section in the document titled Fifteen-Minute Stream

Flow Rates from USGS for accuracy information for the USGS

station.

10.2.4  Additional Quality Assessment Applied.

FIS staff applied a general Quality Assessment (QA) procedure to

the data to identify inconsistencies and problems for potential

users. As a general procedure, the FIS QA consisted of examining

the maximum, minimum, average, and standard deviation for each

numerical field in the data table. An attempt was made to find an

explanation for unexpected high or low values, values outside of

the normal physical range for a variable, or standard deviations

that appeared inconsistent with the mean. In some cases, histog-

rams were examined to determine whether outliers were consistent

with the shape of the data distribution.

The discrepancies, which were identified, are reported as

problems in Section 11.1.

11.  NOTES

11.1  Known Problems With The Data.

During the general QA performed by the FIS staff the following 26 days

were found to have mean flow less than the minimum flow rates:

OBS_DATE                           OBS_DATE

---------                          ---------

27-JUN-85                          30-SEP-85

28-JUN-85                          02-OCT-85

28-JUN-85                          04-OCT-85

03-JUL-85                          25-OCT-85

03-JUL-85                          30-DEC-85

03-JUL-85                          28-JAN-86

07-JUL-85                          03-FEB-86

07-JUL-85                          10-APR-86

08-JUL-85                          22-MAY-86

26-AUG-85                          03-JUN-86

23-SEP-85                          28-AUG-86

23-SEP-85                          11-SEP-86

23-SEP-85                          24-SEP-86

The data on these days should be used with caution.

11.2  Usage Guidance.

The data collected by the LTER site staff should be used with caution.

It should be checked against the spot measurements which are available

from the LTER site at Kansas State University (KSU). See Section 2.3

for contact information.

None available for the data collected at the USGS station.

11.3  Other Relevant Information.

More detailed information on the USGS gauging stations, such as discharge

measurements, gauge-height records, and rating tables, is on file in the

USGS Kansas District office in Lawrence, Kansas. The USGS keeps most of

this information in computer-usable form along with the statistical

analysis which have been performed on the data.

12.  REFERENCES

12.1  Satellite/Instrument/Data Processing Documentation.

Anonymous. 1993. Methods Manual for Konza Prairie Research Natural Area.

Konza LTER publication.

Replogle, J.A., H. Reikert, B. F. Swindel. 1978. Water Monitoring in

Coastal Forest Watershed Studies. IMPAC Report 2, Vol. 3, No. 2.

Southwestern Forest Expt. Station. USDA. Gainesville, Florida.

U.S. Geological Survey Water-Data Report KS-79-1 Through KS-89-1,

Water Resources Data Kansas, Water Year 1979-, Prepared in cooperation

with the State of Kansas and with other agencies.

12.2  Journal Articles and Study Reports.

Engman, E.T., W. Kustas, T.J. Schmugge, and J.R. Wang. 1987. Relationship

among the remotely sensed soil moisture, streamflow, and evapotrans-

piration. AGU Fall Meeting. San Francisco.

Engman, E.T., G. Angus, and W. Kustas. 1989. Relationships between the

hydrologic balance of a small watershed and remotely sensed soil

moisture. Remote Sensing and Large Scale Processes. IAHS Publ.

No. 186, Proc. IAHS, 3rd Int. Assoc. Baltimore, MD.

Sellers, P.J., F.G. Hall, G. Asrar, D.E. Strebel, and R.E. Murphy.

1988. The First ISLSCP Field Experiment (FIFE). Am. Meteor. Soc.

69:22-27.

Wood, E.F. 1990. Water balance model for Kings Creek. Symposium on

FIFE. Am. Meteorol. Soc. Boston, Massachusetts, p 163-166.

12.3  Archive/DBMS Usage Documentation.

The Collected Data of the First ISLSCP Field Experiment is archived at

the EOS Distributed Active Archive Center (DAAC) at Oak Ridge National

Laboratory (ORNL), Oak Ridge, Tennessee (Section 13 below).

Documentation about using the archive and/or online access to the data

at the ORNL DAAC is not available at this revision.

13.  DATA ACCESS

13.1  Contacts for Archive/Data Access Information.

ORNL DAAC User Services

P.O. Box 2008

Mail Stop 6490

Oak Ridge National Laboratory

Oak Ridge, TN 37831-6490

(615) 241-3952

13.2  Archive Identification.

EOSDIS Distributed Active Archive Center

P.O. Box 2008

Mail Stop 6490

Oak Ridge National Laboratory

Oak Ridge, TN 37831-6490

USA

Telephone:       (615) 241-3952

FAX:             (615) 574-4665

Internet Email:   ornldaac@onrl.gov

13.3  Procedures for Obtaining Data.

Users may place requests by letter, telephone, electronic mail, FAX, or

personal visit.

13.4  ORNL DAAC Status/Plans.

FIFE data will be available from the ORNL DAAC.  Please contact the

ORNL DAAC User Services Office for the most current information about

these data.

14.  OUTPUT PRODUCTS AND AVAILABILITY

14.1  Tape Products.

None.

14.2  Film Products.

None.

14.3  Other Products.

Daily Stream Flow Amounts are available on FIFE CD-ROM Volume 1.

15.  GLOSSARY OF ACRONYMS

CD-ROM            Compact Disk-Read Only Memory

DAAC              Distributed Active Archive Center

EOSDIS            Earth Observing System Data and Information System

FIFE              First ISLSCP Field Experiment

FIS               FIFE Information System

GMT               Greenwich Mean Time

GSFC              Goddard Space Flight Center

IFC               Intensive Field Campaign

ISLSCP            International Satellite Land Surface Climatology Project

KSU               Kansas State University

LTER              Long Term Ecological Research Site, Konza Prairie

ORNL              Oak Ridge National Laboratory

PI                Principal Investigator

USGS              United States Geological Survey

UTM               Universal Transverse Mercator