1.  TITLE

1.1  Data Set Identification.

Site Reflectances Extracted from NS001 Imagery.

1.2  FIS Data Base Table Name.

NS001_TMS_EXTRACT_DATA.

1.3  CD-ROM File Name.

\DATA\SAT_OBS\NS001TMS\ydddMULT.TMS

Note: capital letters indicate fixed values that appear on the CD-ROM

exactly as shown here, lowercase indicates characters (values) that change

for each path and file.

The format used for the filenames is: ydddMULT.sfx, y is the last digit

of the year (e.g., 7=1987, and 9=1989), and ddd is the day of the

year (e.g., 061=sixty-first day in the year). The filename extension

(.sfx), identifies the data set content for the file (see Section 8.2) and

is equal to .TMS for this data set.

1.4  Revision Date Of This Document.

April 24, 1994.

Warning:  This document has not been checked for technical or editorial

accuracy by the FIFE Information Scientist. There may be

inconsistencies with other documents, technical or editorial

errors that were inadvertently introduced when the document was

compiled or references to preliminary data that were not

included on the final CD-ROM.

Previous versions of this document have been reviewed by the

Principal Investigator, the person who transmitted the data to

FIS, a FIS staff member, or a FIFE scientist generally familiar

with the data.

2.  INVESTIGATOR(S)

2.1  Investigator(s) Name And Title.

Staff Science.

2.2  Title Of Investigation.

Staff Science Satellite Data Acquisition Program.

2.3  Contacts (For Data Production Information).

______________________________________________________________________

|              |         Contact 1         |         Contact 2         |

|______________|___________________________|___________________________|

|2.3.1 Name    |Scott Goetz                |Jeffrey A. Newcomer        |

|2.3.2 Address |Code 923                   |Code 923                   |

|              |NASA/Goddard Sp. Fl. Ctr.  |NASA/Goddard Sp. Fl. Ctr.  |

|      City/St.|Greenbelt, MD.             |Greenbelt, MD.             |

|      Zip Code|20771                      |20771                      |

|2.3.3 Tel.    |(301) 286-2447             |(301) 286-7858             |

|2.3.4 Email   |goetz@ltpsun.gsfc.nasa.gov |newcomer@ltp.gsfc.nasa.gov |

|______________|___________________________|___________________________|

2.4  Requested Form of Acknowledgment.

The NS001 site reflectances were extracted from the FIFE Level-1 NS001

imagery by the FIFE Information System staff at Goddard Space Flight

Center.  The work of Mr. Scott Goetz and Ms. Brenda Colesanti is part-

icularly appreciated.

3.  INTRODUCTION

3.1  Objective/Purpose.

The FIFE Staff Science effort covered those activities which would

have been community level activities, or required uniform data collection

procedures across sites and time, hence, would be activities more

appropriate to a team than to individual investigators. These included the

acquisition of data from NASA's NS001 sensor flown on a C130 aircraft

managed by Ames Research Center.

As part of the FIFE staff science data processing effort, the FIFE

Information System (FIS) extracted site average radiances from the

level-1 NS001-TMS products.

3.2  Summary of Parameters.

Site specific radiance, and apparent surface temperature.

3.3  Discussion.

The site average radiances extracted from the NS001 imagery are

instrument-corrected spectral radiances for each of the eight spectral

bands. Geographic location and viewing and solar angles for each of 39

FIFE ground measurement sites are also included for each observation.

The sensor calibrated radiance values were corrected using atmospheric

aerosol optical thickness and gaseous absorption profile measurements,

when available. The atmospheric correction algorithm of Fraser et al.

(1989) was used to calculate reflectance in the visible and infrared

channels. The thermal data are corrected using parameters derived from

the Lowtran7 atmospheric path radiance model (Kneizys et al., 1988).

Data were collected by the NS001 during each of the FIFE IFC's.

Selected flights were processed to level-1. The site averages were

extracted from these processed images. Therefore, this data set

contains a small number of observation dates for each site, but at the

multiple angles provided by the "grid" pattern used during each

flight. The data set should be useful for canopy reflectance modeling

studies.

4.  THEORY OF MEASUREMENTS

The NASA Earth Resources Aircraft program at Ames Research Center operates the

C-130 aircraft to acquire data for Earth science research. The NS001 Multi-

spectral Scanner used on the C130 aircraft collects radiance measurements in

the seven Landsat-4 and -5 Thematic Mapper (TM) bands plus a band from 1000 to

1300 nm. Therefore, when reflected or emitted radiation from Earth surface

features are measured from the aircraft, inferences can be made about Landsat

satellite measurements.

Thematic considerations have dictated, within technical constraints, the choice

of spectral band position and width in the Landsat and NS001 sensors. The eight

bands were chosen after many years of analysis for their value in discrimination of several Earth surface features. A blue (450 to 520 nm) band

provides increased penetration of water bodies as well as supporting analyses

of land use, soil, and vegetation characteristics. The lower-wavelength cutoff

is just below the peak transmittance of clear water, while the upper-wavelength

cutoff is the limit of chlorophyll absorption in the blue region of the

electromagnetic spectrum for healthy green vegetation. Wavelengths below 450 nm

are substantially influenced by atmospheric scattering and absorption.

A green (520 to 600 nm) band spans the region between the blue and red chloro-

phyll absorption bands, and therefore corresponds to the green reflectance of

healthy vegetation. A red (630 to 690 nm) band includes the chlorophyll

absorption band of healthy green vegetation and represents one of the most

important bands for vegetation discrimination. The latter is also useful for

soil boundary and geological boundary delineation. A reflective-infrared (760

to 900 nm) band is especially responsive to the amount of vegetation biomass

present in a scene. It is useful for crop identification and emphasizes soil-

crop and land-water contrasts.

Finally, the thermal infrared band (10.4 to 12.5 um) measures the amount

of infrared radiant flux emitted from surfaces. The apparent temperature is a

function of the emissivities and true or kinetic temperature of the surface.

It is useful for locating geothermal activity, thermal inertia mapping for

geologic investigations, energy balance estimation, evapotranspiration

characteristics, vegetation stress analysis, and soil moisture studies.

5.  EQUIPMENT

5.1  Instrument Description.

The Thematic Mapper Sensor system used to collect the original data from

which this data set was produced has been described in detail in the

document FIFE Level-1 NS001 Thematic Mapper Simulator (TMS) contained on

FIFE CD-ROM Volume 1.

5.1.1  Platform.

See Section 5.1.

5.1.2  Mission Objectives.

See Section 5.1.

5.1.3  Key Parameters.

See Section 5.1.

5.1.4  Principles of Operation.

See Section 5.1.

5.1.5  Instrument Measurement Geometry.

See Section 5.1.

5.1.6  Manufacturer of Instrument.

See Section 5.1.

5.2  Calibration.

See Section 5.1.

5.2.1  Specifications.

See Section 5.1.

5.2.1.1  Tolerance.

See Section 5.1.

5.2.2  Frequency of Calibration.

See Section 5.1.

5.2.3  Other Calibration Information.

See Section 5.1.

6.  PROCEDURE

6.1  Data Acquisition Methods.

The NS001 instrument was flown on NASA's C-130 aircraft during FIFE (see

the FIFE Experiment Plan or the FIFE Interim Report for flight pattern

details and objectives). The FIFE Information System (FIS) staff processed

8-band NS001 Thematic Mapper Simulator (TMS) multispectral scanner data

to FIFE level-1 products. (See the document entitled FIFE Level-1 NS001

Thematic Mapper Simulator (TMS) data for a description of these products.)

6.2  Spatial Characteristics.

The FIFE study area, with areal extent of 15 km by 15 km, is located south

of the Tuttle Reservoir and Kansas River, and about 10 km from Manhattan,

Kansas, USA. The northwest corner of the area has UTM coordinates of

4,334,000 Northing and 705,000 Easting in UTM Zone 14.

6.2.1  Spatial Coverage.

Image coverage was dependent upon the type of mission flown (e.g.,

Coordinated Mission Plan (CMP) 1, 2, or 3, see FIFE Experiment

Plan). In the FIFE, only high level grid flights, with altitudes

ranging from 3313 to 8146 meters, were processed to level 1, and

subsequently to level 2. NS001 data were collected at the 4878

meter altitude during 1987 on flight lines that were approximately

3.5 km apart and 8 km wide. The flight lines were oriented

perpendicular and parallel to the solar plane, ideally with three

lines in each direction. This pattern covered the entire FIFE area,

with multiple views (up to 6) of most of the central portion. The

pattern, altitude, and spacing were modified somewhat in 1989, to

provide intensive coverage of the three "super-sites" (see FIFE-89

Experiment Plan). The pixels extracted from the NS001 images

overlay 61 stations located within 35 sitegrids scattered

throughout the FIFE study area. The sitegrids and stations

underlying these pixels are listed below.

SITEGRID_ID  STN_ID     SITEGRID_ID  STN_ID     SITEGRID_ID  STN_ID

-----------  ------     -----------  ------     -----------  ------

0847-TMS        29      2139-TMS      31        4168-TMS    25

0847-TMS       929      2330-TMS     908        4168-TMS   925

1246-TMS        40      2428-TMS       3        4439-TMS    16

1246-TMS       940      2428-TMS     903        4439-TMS   916

1445-TMS        42      2516-TMS      14        4609-TMS    22

1445-TMS       942      2516-TMS     914        4609-TMS   922

1478-TMS        38      2655-TMS      36        5926-TMS    15

1478-TMS       938      2655-TMS     936        5926-TMS   915

1563-TMS        27      2731-TMS       4        6340-TMS    20

1563-TMS       927      2915-TMS      12        6340-TMS   920

1916-TMS         2      3129-TMS       8        6469-TMS    23

1916-TMS       902      3129-TMS     912        6469-TMS   923

1942-TMS       944      3221-TMS       7        6735-TMS    13

2043-TMS        44      3221-TMS     907        6735-TMS   913

2123-TMS         5      3317-TMS     910        6912-TMS    24

2123-TMS       905      3414-TMS      10        6912-TMS   924

2132-TMS         6      3479-TMS      34        6943-TMS    28

2133-TMS        60      3479-TMS     934        6943-TMS   928

2133-TMS       906      3921-TMS       9        8739-TMS    26

2133-TMS       931      3921-TMS     909        8739-TMS   926

6.2.2  Spatial Resolution.

The NS001 IFOV provides a footprint of 12.2 meters at nadir at an

altitude of 4878 meters (16000 feet). The high-level grid flights

were flown at or near this altitude in 1987.

In 1989, the FIFE grid missions were flown at an altitude of 8077

meters (26500 feet), resulting in a nadir footprint size of 20.2

meters.

6.3  Temporal Characteristics.

6.3.1  Temporal Coverage.

The NS001 data were collected from the C130 during FIFE's five

IFC's during 1987 and 1989. Extracted site radiances are

available for two dates in each IFC except IFC-4:

04-JUN-87         10-JUL-87       11-OCT-87

06-JUN-87         15-AUG-87       04-AUG-89

28-JUN-87         17-AUG-87       11-AUG-89

6.3.2  Temporal Resolution.

The grid-pattern flight lines are 10-20 minutes apart. The lines

processed to level 1 average 20-30 minutes apart, depending on the

day. Two grids, separated by 4 hours, were processed for August 15,

1987. On the day when pixels were extracted from the NS001 images

there were 2 to 5 pixels for each station and each of these pixels

were collected about twenty minutes apart from each other.

7.  OBSERVATIONS

7.1  Field Notes.

Printed flight summary reports produced by Ames Research Center include

the pilot's comments. Other flight notes are available as verbal

records on video tapes. These materials are available at the archive

(see Section 13.0).

8.  DATA DESCRIPTION

8.1  Table Definition With Comments.

The SQL definition for this table is found in the NS001TMS.TDF file

found on FIFE CD-ROM Volume 1.

8.2  Type of Data.

--------------------------------------------------------------------------------

|                 8.2.1                  |               |          |          |

|Parameter/Variable Name                 |               |          |          |

--------------------------------------------------------------------------------

|    |               8.2.2               |     8.2.3     |  8.2.4   |  8.2.5   |

|    |Parameter/Variable Description     |Range          |Units     |Source    |

--------------------------------------------------------------------------------

|SITEGRID_ID                             |               |          |          |

|    |This is a FIS grid location code.  |               |          |          |

|    |Site grid codes (SSEE-III) give    |               |          |          |

|    |the south (SS) and the east (EE)   |               |          |          |

|    |cell number in a 100x100 array of  |               |          |          |

|    |200m square cells.  The last 3     |               |          |          |

|    |characters (III) are an instrument |               |          |          |

|    |identifier.                        |               |          |          |

--------------------------------------------------------------------------------

|STATION_ID                              |               |          |          |

|    |The FIS site identifier used to    |               |          |          |

|    |designate this site (details are   |               |          |          |

|    |in the FIFE_SITE_REF table).       |               |          |          |

--------------------------------------------------------------------------------

|OBS_DATE                                |               |          |          |

|    |The date (expressed as             |               |          |          |

|    |DD-MMM-YY), on which the image     |               |          |          |

|    |data was recorded.                 |               |          |          |

--------------------------------------------------------------------------------

|OBS_TIME                                |               |          |          |

|    |The time (GMT) at the center of    |               |[GMT]     |          |

|    |the level-1 image when the data    |               |          |          |

|    |were collected.                    |               |          |          |

--------------------------------------------------------------------------------

|IMAGE_ID                                |               |          |          |

|    |The image identifier that          |               |          |          |

|    |identifies the FIS level-1         |               |          |          |

|    |satellite image from which the     |               |          |          |

|    |site statistics were derived.      |               |          |          |

--------------------------------------------------------------------------------

|NUM_OBS                                 |               |          |          |

|    |The number of observations         |               |[pixels]  |          |

|    |(pixels) found within the site     |               |          |          |

|    |coordinate boundaries and used in  |               |          |          |

|    |the statistics calculations.       |               |          |          |

--------------------------------------------------------------------------------

|MIN_LAT                                 |               |          |          |

|    |The minimum latitude of all the    |               |          |          |

|    |pixels extracted from the level-1  |               |          |          |

|    |image and used to derive the site  |               |          |          |

|    |statistics (DD MM SS.SS).          |               |          |          |

--------------------------------------------------------------------------------

|MAX_LAT                                 |               |          |          |

|    |The maximum latitude of all the    |               |          |          |

|    |pixels extracted from the level-1  |               |          |          |

|    |image and used to derive the site  |               |          |          |

|    |statistics (DD MM SS.SS).          |               |          |          |

--------------------------------------------------------------------------------

|MIN_LON                                 |               |          |          |

|    |The minimum longitude of all the   |               |          |          |

|    |pixels extracted from the level-1  |               |          |          |

|    |image and used to derive the site  |               |          |          |

|    |statistics (DDD MM SS.SS).         |               |          |          |

--------------------------------------------------------------------------------

|MAX_LON                                 |               |          |          |

|    |The maximum longitude of all the   |               |          |          |

|    |pixels extracted from the level-1  |               |          |          |

|    |image and used to derive the site  |               |          |          |

|    |statistics (DDD MM SS.SS).         |               |          |          |

--------------------------------------------------------------------------------

|CENTER_VIEW_ZEN_ANG                     |               |          |          |

|    |The view zenith angle at the       |               |[degrees] |          |

|    |center of the site.                |               |          |          |

--------------------------------------------------------------------------------

|CENTER_VIEW_AZIM_ANG                    |               |          |          |

|    |The view azimuth angle at the      |               |[degrees f|          |

|    |center of the site (North = 0,     |               |rom North]|          |

|    |East = 90, South = 180, West       |               |          |          |

|    |= 270).                            |               |          |          |

--------------------------------------------------------------------------------

|CENTER_SOLAR_ZEN_ANG                    |               |          |          |

|    |The solar zenith angle at the      |               |[degrees] |          |

|    |center of the site.                |               |          |          |

--------------------------------------------------------------------------------

|CENTER_SOLAR_AZIM_ANG                   |               |          |          |

|    |The solar azimuth angle at the     |               |[degrees f|          |

|    |center of the site (North = 0,     |               |rom North]|          |

|    |East = 90, South = 180, West       |               |          |          |

|    |= 270).                            |               |          |          |

--------------------------------------------------------------------------------

|BAND1_RADNC                             |               |          |          |

|    |The average radiance over the      |               |[Watts][me|          |

|    |site for TMS Band 1 (.458-.519     |               |ter^-2]   |          |

|    |microns).                          |               |          |          |

--------------------------------------------------------------------------------

|BAND1_RADNC_SDEV                        |               |          |          |

|    |The standard deviation of the      |               |[Watts][me|          |

|    |radiance values over the site for  |               |ter^-2]   |          |

|    |TMS Band 1 (.458-.519 microns).    |               |          |          |

--------------------------------------------------------------------------------

|BAND2_RADNC                             |               |          |          |

|    |The average radiance over the      |               |[Watts][me|          |

|    |site for TMS Band 2 (.529-.603     |               |ter^-2]   |          |

|    |microns).                          |               |          |          |

--------------------------------------------------------------------------------

|BAND2_RADNC_SDEV                        |               |          |          |

|    |The standard deviation of the      |               |[Watts][me|          |

|    |radiance values over the site for  |               |ter^-2]   |          |

|    |TMS Band 2 (.529-.603 microns).    |               |          |          |

--------------------------------------------------------------------------------

|BAND3_RADNC                             |               |          |          |

|    |The average radiance over the      |               |[Watts][me|          |

|    |site for TMS Band 3 (.633-.697     |               |ter^-2]   |          |

|    |microns).                          |               |          |          |

--------------------------------------------------------------------------------

|BAND3_RADNC_SDEV                        |               |          |          |

|    |The standard deviation of the      |               |[Watts][me|          |

|    |radiance values over the site for  |               |ter^-2]   |          |

|    |TMS Band 3 (.633-.697 microns).    |               |          |          |

--------------------------------------------------------------------------------

|BAND4_RADNC                             |               |          |          |

|    |The average radiance over the      |               |[Watts][me|          |

|    |site for TMS Band 4 (.767-.910     |               |ter^-2]   |          |

|    |microns).                          |               |          |          |

--------------------------------------------------------------------------------

|BAND4_RADNC_SDEV                        |               |          |          |

|    |The standard deviation of the      |               |[Watts][me|          |

|    |radiance values over the site for  |               |ter^-2]   |          |

|    |TMS Band 4 (.767-.910 microns).    |               |          |          |

--------------------------------------------------------------------------------

|BAND5_RADNC                             |               |          |          |

|    |The average radiance over the      |               |[Watts][me|          |

|    |site for TMS Band 5 (1.13-1.35     |               |ter^-2]   |          |

|    |microns).                          |               |          |          |

--------------------------------------------------------------------------------

|BAND5_RADNC_SDEV                        |               |          |          |

|    |The standard deviation of the      |               |[Watts][me|          |

|    |radiance values over the site for  |               |ter^-2]   |          |

|    |TMS Band 5 (1.13-1.35 microns).    |               |          |          |

--------------------------------------------------------------------------------

|BAND6_RADNC                             |               |          |          |

|    |The average radiance over the      |               |[Watts][me|          |

|    |site for TMS Band 6 (1.57-1.71     |               |ter^-2]   |          |

|    |microns).                          |               |          |          |

--------------------------------------------------------------------------------

|BAND6_RADNC_SDEV                        |               |          |          |

|    |The standard deviation of the      |               |[Watts][me|          |

|    |radiance values over the site for  |               |ter^-2]   |          |

|    |TMS Band 6 (1.57-1.71 microns).    |               |          |          |

--------------------------------------------------------------------------------

|BAND7_RADNC                             |               |          |          |

|    |The average radiance over the      |               |[Watts][me|          |

|    |site for TMS Band 7 (2.10-2.38     |               |ter^-2]   |          |

|    |microns).                          |               |          |          |

--------------------------------------------------------------------------------

|BAND7_RADNC_SDEV                        |               |          |          |

|    |The standard deviation of the      |               |[Watts][me|          |

|    |radiance values over the site for  |               |ter^-2]   |          |

|    |TMS Band 7 (2.10-2.38 microns).    |               |          |          |

--------------------------------------------------------------------------------

|BAND8_RADNC                             |               |          |          |

|    |The average radiance over the      |               |[Watts][me|          |

|    |site for TMS Band 8 (10.9-12.3     |               |ter^-2]   |          |

|    |microns).                          |               |          |          |

--------------------------------------------------------------------------------

|BAND8_RADNC_SDEV                        |               |          |          |

|    |The standard deviation of the      |               |[Watts][me|          |

|    |radiance values over the site for  |               |ter^-2]   |          |

|    |TMS Band 8 (10.9-12.3 microns).    |               |          |          |

--------------------------------------------------------------------------------

|BAND8_TEMP                              |               |          |          |

|    |The average temperature over the   |               |[degrees C|          |

|    |site for TMS Band 8 (10.9-12.3     |               |elsius]   |          |

|    |microns).                          |               |          |          |

--------------------------------------------------------------------------------

|BAND8_TEMP_SDEV                         |               |          |          |

|    |The standard deviation of the      |               |[degrees C|          |

|    |temperature values over the site   |               |elsius]   |          |

|    |for TMS Band 8 (10.9-12.3 microns).|               |          |          |

--------------------------------------------------------------------------------

|FIFE_DATA_CRTFCN_CODE                   |      *        |          |          |

|    |The FIFE Certification Code for    |               |          |          |

|    |the data, in the following format: |               |          |          |

|    |CPI (Certified by PI), CPI-???     |               |          |          |

|    |(CPI - questionable data).         |               |          |          |

--------------------------------------------------------------------------------

|LAST_REVISION_DATE                      |               |          |          |

|    |The last revision date for the     |               |          |          |

|    |data, in the format (DD-mmm-YY).   |               |          |          |

--------------------------------------------------------------------------------

Footnote:

*  Valid levels

The primary certification codes are:

EXM - Example or Test data (not for release).

PRE - Preliminary (unchecked, use at your own risk).

CPI - Checked by Principal Investigator (reviewed for quality).

CGR - Checked by a group and reconciled (data comparisons and

cross-checks).

The certification code modifiers are:

PRE-NFP  - Preliminary - Not for publication, at the request of

investigator.

CPI-MRG  - PAMS data that are "merged" from two separate receiving

stations to eliminate transmission errors.

CPI-???  - Investigator thinks data item may be questionable.

8.3  Sample Data Base Data Record.

SITEGRID_ID STATION_ID OBS_DATE  OBS_TIME IMAGE_ID         NUM_OBS

----------- ---------- --------- -------- ---------------- -------

1916-TMS             2 17-AUG-87     2000 NS870619L4R1J-1      120

2428-TMS             3 17-AUG-87     2000 NS870619L4R1J-1      208

2731-TMS             4 17-AUG-87     2000 NS870619L4R1J-1       64

3221-TMS             7 15-AUG-87     1531 NS870614L3R1E-1       10

MIN_LAT      MAX_LAT      MIN_LON        MAX_LON        CENTER_VIEW_ZEN_ANG

------------ ------------ -------------- -------------- -------------------

39 05 52.94  39 05 59.02   -96 35 39.16   -96 35 32.56                14.9

39 05 18.83  39 05 27.80   -96 33 54.44   -96 33 43.63                32.4

39 04 57.75  39 05 02.41   -96 33 39.47   -96 33 33.50                32.3

39 04 30.56  39 04 32.13   -96 34 54.72   -96 34 52.38                35.6

CENTER_VIEW_AZIM_ANG CENTER_SOLAR_ZEN_ANG CENTER_SOLAR_AZIM_ANG BAND1_RADNC

-------------------- -------------------- --------------------- -----------

239.3                 32.6                 224.6      35.766

239.3                 32.6                 224.6      40.556

239.3                 32.6                 224.6      37.797

221.3                 45.3                 109.9      21.050

BAND1_RADNC_SDEV BAND2_RADNC BAND2_RADNC_SDEV BAND3_RADNC BAND3_RADNC_SDEV

---------------- ----------- ---------------- ----------- ----------------

1.2397      38.287           1.7649      33.718           2.5245

.8878      48.669           1.8404      38.383           1.8919

.9827      47.554           1.2760      35.636           1.9901

.3893      25.382           1.1843      19.822            .9865

BAND4_RADNC BAND4_RADNC_SDEV BAND5_RADNC BAND5_RADNC_SDEV BAND6_RADNC

----------- ---------------- ----------- ---------------- -----------

55.038           4.4421     -99.000            .0000      13.851

91.634           5.4980     -99.000            .0000      14.375

93.334           2.9207     -99.000            .0000      13.262

54.597           5.8005     -99.000            .0000       8.827

BAND6_RADNC_SDEV BAND7_RADNC BAND7_RADNC_SDEV BAND8_RADNC BAND8_RADNC_SDEV

---------------- ----------- ---------------- ----------- ----------------

.7164       2.543            .1931      9.9221            .1732

.6432       2.297            .1616      9.6822            .1528

.5623       2.059            .1642      9.3804            .0737

.4354       1.297            .0989      9.4378            .0440

BAND8_TEMP BAND8_TEMP_SDEV FIFE_DATA_CRTFCN_CODE LAST_REVISION_DATE

---------- --------------- --------------------- ------------------

31.5            1.27 CPI                   30-AUG-93

29.8            1.13 CPI                   30-AUG-93

27.5             .55 CPI                   30-AUG-93

27.9             .33 CPI                   30-AUG-93

8.4  Data Format.

The CD-ROM file format consists of numerical and character fields of

varying length separated by commas. The character fields are enclosed with

a single apostrophe. There are no spaces between the fields.  Each file

begins with five header records.  Header records contain the following

information:

Record 1    Name of this file, its table name, number of records in

this file, path and name of the document that describes

the data in this file, and name of principal investigator

for these data.

Record 2    Path and filename of the previous data set, and path and

filename of the next data set.  (Path and filenames for files

that contain another set of data taken at the same site on

the same day.)

Record 3    Path and filename of the previous site, and path and filename

of the next site.  (Path and filenames for files of the same

data set taken on the same day for the previous and next

sites (sequentially numbered by SITEGRID_ID)).

Record 4    Path and filename of the previous date, and path and filename

of the next date.  (Path and filenames for files of the same

data set taken at the same site for the previous and next

date.)

Record 5    Column names for the data within the file, delimited by

commas.

Record 6    Data records begin.

Each field represents one of the attributes listed in the chart in

section 8.2 and described in detail in the TDF file (see section 8.1).

These fields are in the same order as in the chart.

8.5  Related Data Sets.

Landsat Thematic Mapper (TM) Averages.

FIFE Level-3 Multispectral Atmospheric Mapping Sensor (MAMS).

FIFE Level-1 NS001 Thematic Mapper Simulator (TMS).

FIFE Level-2 NS001 Atmospherically Corrected Radiances and Surface

Temperatures.

Sunphotometer Optical Thickness Data from C130 Aircraft.

SPOT High Resolution Visible (HRV) Averages.

FIFE Level-1 Advanced Solid-state Array Spectroradiometer (ASAS) Imagery.

9.  DATA MANIPULATIONS

9.1  Formulas.

9.1.1  Derivation Techniques/Algorithms.

Thermal Band apparent temperature:

GSFC Gain (G) and Offset (C), as found in the header summary file

(s), are calculated as follows:

a) Calculate black body radiances, Lw([mWatts][centimeter^-2]

[steradian^-1]

[micrometer^-1])

(assume emissivity = 1) for BB1 and BB2 temperatures T(K)

e.g.,

Lw,bb1 = (K1 / (exp(K2/Tbb1)-1))

K1 = 60.705 [mWatts][centimeter^-2][steradian^-1]

[micrometer^-1]

K2 = 1258.39 [degrees K]

K1, K2 were "best fit" parameters for the temperature

range of 273-323 K using the 8/87 NS001 spectral

data and the Planck equation.

b) G = ((BB2 View - BB1 View) / (Lw,bb2 - Lw,bb1))

([Counts][mWatts^-1][centimeter^2][steradian]

[micrometer])

C = BB1 View - G * Lw,bb1 [counts]

Target Radiance (Lw) can then be calculated as:

(pixel value - C) / G

and at-sensor apparent temperature as:

T = K2 / ln(K1/Lw + 1)

9.2  Data Processing Sequence.

9.2.1  Processing Steps and Data Sets.

FIS created the level-1A data by:

1) Enumerating the pixel and line coordinates of the pixels

that fall within the site locations as identified on a June

6, 1987 SPOT panchromatic reference image. The sites are

each manually located because automated enumeration using

image registration transformations cannot adequately capture

the distortions introduced to the imagery by aircraft motion.

2) Extracting the spectral, geographic, viewing, and solar

information for the enumerated pixels.

3) Converting and correcting the digital spectral counts to

radiance

4) Storing appropriate information in the data base.

9.2.2  Processing Changes.

Not available at this revision.

9.3  Calculations.

9.3.1  Special Corrections/Adjustments.

The NS001 internal integrating sphere is radiance calibrated in

the laboratory and used as the in-flight reference for calibrating

NS001 data. Documented here are the procedures used for FIFE data.

Laboratory Procedures:

a. Calculate the radiance of the laboratory source in the spectral

band-passes of the instrument.

b. Measure the instrument response to at least two known radiance

levels and determine the instrument gain and offset.

c. Transfer the calibration to the internal source.

d. Monitor the internal source calibration by reference to an

external radiance source.

Data Processing Procedures:

e. Determine "in-flight" sensor gain and offset.

f. Apply the calibration to the field data.

To correct at-sensor values to apparent surface values, Lowtran7-

calculated path radiance and transmission at 11 view zenith angles

(-50,-40,-30,-20,-10,0,+10,+20,+30,+40, and +50 degrees) are convolved

with the NS001 thermal channel sensor response and are then linearly

interpolated across the 100 degree scan angle of the instrument. These

data are used to correct the at-sensor radiances to apparent atmospheric-

ally corrected surface radiances and then to surface temperature, on a

pixel-by-pixel basis. The program that calculates the surface temperature

scales the data from 0-255, with each count representing 0.2 degrees

Celsius (for example, a count of 198 is 39.6 degrees C).

9.4  Graphs and Plots.

None.

10.  ERRORS

10.1 Sources of Error.

The NS001 data are calibrated in-flight by reference to the NS001

internal integrating sphere source. Apparent instabilities in this

source or its monitoring circuitry, which are not fully understood,

are the principal limiting factors in the absolute calibration of

NS001 data. Uncertainties due solely to this behavior may reach 25%

in 1987, though more typically are expected to be less than 15%.

Other identified error sources at the 1-2% level for typical signals

include dark current drift along the scan line, hysteresis-like

sensitivity changes along the scan line, random noise,

scan-speed-induced errors, and non-linearity of radiance with

wavelength. Also, possible operator or human error could result

due to gain/offset setting made by the crew to "optimize" instrument

sensitivity for each flight line.

The NS001 optical system failed during IFC-2, resulting in early

termination of that IFC. The dichroic lens was replaced at that time,

potentially changing alignments in the optical system and changing

calibration characteristics.

Polarization sensitivity of the NS001 was such that for typical atmos-

pheric conditions, errors in channel 1 (0.45-0.52 um) radiances would be

up to +/-10% and vary with scan angle; this progressively decreases with

increasing wavelength (Markham and Ahmad 1990).

The geometric quality of the image depends on (a) the accuracy of the

viewing geometry, and (b) the ground control points as required to adjust

the viewing model.

Spectral errors could arise due to image wide signal-to-noise ratio,

saturation, cross-talk, spikes, response normalization due to change in

gain.

10.2  Quality Assessment.

10.2.1  Data Validation by Source.

The extracted FIFE site radiances have been used in several

multiple-sensor calibration studies.

10.2.2  Confidence Level/Accuracy Judgment.

Ahmad and Markham (1992) have determined that the NS001 when

scanning in the solar meridian plane, a typical condition for

FIFE, produce radiance errors over a vegetated target of 10%,

depending on the scanner orientations and sample number. When the

radiances are converted to surface reflectance, in the worst case

a 1.5 to 4.5% surface reflectance value will result in a +/-45%

error in reflectivity.

10.2.3  Measurement Error for Parameters and Variables.

The Noise Equivalent Spectral Radiance for the channels ranges

from 0.08 to 2.77 micro watts per square cm. Uncertainties due to

the behavior of the internal integrating sphere may have reached

25% in 1987, though more typically they are expected to be less

than 15%. Random error in the NS001 mean reflectance data was

primarily due to instrument noise, since the whole site is imaged

and averaged. Instrumental noise is roughly 1 count, which

translates to a 0.001 to 0.002 error in the mean retrieved

reflectance.

Accuracy is plus or minus 1 degree in view zenith. View azimuth

accuracy is dependent on the quality of onboard navigational data

and aircraft tracking, and is probably no better than 2-3

degrees.

10.2.4  Additional Quality Assessment Applied.

FIS staff applied a general Quality Assessment (QA) procedure to

the data to identify inconsistencies and problems for potential

users. As a general procedure, the FIS QA consisted of examining

the maximum, minimum, average, and standard deviation for each

numerical field in the data table. An attempt was made to find an

explanation for unexpected high or low values, values outside of

the normal physical range for a variable, or standard deviations

that appeared inconsistent with the mean. In some cases, histog-

rams were examined to determine whether outliers were consistent

with the shape of the data distribution.

The discrepancies, which were identified, are reported as

problems in Section 11.1.

11.  NOTES

11.1  Known Problems With the Data.

During the general quality assessment carried out by the FIFE staff the

following questionable data points were found. These data should be used

with caution.

Variable              Date                      Value

--------------        --------                  --------

BAND1_RADNC           10-JUL-87                  9 values < 0

BAND1_RADNC_SDEV      11-OCT, 6-JUN, &

10-JUL-87                 13 values > 10

BAND2_RADNC            4-AUG-89                  9 values > 100

BAND2_RADNC_SDEV      JUN 4, 6, & 28 1987,

10-JUL-87 &

4-AUG-89#                51 values > 10

BAND4_RADNC           10-JUL-87                  4 values < 0

BAND4_RADNC_SDEV      10-JUL-87, 15-AUG-87,

4,6,10-JUN-87             35 values > 1

BAND5_RADNC                                            ALL -99

BAND5_RADNC_SDEV                                -99 <-> 0

BAND6_RADNC            4-JUN-87   Station 15     20.539

BAND7_RADNC            4-JUN-87   Station 15      4.627, 5.453

BAND8_TEMP            Peculiarly low values      4 values < 0

------------------

Footnotes:

#  Majority of the high readings were on this date.

In addition, these extract data have most of the problems encountered

with the original level-1 NS001 data. These problems are detailed in the

document entitled FIFE Level-1 NS001 Thematic Mapper Simulator (TMS).

Consult that document if questions arise concerning an individual extract

provided in this data set.

11.2  Usage Guidance.

The user should be aware that these radiance and temperature values are

not calibrated to measure ground values for radiance or temperatures.

Therefore, while they are useful in a relative sense for comparisons with

in a scene, they may not be accurate where absolute radiances or

temperatures are required. Comparisons with actual ground-acquired

values should be made in such cases. See Markham et al. (1992);

Goetz et al. (1993, 1994) for validation results.

11.3  Other Relevant Information.

Two in-flight adjustments are made that affect the radiometric

calibration of the reflective channels. The primary adjustment is to

the post-amplifier gain of each channel. This is adjusted by means of a

channel specific potentiometer before and between data acquisitions to

optimize the spread of the data across the range of the A/D converter

(8 bits). The gain settings are continuously variable and are not

directly recorded in the data. They are inferred from changes in the

instrument response to the integrating sphere. The second adjustment

is for scan speed, which is adjusted between 10 and 85 scans per second

to maintain contiguous scan lines, or some multiple of contiguous if

contiguity is not maintainable at the altitude required for data

collection.

12.  REFERENCES

12.1  Satellite/Instrument/Data Processing Documentation.

Operations Manual - NS001 Multispectral Scanner. 1977. Lyndon B. Johnson

Space Flight Center. Document No. JSC 12715.

NASA. 1990. C-130 Earth Resources Aircraft Experimenter's Handbook.

National Aeronautics and Space Administration. Ames Research Center.

Moffett Field, California.

Kneizys, F.X., E.P. Shettle, L.W. Abreu, J.H. Chetwynd, G.P. Anderson,

W.O. Gallery, J.E.A. Selby, and S.A. Clough. 1988. User's Guide to

LOWTRAN 7. Rep. AFGL-TR-88-0177. E.R.P. 1010. 146 p. Air Force

Geophys. Lab. Hanscom AFB, Massachusetts.

Airborne Instrumentation Research Project - Flight Summary Report,

Flight No. 87-004 Series. May-June 1987. NASA Ames Research Center.

Airborne Missions and Applications Division. Moffett Field.

California 94035.

Airborne Instrumentation Research Project - Flight Summary Report,

Flight No. 87-005 Series. June-July 1987. NASA Ames Research Center.

Airborne Missions and Applications Division. Moffett Field.

California 94035.

Airborne Instrumentation Research Project - Flight Summary Report,

Flight No. 87-006 Series. August 1987. NASA Ames Research Center.

Airborne Missions and Applications Division. Moffett Field.

California 94035

Airborne Instrumentation Research Project - Flight Summary Report,

Flight No. 88-001 Series. October 1987. NASA Ames Research Center.

Airborne Missions and Applications Division. Moffett Field.

California 94035.

Airborne Instrumentation Research Project - Flight Summary Report,

Flight No. 89-006 Series. July-August 1987. NASA Ames Research Center.

Airborne Missions and Applications Division. Moffett Field.

California 94035.

12.2  Journal Articles and Study Reports.

Ahmad, S.P., and B.L. Markham. 1990. Polarization sensitivity of the NS001

Thematic Mapper Simulator. J. Geophys. Res. 97:18815-18827.

Fraser, R.S., R.A. Ferrare, Y.J. Kaufman, B.L. Markham, and S. Mattoo.

1992. Algorithm for atmospheric correction of aircraft and satellite

imagery. Int. J. Remote sensing. 13:541-557.

Fraser, R.S., R.A. Ferrare, Y.J. Kaufman, and S. Mattoo. 1989. Algor-

ithm for atmospheric corrections of aircraft and satellite imagery.

Tech. Memo. No. 100751. NASA/GSFC. Greenbelt, Maryland.

Goetz, S.J., R. Halthore, F.G. Hall, and B.L. Markham. 1994. Surface

temperature retrieval from multi-resolution sensors at the FIFE site.

J. Geophys. Res. FIFE Special Issue II. In press.

Goetz, S.J., F.G. Hall, B.L. Markham, and R.O. Dubayah 1993. Inter-

comparison of retrieved surface temperature from multi-resolution

sensors at the FIFE site. Proceedings of the American Society of

Photogrammetry. p. 108-117. New Orleans, LA.

Goetz, S.J., B.L. Markham, and J.E. Newcomer. 1992. Radiometric

calibration and atmospheric correction of satellite and aircraft data

for FIFE. Proc. International Geoscience and Remote Sensing Symposium.

1:798-801. Houston, Texas.

Hall, F.G., P.J. Sellers, I. MacPherson, R.D. Kelly, S. Verma, B. Markham,

B. Blad, J. Wang, and D.E. Strebel. 1989. FIFE: Analysis and Results

- A Review. Adv. Space Res. 9:275-293.

King, M.D., D.M. Byrne, B.M. Herman, and J.A. Reagan. 1978. Aerosol size

distributions obtained by inversion of spectral optical depth

measurements. J. Atmos. Sci. 35:2153-2167.

Markham, B.L., and J.L. Baker. 1986. Landsat MSS and TM post calibration

dynamic ranges, exoatmospheric reflectances and at-satellite

temperatures. Landsat Tech. Note. Vol. 1:3-8. EOSAT. Lanham, Maryland.

Markham, B.L., F.M. Wood, Jr., and S.P. Ahmad. 1988. Radiometric

calibration of the reflective bands of NS001-Thematic Mapper Simulator

and Modular Multispectral Radiometers. In: Recent Advances in Sensors.

Radiometry and Data Processing for Remote Sensing. Proc. SPIE Vol.

924. Bellingham, WA. p. 96-108.

Markham, B.L., and S.P. Ahmad. 1990. Radiometric properties of the NS001

Thematic Mapper Simulator aircraft multispectral scanner. Remote Sens.

Environ. 34:133-149.

Markham, B.L., R.N. Halthore, and S.J. Goetz. 1992. Surface reflectance

retrieval from satellite and aircraft sensors: results of sensor and

algorithm comparisons during FIFE. J. Geophys. Res. 97:18785-18795.

Newcomer, J.A., S.J. Goetz, D.E. Strebel, and F.G. Hall. 1989. Image

processing software for providing radiometric inputs to land surface

climatology models. IGARSS '89, 12th Can. Symp. on Remote Sensing.

p. 1779-1782.

Richard, R.R., R.F. Merkel, and G.R. Meeks. 1978. NS001MS - Landsat-D

Thematic Mapper Band Aircraft Scanner. In: Proc. 12th Int. Sym. Remote

Sens. Environ. p. 719-728.

Sellers, P.J., F.G. Hall, G. Asrar, D.E. Strebel, and R.E. Murphy. 1988.

The First ISLSCP Field Experiment (FIFE). Bull. of the Am. Meteorol-

ogical Soc. 69:22-27.

Sellers, P.J., and F.G. Hall. 1987. The First ISLSCP Field Experiment

(FIFE). FIFE-87 Experiment Plan. GSFC. Greenbelt, Maryland.

Sellers, P.J., and F.G. Hall. 1989. The First ISLSCP Field Experiment

(FIFE). FIFE-89 Experiment Plan. GSFC. Greenbelt, Maryland.

Sellers P.J., F.G. Hall, D.E. Strebel, E.T. Kanemasu, R.D. Kelly, B.L.

Blad, B.J. Markham, and J.R. Wang. 1990. Experiment design and

operations. AMS Symposium on the First ISLSCP Field Experiment (FIFE).

Anaheim, CA. February 7-9.

Sellers, P.J., F.G. Hall, D.E. Strebel, R.D. Kelly, S.B. Verma, B.L.

Markham, B.L. Blad, D.S. Schimel, J.R. Wang, and E. Kanemasu. 1990.

The First ISLSCP Field Experiment (FIFE). FIFE Interim Report. GSFC.

Greenbelt, Maryland.

Strebel, D.E., S.J. Goetz, and F.G. Hall. 1987. Atmospheric correction of

NS001 data and extraction of multiple angle reflectance data sets. In:

Proc. 21st Int. Sym. Remote Sens. Environ. ERIM. Ann Arbor, MI.

p. 939-948.

Wrigley, R.C., M.A. Spanner, R.E. Slye, R.F. Pueschel, and H.R. Aggarwal.

1992. Atmospheric correction of remotely sensed image data. J.

Geophys. Res. 97:18797-18814.

12.3  Archive/DBMS Usage Documentation.

The Collected Data of the First ISLSCP Field Experiment is archived at

the EOS Distributed Active Archive Center (DAAC) at Oak Ridge National

Laboratory (ORNL), Oak Ridge, Tennessee (Section 13 below).

Documentation about using the archive and/or online access to the data

at the ORNL DAAC is not available at this revision.

13.  DATA ACCESS

13.1  Contacts for Archive/Data Access Information.

ORNL DAAC User Services

P.O. Box 2008

Mail Stop 6490

Oak Ridge National Laboratory

Oak Ridge, TN 37831-6490

(615) 241-3952

13.2  Archive Identification.

EOSDIS Distributed Active Archive Center

P.O. Box 2008

Mail Stop 6490

Oak Ridge National Laboratory

Oak Ridge, TN 37831-6490

USA

Telephone:       (615) 241-3952

FAX:             (615) 574-4665

Internet Email:   ornldaac@onrl.gov

13.3  Procedures for Obtaining Data.

Users may place requests by letter, telephone, electronic mail, FAX, or

personal visit.

13.4  ORNL DAAC Status/Plans.

FIFE data will be available from the ORNL DAAC.  Please contact the

ORNL DAAC User Services Office for the most current information about

these data.

14.  OUTPUT PRODUCTS AND AVAILABILITY

14.1  Tape Products.

None.

14.2  Film Products.

None.

14.3  Other Products.

The Site Reflectances Extracted from NS001 Imagery are available on FIFE

CD-ROM Volume 1.

15.  GLOSSARY OF ACRONYMS

A/D               Analog to Digital

agl               above ground level

ARC               Ames Research Center

ASAS              Advanced Solid-state Array Spectroradiometer

BPI               Byte per inch

CCT               Computer Compatible Tape

CD-ROM            Compact Disk (optical), Read-Only Memory

CMP               Coordinated Mission Plan

DAAC              Distributed Active Archive Center

EOSDIS            Earth Observing System Data and Information System

FIFE              First ISLSCP Field Experiment

FIS               FIFE Information System

GMT               Greenwich Mean Time

GSFC              Goddard Space Flight Center

IFC               Intensive Field Campaign

IFOV              Instantaneous Field of View

IR                Infrared

ISLSCP            International Satellite Land Surface Climatology Project

ORNL              Oak Ridge National Laboratory

PRT               Precision Radiation Thermometer

TM                Thematic Mapper

TMS               Thematic Mapper Simulator

UTM               Universal Transverse Mercator