1.  TITLE

1.1  Data Set Identification.

Site Average Reflectances Extracted from AVHRR-LAC Imagery.

1.2  FIS Data Base Table Name.

SATELLITE_EXTRACT_AVHRR_DATA.

1.3  CD-ROM File Name.

\DATA\SAT_OBS\SAT_AVHR\Yyyyy\ydddFIFE.AVH

Where yyyy are the four digits of the century and year (e.g. 1987).

Note: capital letters indicate fixed values that appear on the CD-ROM

exactly as shown here, lower case indicates characters (values) that

change for each path and file.

The format used for the filenames is: ydddFIFE.sfx, where y is the

last digit of the year (e.g. 7=1987, and 9=1989), and ddd is the day of

the year (e.g. 061=sixty-first day in the year). The filename extension

(.sfx), identifies the data set content for the file (see Section 8.2)

and is equal to .AVH for this data set.

1.4  Revision Date Of This Document.

April 25, 1994.

Warning:  This document has not been checked for technical or editorial

accuracy by the FIFE Information Scientist. There may be

inconsistencies with other documents, technical or editorial

errors that were inadvertently introduced when the document was

compiled or references to preliminary data that were not

included on the final CD-ROM.

Previous versions of this document have been reviewed by the

Principal Investigator, the person who transmitted the data to

FIS, a FIS staff member, or a FIFE scientist generally familiar

with the data.

2.  INVESTIGATOR(S)

2.1  Investigator(s) Name And Title.

Staff Science.

2.2  Title Of Investigation.

Staff Science Satellite Data Acquisition Program.

2.3  Contacts (For Data Production Information).

__________________________________________

|              |         Contact 1         |

|______________|___________________________|

|2.3.1 Name    |Jeffrey A. Newcomer        |

|2.3.2 Address |Code 923                   |

|              |NASA/Goddard Sp. Fl. Ctr.  |

|      City/St.|Greenbelt, MD.             |

|      Zip Code|20771                      |

|2.3.3 Tel.    |(301) 286-7858             |

|2.3.4 Email   |newcomer@ltp.gsfc.nasa.gov |

|______________|___________________________|

2.4  Requested Form of Acknowledgment.

The Site Average Reflectance Extracted from AVHRR-LAC Imagery were

produced by the FIFE Information System staff. The dedicated work

of Scott Goetz and Jeff Newcomer is especially appreciated.

3.  INTRODUCTION

3.1  Objective/Purpose.

The FIFE Staff Science effort covered those activities which were FIFE

community level activities, or required uniform data collection procedures

across sites and time. These activities included acquiring and processing

data from the Advanced Very High Resolution Radiometer (AVHRR) instruments

on the NOAA-9, NOAA-10, and NOAA-11 satellites.

As part of the FIFE staff science data collection effort, the FIFE Inform-

ation System (FIS) processed the level-1 NOAA AVHRR Local Area Coverage

(LAC) products to extract products.

3.2  Summary of Parameters.

Site specific radiance, exoatmospheric reflectance and surface

reflectance.

3.3  Discussion.

This data set consists of averages of pixel extracts from AVHRR-LAC

(1 km resolution) scenes that overlay the FIFE site. The entire FIFE

area is designated as STATION_ID 99 (sitegrid=FIFE-LAC) in the data set.

Average radiances for dates are available for five sensor wavebands and

average reflectance and exoatmospheric reflectances are available for

wavebands 1 and 2. Site averages are clustered in 1987 and during the

summer of 1989. Some data are also available for early 1988.

4.  THEORY OF MEASUREMENTS

The Advanced Very High Resolution Radiometer (AVHRR) is a four- or five-channel

scanning radiometer capable of providing global daytime and nighttime sea-

surface temperature and information about ice, snow, and clouds. They have also

been used to make measurements related to vegetation and other land surface

parameters (Tucker 1984).

The sensor measures emitted and reflected radiation in five channels (bands) of

the electromagnetic spectrum: 1) A blue (0.58 to 0.68 um) band that used for

daytime cloud and surface mapping; a reflective infrared (0.725 to 1.1 um) band

used for surface water delineation, and vegetation cover mapping; a

mid-infrared (3.55 to 3.93 um) band used for sea surface temperature, and

nighttime cloud  mapping; a thermal infrared (10.5 to 11.5 um) band used for

surface temperature, and day/night cloud mapping; and another thermal

infrared (11.5 to 12.5 um) band used for surface temperature mapping

(Kidwell, 1991).

The AVHRR is capable of operating in both real-time or recorded modes.

Direct readout data were transmitted to ground stations of the automatic

picture transmission (APT) class at low-resolution (4x4 km) and to ground

stations of the high-resolution picture transmission (HRPT) class at high

resolution (1x1 km). Data recorded on board were available for processing

in the NOAA Central Computer Facility. They included local area coverage

(LAC) data which were from selected portions of each orbit with a 1x1 km

resolution.

5.  EQUIPMENT

5.1  Instrument Description.

The AVHRR sensor system used to collect the original data from which this

data set was produced is described in detail in the documentation for

Level-1 Advanced Very High Resolution Radiometer (AVHRR) Images data set.

See that document for more details.

5.1.1  Platform.

See Section 5.1.

5.1.2  Mission Objectives.

See Section 5.1.

5.1.3  Key Parameters.

See Section 5.1.

5.1.4  Principles of Operation.

See Section 5.1.

5.1.5  Instrument Measurement Geometry.

See Section 5.1.

5.1.6  Manufacturer of Instrument.

See Section 5.1.

5.2  Calibration.

See Section 5.1.

5.2.1  Specifications.

See Section 5.1.

5.2.1.1  Tolerance.

See Section 5.1.

5.2.2  Frequency of Calibration.

See Section 5.1.

5.2.3  Other Calibration Information.

See Section 5.1.

6.  PROCEDURE

6.1  Data Acquisition Methods.

The Site Average Reflectances Extracted from AVHRR-LAC Imagery were

produced from the FIFE level-1 AVHRR data by FIFE staff at Goddard

Space Flight Center. The original imagery from the Advanced Very

High Resolution Radiometer instruments on the NOAA-9, NOAA-10,

and NOAA-11 satellites were acquired from the Naval Research

Laboratory (NRL) and the EROS Data Center.

6.2  Spatial Characteristics.

The FIFE site, with areal extent of 15 km by 15 km, is located south

of the Tuttle Reservoir and Kansas River, and about 10 km from

Manhattan, Kansas, USA. The northwest corner of the area has UTM

coordinates of 4,334,000  Northing and 705,000 Easting in UTM Zone 14.

6.2.1  Spatial Coverage.

The extracted data cover the area of the FIFE site, i.e., the area

between 38 degrees 52 minutes 30 seconds and 39 degrees 07 minutes

30 seconds North latitude, and between 96 degrees 22 minutes 30

seconds and 96 degrees 37 minutes 00 seconds West longitude.

6.2.2  Spatial Resolution.

The resolution of these data is approximately 15 km since the site

average was produced using 200-300 pixels scattered throughout

the site.

6.3  Temporal Characteristics.

6.3.1  Temporal Coverage.

NOAA AVHRR-LAC acquisition used for extract products covers the

period from February 3, 1987 through October 13, 1989.

Satellite extracts were computed for 331 days over the range

of data collection. The distribution of extracts by month over

these 33 months is as follows:

Date       Averages               Date       Averages

Month/Year   per Month            Month/Year   per Month

---------    ---------            ----------   ---------

Feb. 1987       17                July 1988        4

March 1987      15                Aug. 1988        3

April 1987      24                Sept. 1988       9

May 1987        15                Oct. 1988        2

June 1987       21                Nov. 1988        0

July 1987       26                Dec. 1988        0

Aug. 1987       23                Jan. 1989        0

Sept. 1987      24                Feb. 1989        0

Oct. 1987       25                Mar. 1989        0

Nov. 1987       17                Apr. 1989        0

Dec. 1987       17                May 1989         0

Jan. 1988       16                June 1989        4

Feb. 1988        4                July 1989       16

Mar. 1988        6                Aug. 1989       10

Apr. 1988        4                Sept. 1989      11

May 1988         5                Oct. 1989        5

June 1988        8

6.3.2  Temporal Resolution.

The average interval between images used to produce the extract

data was roughly 12 hours. There are generally two overpasses

per day per satellite at approximate times of 0200, 0900, 1400,

and 2100 GMT. However, not all of these data were processed due

to cloud cover and other limitations of the data.

7.  OBSERVATIONS

7.1  Field Notes.

None.

8.  DATA DESCRIPTION

8.1  Table Definition With Comments.

The SQL definition for this table is found in the SAT_AVHR.TDF file

found on FIFE CD-ROM Volume 1.

8.2  Type of Data.

--------------------------------------------------------------------------------

|                 8.2.1                  |               |          |          |

|Parameter/Variable Name                 |               |          |          |

--------------------------------------------------------------------------------

|    |               8.2.2               |     8.2.3     |  8.2.4   |  8.2.5   |

|    |Parameter/Variable Description     |Range          |Units     |Source    |

--------------------------------------------------------------------------------

|SITEGRID_ID                             |               |          |          |

|    |This is a FIS grid location code.  |               |          |          |

|    |Site grid codes (SSEE-III) give    |               |          |          |

|    |the south (SS) and the east (EE)   |               |          |          |

|    |cell number in a 100x100 array of  |               |          |          |

|    |200m square cells.  The last 3     |               |          |          |

|    |characters (III) are an instrument |               |          |          |

--------------------------------------------------------------------------------

|STATION_ID                              |               |          |          |

|    |The station ID designating the     |               |          |          |

|    |location of the observations.      |               |          |          |

--------------------------------------------------------------------------------

|OBS_DATE                                |               |          |          |

|    |The date (expressed as             |               |          |          |

|    |DD-MMM-YY), on which the image     |               |          |          |

|    |data was recorded.                 |               |          |          |

--------------------------------------------------------------------------------

|OBS_TIME                                |               |          |          |

|    |The time (GMT) when the data at    |               |          |          |

|    |the center of the level-1 image    |               |          |          |

|    |were collected.                    |               |          |          |

--------------------------------------------------------------------------------

|IMAGE_ID                                |               |          |          |

|    |The FIS image identification code  |               |          |          |

|    |for the level-1 satellite image    |               |          |          |

|    |from which the site statistics     |               |          |          |

|    |were derived.                      |               |          |          |

--------------------------------------------------------------------------------

|PLATFORM                                |               |          |          |

|    |The satellite platform on which    |               |          |          |

|    |the data collecting instrument is  |               |          |          |

|    |mounted.                           |               |          |          |

--------------------------------------------------------------------------------

|INSTR_ID                                |               |          |          |

|    |The instrument which collected     |               |          |          |

|    |the image data.                    |               |          |          |

--------------------------------------------------------------------------------

|NUM_OBS                                 |               |          |          |

|    |The number of observations         |               |          |          |

|    |(pixels) found within the site     |               |          |          |

|    |coordinate boundaries and used in  |               |          |          |

|    |the statistics calculations.       |               |          |          |

--------------------------------------------------------------------------------

|MIN_LAT                                 |               |          |          |

|    |The minimum latitude of all the    |               |[Degrees, |          |

|    |pixels extracted from the level-1  |               | Minutes, |          |

|    |image and used to derive the site  |               | Seconds] |          |

|    |statistics (expressed as DD MM     |               |          |          |

|    |SS.SS).                            |               |          |          |

--------------------------------------------------------------------------------

|MAX_LAT                                 |               |          |          |

|    |The maximum latitude of all the    |               |[Degrees, |          |

|    |pixels extracted from the level-1  |               | Minutes, |          |

|    |image and used to derive the site  |               | Seconds] |          |

|    |statistics (expressed as DD MM     |               |          |          |

|    |SS.SS).                            |               |          |          |

--------------------------------------------------------------------------------

|MIN_LON                                 |               |          |          |

|    |The minimum longitude of all the   |               |[Degrees, |          |

|    |pixels extracted from the level-1  |               | Minutes, |          |

|    |image and used to derive the site  |               | Seconds] |          |

|    |statistics (expressed as DDD MM    |               |          |          |

|    |SS.SS).                            |               |          |          |

--------------------------------------------------------------------------------

|MAX_LON                                 |               |          |          |

|    |The maximum longitude of all the   |               |[Degrees, |          |

|    |pixels extracted from the level-1  |               | Minutes, |          |

|    |image and used to derive the site  |               | Seconds] |          |

|    |statistics (expressed as DDD MM    |               |          |          |

|    |SS.SS).                            |               |          |          |

--------------------------------------------------------------------------------

|VIEW_ZEN_ANG                            |               |          |          |

|    |The view zenith at the center of   |               |[degrees] |          |

|    |the site.                          |               |          |          |

--------------------------------------------------------------------------------

|VIEW_AZIM_ANG                           |               |          |          |

|    |The view azimuth at the center of  |               |[degrees f|          |

|    |the site (North = 0, East =        |               |rom North]|          |

|    |90, South = 180, West = 270).      |               |          |          |

--------------------------------------------------------------------------------

|SOLAR_ZEN_ANG                           |               |          |          |

|    |The solar zenith at the center of  |               |[degrees] |          |

|    |the site.                          |               |          |          |

--------------------------------------------------------------------------------

|SOLAR_AZIM_ANG                          |               |          |          |

|    |The solar azimuth at the center    |               |[degrees f|          |

|    |of the site (North = 0, East       |               |rom North]|          |

|    |= 90, South = 180, West = 270).    |               |          |          |

--------------------------------------------------------------------------------

|BAND1_AVG_RADNC                         |               |          |          |

|    |The average radiance over the      |               |[Watts][me|          |

|    |site for band 1 of the designated  |               |ter^-2]   |          |

|    |sensor.                            |               |          |          |

--------------------------------------------------------------------------------

|BAND1_SDEV_RADNC                        |               |          |          |

|    |The standard deviation of the      |               |[Watts][me|          |

|    |radiance values over the site for  |               |ter^-2]   |          |

|    |band 1 of the designated sensor.   |               |          |          |

--------------------------------------------------------------------------------

|BAND2_AVG_RADNC                         |               |          |          |

|    |The average radiance over the      |               |[Watts][me|          |

|    |site for band 2 of the designated  |               |ter^-2]   |          |

|    |sensor.                            |               |          |          |

--------------------------------------------------------------------------------

|BAND2_SDEV_RADNC                        |               |          |          |

|    |The standard deviation of the      |               |[Watts][me|          |

|    |radiance values over the site for  |               |ter^-2]   |          |

|    |band 2 of the designated sensor.   |               |          |          |

--------------------------------------------------------------------------------

|BAND3_AVG_RADNC                         |               |          |          |

|    |The average radiance over the      |               |[Watts][me|          |

|    |site for band 3 of the designated  |               |ter^-2]   |          |

|    |sensor.                            |               |          |          |

--------------------------------------------------------------------------------

|BAND3_SDEV_RADNC                        |               |          |          |

|    |The standard deviation of the      |               |[Watts][me|          |

|    |radiance values over the site for  |               |ter^-2]   |          |

|    |band 3 of the designated sensor.   |               |          |          |

--------------------------------------------------------------------------------

|BAND4_AVG_RADNC                         |               |          |          |

|    |The average radiance over the      |               |[Watts][me|          |

|    |site for band 4 of the designated  |               |ter^-2]   |          |

|    |sensor.                            |               |          |          |

--------------------------------------------------------------------------------

|BAND4_SDEV_RADNC                        |               |          |          |

|    |The standard deviation of the      |               |[Watts][me|          |

|    |radiance values over the site for  |               |ter^-2]   |          |

|    |band 4 of the designated sensor.   |               |          |          |

--------------------------------------------------------------------------------

|BAND5_AVG_RADNC                         |               |          |          |

|    |The average radiance over the      |               |[Watts][me|          |

|    |site for band 5 of the designated  |               |ter^-2]   |          |

|    |sensor.                            |               |          |          |

--------------------------------------------------------------------------------

|BAND5_SDEV_RADNC                        |               |          |          |

|    |The standard deviation of the      |               |[Watts][me|          |

|    |radiance values over the site for  |               |ter^-2]   |          |

|    |band 5 of the designated sensor.   |               |          |          |

--------------------------------------------------------------------------------

|BAND1_AVG_REFL                          |               |          |          |

|    |The average reflectance over the   |               |[percent] |          |

|    |site for band 1 of the designated  |               |          |          |

|    |sensor.                            |               |          |          |

--------------------------------------------------------------------------------

|BAND2_AVG_REFL                          |               |          |          |

|    |The average reflectance over the   |               |[percent] |          |

|    |site for band 2 of the designated  |               |          |          |

|    |sensor.                            |               |          |          |

--------------------------------------------------------------------------------

|BAND1_EXOATMOSIC_REFL                   |               |          |          |

|    |The at-satellite reflectance for   |               |[percent] |          |

|    |band 1 of the sensor, calculated   |               |          |          |

|    |as observed reflected radiance     |               |          |          |

|    |divided by the solar irradiance at |               |          |          |

|    |the top of the atmosphere.         |               |          |          |

--------------------------------------------------------------------------------

|BAND2_EXOATMOSIC_REFL                   |               |          |          |

|    |The at-satellite reflectance for   |               |[percent] |          |

|    |band 2 of the sensor, calculated   |               |          |          |

|    |as observed reflected radiance     |               |          |          |

|    |divided by the solar irradiance at |               |          |          |

|    |the top of the atmosphere.         |               |          |          |

--------------------------------------------------------------------------------

|FIFE_DATA_CRTFCN_CODE                   |               |          |          |

|    |The FIFE Certification Code for    |       *       |          |          |

|    |the data, in the following format: |               |          |          |

|    |CPI (Certified by PI), CPI-???     |               |          |          |

|    |(CPI - questionable data).         |               |          |          |

--------------------------------------------------------------------------------

|LAST_REVISION_DATE                      |               |          |          |

|    |The last revision date for the     |               |          |          |

|    |data, in the format (DD-mmm-YY).   |               |          |          |

--------------------------------------------------------------------------------

Footnote:

* Valid levels

The primary certification codes are:

EXM - Example or Test data (not for release).

PRE - Preliminary (unchecked, use at your own risk).

CPI - Checked by Principal Investigator (reviewed for quality).

CGR - Checked by a group and reconciled (data comparisons and

cross-checks).

The certification code modifiers are:

PRE-NFP  - Preliminary - Not for publication, at the request of

investigator.

CPI-MRG  - PAMS data that are "merged" from two separate receiving

stations to eliminate transmission errors.

CPI-???  - Investigator thinks data item may be questionable.

8.3  Sample Data Base Data Record.

SITEGRID_ID STATION_ID OBS_DATE  OBS_TIME IMAGE_ID         PLATFORM

----------- ---------- --------- -------- ---------------- ----------

FIFE-LAC            99 03-FEB-87     1401 LAC1001975N-1    NOAA-10

FIFE-LAC            99 07-FEB-87     1414 LAC1002032N-1    NOAA-10

FIFE-LAC            99 08-FEB-87      135 LAC1002039N-1    NOAA-10

FIFE-LAC            99 10-FEB-87     1449 LAC1002075N-1    NOAA-10

INSTR_ID     NUM_OBS MIN_LAT      MAX_LAT      MIN_LON

------------ ------- ------------ ------------ --------------

AVHRR-LAC        475  38 52 30.58  39 07 28.58   -96 37 28.31

AVHRR-LAC        615  38 52 30.94  39 07 29.66   -96 37 29.66

AVHRR-LAC        687  38 52 31.81  39 07 28.09   -96 37 29.88

AVHRR-LAC        499  38 52 31.22  39 07 29.42   -96 37 24.47

MAX_LON        VIEW_ZEN_ANG VIEW_AZIM_ANG SOLAR_ZEN_ANG SOLAR_AZIM_ANG

-------------- ------------ ------------- ------------- --------------

-96 22 30.72         35.8          97.5          85.4          115.7

-96 22 30.44         21.0          99.2          82.3          117.0

-96 22 34.59          7.7         259.5         110.1          266.3

-96 22 30.53         33.3         284.8          75.8          122.4

BAND1_AVG_RADNC BAND1_SDEV_RADNC BAND2_AVG_RADNC BAND2_SDEV_RADNC

--------------- ---------------- --------------- ----------------

6.706            .4231           4.097            .4460

9.293            .7923           6.578            .6955

-1.407            .1872          -1.171            .0232

17.703           1.8272          12.248           1.3916

BAND3_AVG_RADNC BAND3_SDEV_RADNC BAND4_AVG_RADNC BAND4_SDEV_RADNC

--------------- ---------------- --------------- ----------------

.111            .0043           5.563            .0528

.129            .0047           6.117            .0518

.146            .0099           6.496            .1248

.164            .0056           6.252            .0989

BAND5_AVG_RADNC BAND5_SDEV_RADNC BAND1_AVG_REFL BAND2_AVG_REFL

--------------- ---------------- -------------- --------------

5.563            .0515

6.113            .0438

6.495            .1251

6.245            .0971            3.1           22.7

BAND1_EXOATMOSIC_REFL BAND2_EXOATMOSIC_REFL FIFE_DATA_CERTFN_CODE

--------------------- --------------------- ---------------------

CPI

CPI

CPI

13.3                  14.7 CPI

LAST_REVISION_DATE

------------------

31-JUL-90

31-JUL-90

31-JUL-90

25-APR-91

8.4  Data Format.

The CD-ROM file format consists of numerical and character fields of

varying length separated by commas. The character fields are enclosed with

a single apostrophe. There are no spaces between the fields.  Each file

begins with five header records.  Header records contain the following

information:

Record 1    Name of this file, its table name, number of records in

this file, path and name of the document that describes

the data in this file, and name of principal investigator

for these data.

Record 2    Path and filename of the previous data set, and path and

filename of the next data set. (Path and filenames for files

that contain another set of data taken at the same site on

the same day.)

Record 3    Path and filename of the previous site, and path and filename

of the next site. (Path and filenames for files of the same

data set taken on the same day for the previous and next

sites (sequentially numbered by SITEGRID_ID)).

Record 4    Path and filename of the previous date, and path and filename

of the next date. (Path and filenames for files of the same

data set taken at the same site for the previous and next

date.)

Record 5    Column names for the data within the file, delimited by

commas.

Record 6    Data records begin.

Each field represents one of the attributes listed in the chart in

section 8.2 and described in detail in the TDF file (see section 8.1).

These fields are in the same order as in the chart.

8.5  Related Data Sets.

Level-1 Advanced Very High Resolution Radiometer (AVHRR) Images.

Site Reflectances Extracted from Landsat at TM Imagery.

Site Reflectances Extracted from SPOT HRV Imagery.

Satellite Image Value Conversion Coefficients.

Daily and Site Radiation Flux Averages Derived from GOES.

9.  DATA MANIPULATIONS

9.1  Formulas.

9.1.1  Derivation Techniques/Algorithms.

The procedure used to calculate AVHRR atmospherically corrected

reflectance was developed at GSFC as part of the FIFE staff science

effort. The data were calculated from FIFE level-1 data and the

reflectance and atmospheric correction calculations were performed

using data and information available in the FIFE data collection.

The radiance values for bands 1 and 2 (RAD1, RAD2) were calculated

using the pre-launch gains and offsets given below. The radiance

values for bands 3, 4, and 5 were derived based upon the

information contained in the NOAA Polar Orbiter User's Guide

(Kidwell 1990) and Data Extraction and Calibration of TIROS-N/NOAA

Radiometers documents.  The conversion from digital counts to

radiance is as follows:

I) Derive at-satellite radiance:

The radiance values for the respective bands were calculated as

follows:

Radiance = (DN(I) - OFFSET(I))/GAIN(I)

where:

Radiance = spectral radiance for channel I

[Watt][m^-2][sr^-1][um^-1] for I = 1,2;

[mWatt][cm][m^-2][sr^-1] for I = 3,4,5;

DN(I) = digital number for channel I [counts]

OFFSET(I) = sensor offset for channel I [counts]

GAIN(I) = sensor gain for channel I

[counts][radiance unit^-1]

Note: FIFE terminology for GAIN and OFFSET is reversed from

NOAA's :

NOAAGAIN(I) = 1.0/GAIN(I)

NOAAOFFSET(I) = -(OFFSET(I)/GAIN(I))

A) GAIN, OFFSET Determination.

1) Reflective Channels (1,2)

Use pre-launch gains (GSFC calculated)

NOAA-9  : GAIN(1) = 1.908;  GAIN(2) = 3.040

NOAA-10 : GAIN(1) = 1.957;  GAIN(2) = 2.899

NOAA-11 : GAIN(1) = 1.912;  GAIN(2) = 3.178

Radiances calculated using these pre-launch gains can be

updated using a multiplication factor for observed

degradation.

Use SPACE VIEWS as offsets

OFFSET(I) = average space view of the channel from FIFE

Level-1 ASCII image header file

2) Thermal Channels (3,4,5)

Calculate gains from FIFE Level-1 ASCII image header file

information (negative radiance of space not used)

Let,

K1 = 1.1910659E-05 [mWatt][cm][m^-2][sr^-1]

K2 = 1.438833 [cm][deg K]

WAVE CENTER (VC) TABLE (cm-1)

----------------------------------

NOAA-9                   NOAA-10

Temp.

Deg (K)  BAND 3   BAND 4  BAND 5  BAND 3   BAND 4  BAND 5

-------  -------  ------  ------  -------  ------  ------

180-225  2670.93  928.50  844.41  2658.53  908.73  909.73

225-270  2674.81  929.02  844.80  2657.60  909.18  909.18

270-310  2677.67  929.39  845.12  2660.35  909.52  909.52

310-320  2678.11  929.46  845.19  2660.76  909.58  909.58

NOAA-11

Temp.

Deg (K) BAND 3   BAND 4  BAND 5

-------- -------  ------  ------

180-225  2663.50  926.81  841.40

225-270  2668.15  927.36  841.81

270-310  2670.96  927.75  842.14

310-320  2671.40  927.83  842.20

Based on the platform, set VC to the values for

the 270 - 310 temp range

Then,

ENUM = (K1*VC(I+2)) * VC(I+2) * VC(I+2)

DENOM = EXP[K2*VC(I+2)/BB_TEMP] - 1

BB_RAD = ENUM/DENOM

GAIN(I+2) = (BB_VIEW(I+2)-OFFSET(I+2))/BB_RAD

Where,

BB_TEMP is the average black body temp (K) given in

the FIFE level-1 ASCII image header file.

BB_VIEW(I+2) is the respective average black body

view (DN) of the channel obtained from the FIFE

level-1 ASCII image header file.

OFFSET(I+2) is the average space view (DN) of the

channel obtained from the FIFE level-1 ASCII

image header file.

I = 1,2,3 (I+2 designates the respective channels)

B) Non-linearity correction for bands 4 and 5:

The basic equation provides the final radiances (except for

unit conversions) for channels 1,2,3.  Channels 4 and 5

require a non-linearity correction which depends on the

instrument temperature.

1) Calculate average scene temperatures for bands 4 and 5

using the assumed wave centers.

ENUM = K2 * VC(I+3)

DENOM = (K1 * VC(I+3)) * VC(I+3) * VC(I+3)

ARG = ALOG( DENOM/PIX_RAD(I+3) + 1.0 )

TEMP(I+3) =  ENUM/ARG

Where,

I = 1,2 (I+3 designates the respective channels)

PIX_RAD(I+3) is the band radiance for a given pixel

TEMP(I+3) is the temperature for the given pixel

2) Reset the wave centers (VC) for bands 4,5 based on the

respective average scene temperatures.

Note: The temperature to radiance conversion is a fitted

equation with the parameter VC fit for different

temperature ranges.

3) Convert Band 4 and 5 radiances to temperatures [deg K] with

the updated wave centers.

ENUM = K2 * VC(I+3)

DENOM = (K1 * VC(I+3)) * VC(I+3) * VC(I+3)

ARG = ALOG( DENOM/PIX_RAD(I+3) + 1.0 )

TEMP(I+3) =  ENUM/ARG

where,

I = 1, 2 (I+3 designates the respective

channels)

PIX_RAD(I+3) = the band radiance for a given pixel

TEMP(I+3) = the temperature for the given pixel

4) Perform linear interpolation between tabulated temperature

correction sets to derive needed correction set for black

body temperature obtained from FIFE level-1 ASCII image

header file. The result is a column of non-linearity

corrections for the observed BB_TEMP.

NOAA-9                          NOAA-10

------                          -------

BAND 4         BAND 5              BANDS 4 & 5

-----------------------------------------------------------

BB_TEMP (K)       BB_TEMP (K)        BB_TEMP (K)

PIXEL

TEMP   283   288   293    283   288   293    283   288   293

-----  ---   ---   ---    ---   ---   ---    ---   ---   ---

320    2.3   2.3   2.3    0.8   1.0   1.2    2.1   1.6   1.5

315    1.8   1.9   1.8    0.6   0.9   0.9    1.8   1.3   1.2

310    1.6   1.4   1.3    0.8   0.7   0.7    1.5   1.0   0.9

305    1.3   1.0   0.9    0.7*  0.4   0.5    1.1   0.7   0.6

295    0.7   0.4   0.2    0.4   0.2   0.1    0.5   0.2   0.1

285    0.0  -0.2  -0.5    0.0  -0.1  -0.2   -0.1  -0.3  -0.3

275   -0.5  -0.7  -0.9   -0.3  -0.3  -0.5   -0.2  -0.5  -0.6

265   -0.8  -1.1  -1.2   -0.5  -0.6  -0.7   -0.5  -0.9  -0.9

255   -1.0  -1.3  -1.6   -0.7  -0.8  -1.0   -0.9  -1.1  -1.2

245   -1.1  -1.3  -1.7   -0.8  -0.8  -1.2   -1.3  -1.4  -1.4

235   -1.2  -1.4  -1.6   -1.1  -1.2  -1.2   -1.5  -1.5  -1.6

225   -1.3  -1.3  -1.5   -1.2  -1.0  -1.1   -1.7  -1.7  -1.8

215   -1.2  -1.5  -1.4   -1.2  -1.4  -1.4   -2.0  -1.9  -1.8

205   -1.6  -1.5  -0.7   -1.7  -1.6  -1.1   -2.3  -2.1  -1.8

-------------

Footnote:

* On October 27, 1990 a typographical error was

reported in the values of the above table. For NOAA-9

band 5 at a target temperature of 305 K and a

blackbody temperature of 283 K, the value should be

0.7, not 1.1 as previously reported. the old value of

1.1 was replaced with the new value of 0.7 in the

table above. However, the extracted FIFE AVHRR site

data were processed using the old value of 1.1. The

error resulting from this change has not been

assessed.

NOAA-11

-------

BAND 4                BAND 5

--------------------------------------------------

BB_TEMP (K)           BB_TEMP (K)

PIXEL

TEMP     283    288    293     283    288    293

-----   -----  -----  -----   -----  -----  -----

320     4.29   3.71   3.25    1.43   1.26   1.12

315     3.50   2.98   2.55    1.23   1.03   0.89

310     2.85   2.33   1.91    1.05   0.84   0.70

305     2.23   1.73   1.32    0.85   0.64   0.47

295     1.05   0.68   0.22    0.43   0.28   0.09

285     0.24  -0.21  -0.67    0.07  -0.07  -0.23

275    -0.45  -0.79  -1.15   -0.19  -0.34  -0.47

265    -1.06  -1.37  -1.66   -0.37  -0.51  -0.60

255    -1.41  -1.72  -2.03   -0.60  -0.77  -0.78

245    -1.70  -1.96  -2.22   -0.72  -0.90  -0.92

235    -1.87  -2.10  -2.28   -0.84  -1.02  -1.00

225    -1.90  -2.14  -2.36   -0.94  -1.06  -1.16

215    -1.82  -2.02  -2.20   -1.12  -1.24  -1.16

205    -1.54  -1.76  -1.98   -1.15  -1.27  -1.23

5) Perform linear interpolation between derived temperature

correction set values (from above) to get pixel specific

correction value based on calculated pixel temperature

and apply (add) the temperature corrections to bands 4

and 5 of each pixel.

6) Convert the corrected temperatures back to radiance.

ENUM = (K1*VC(I+3)) * VC(I+3) * VC(I+3)

DENOM = EXP[K2*VC(I+3)/PIXEL_TEMP(I+3)] - 1

PIXEL_RAD(I+3) = ENUM/DENOM

Where,

I = 1, 2 (I+3 designates the respective

channels)

PIX_RAD(I+3) = the band radiance for a given pixel

TEMP(I+3) = the temperature for the given pixel

C) Unit conversion:

Convert the radiances from [mWatt][cm][m^-2][sr^-1] to

[Watt][m^-2][sr^-1][um^-1] for bands 3,4,5.

Let,

FACTOR = 1.383, 11.600, 14.032 FOR NOAA-9

= 1.401, 12.111, 12.111 FOR NOAA-10

= 1.392, 11.647, 14.131 FOR NOAA-11

Then,

PIX_RAD(I) = PIX_RAD(I)/FACTOR(I-2)

Where,

I = 3,4,5 for the respective bands

The final radiances are averaged over the FIFE site and the

mean and standard deviation for each channel are reported in

the data base.

II) Atmospheric correction and reflectance calculations:

The radiance data (calculated as described above) were

converted first to exoatmospheric reflectance, corrected

for atmospheric attenuation, and then converted to surface

reflectance. The conversion to exoatmospheric reflectance

normalizes the data for solar zenith angle and Earth-Sun

distance variation, and generates reflectance values expected

outside of the atmosphere (exoatmospheric reflectance). The

method of Markham and Barker (1986) was used.

A) Convert to exoatmospheric reflectance (L'm) for each band:

L'm = (PI * Lm) / (F'o * cos(szen)

where (an apostrophe indicates a subscript):

Lm = apparent at-satellite calibrated spectral

radiance [Watt][m^-2][sr^-1][um^-1]

F'o = Fo / (R^2)

R = d/d'

d = Earth-Sun distance (astronomical units)

d' = average Earth-Sun distance =1 astronomical unit

szen = solar zenith angle (degrees)

Fo = extraterrestrial solar irradiance in the

AVHRR-LAC bandpass as listed below:

Exoatmospheric Reflectance

([Watt][m^-2][um^-1])

-----------------------------------

Band  NOAA-9  NOAA-10  NOAA-11

----  ------  -------  -------

1   1631.0   1660.5   1633.5

2   1046.0   1037.0   1046.0

B) Optical thickness were assumed constant across the few

degrees of the scan view angle that covered the area

selected as the FIFE site (see the FIFE AVHRR level-1

documentation for view angle determination). Sunphotometer

measurements of the atmospheric properties over the FIFE

site were not available for most dates on which

acquisitions were acquired. Instead of generating a

data set of reflectances based on inconsistent

atmospheric property measurements (i.e., some measured

and some climatological), a set of default optical

thickness values were generated and used to correct

all scenes. Aerosol optical thicknesses were determined

from an average of mid-afternoon sunphotometer

measurements collected throughout the year in 1988

over the FIFE site. Optical thicknesses due to water

vapor and other absorbing gases were calculated from

the standard mid-latitude summer atmospheric model of

the Lowtran-7 radiative transfer algorithm (Kneizys,

et al., 1988). The following optical properties resulted

and were used to correct all scenes:

Default Atmospheric Optical Thicknesses

(all quantities are dimensionless)

------------------------------------------------

Water       Other

Channel     Aerosols    Vapor       Gases

---------   --------   --------    ----------

Channel-1    0.15      0.0208      0.0247

Channel-2    0.10      0.1156      0.0152

C) The FIFE atmospheric correction algorithm (Fraser et. al.,

1989) was then run using the solar angles, viewing

angles, and aerosol optical property values described

above. Coefficients generated from the correction process

were tabulated and formatted as a look-up table. The

coefficients were also entered into the satellite

coefficients data. The coefficients are used to calculate

surface reflectances using the method described below.

A program was developed to search the look-up table and

match the input image by date, extract the coefficients,

and calculate surface reflectance (rho) on a pixel-by-pixel

basis for each image. Surface reflectance was calculated

as:

rho = f / (1 + s * f)

where:

s = backscattering ratio (column BSCAT in FIS)

f = (L'm - L'o) / (F'd * T)

L'm = from (1) above

F'd = normalized surface flux (column IRRAD in FIS)

L'o = normalized path radiance (column PATH_RAD in FIS)

T = path transmission (column TRANS in FIS)

III) Surface temperature calculations:

Spectral radiances (in [mW][cm][m^-2][sr^-1]) from thermal

channels 4 and 5 of the NOAA-9 AVHRR were used to calculate

apparent surface temperature with the split-window technique

of Price (1983). Apparent  at-sensor temperatures (T4, T5) are

first calculated for channels 4 and 5 using the non-linearity

corrections described in the previous section.

Surface temperature (Ts) is then derived as:

Ts = T4 + 3.33 * (T4 - T5)

Temperatures are scaled by 10 to retain 1/10 degree precision.

Divide the values on tape by 10 to get degrees K, and subtract

273 to get degrees Celsius.

To date, no atmospheric correction of the NOAA-10 or NOAA-11

data acquired as part of the FIFE initiative has been

undertaken. This is due, in part, to the lack of cloud-free

data collected from these platforms and to decisions by the

Science Steering Group regarding the utility of these data to

the FIFE science objectives.

9.2  Data Processing Sequence.

9.2.1  Processing Steps and Data Sets.

FIFE staff creates the average instrument corrected spectral

radiance data by :

1) Enumerating the pixel and line coordinates of the pixels that

fall within the latitudes of 38.875 and 39.125 degrees N and

within the longitudes of 96.375 and 96.625 degrees W.

2) Extracting the spectral, geographic, viewing, and solar

information for the enumerated pixels.

3) Converting and correcting the digital spectral counts to

radiance.

4) Storing appropriate information in the data base.

FIFE staff creates the atmospherically corrected spectral reflect-

ance and temperature data by :

1) Converting and correcting the digital spectral counts to

at-sensor radiance.

2) Correcting the at-sensor radiances for atmospheric attenuation

and converting to surface reflectance.

3) Storing the atmospherically corrected mean values by date and

time in the data base.

9.2.2  Processing Changes.

None.

9.3  Calculations.

9.3.1  Special Corrections/Adjustments.

None.

9.4  Graphs and Plots.

None.

10.  ERRORS

10.1  Sources of Error.

Errors could arise in the acquired imagery due to location accuracy,

distortion of lengths, anisomorphism, instrument's local coherence,

multispectral registrability, and relief plotting accuracy. Other

errors could arise from inherent radiometric imperfections of the

sensors. Spectral errors arise due to image wide signal-to-noise

ratio, saturation, cross-talk, spikes, response normalization due to

change in gain.

Whatever the processing level, the geometric quality of the image depends

on, a) the accuracy of the viewing geometry, and b) the ground control

points as required to adjust the viewing model. Errors arise from the

processing of the data to level-1A due to calibration inaccuracies (gains

and offsets/drift of sensor characteristics/optical degradation), and

resampling/positioning in geometric corrections.

On October 27, 1990 a typographical error was reported for NOAA-9 band

5 at a target temperature of 305 K and a blackbody temperature of 283 K,

the value should be 0.7, not 1.1 as previously reported. However, the

extracted FIFE AVHRR site data were processed using the old value of 1.1.

10.2  Quality Assessment.

10.2.1  Data Validation by Source.

The FIFE staff was responsible for screening imagery for quality,

applying radiometric corrections, computing geometric corrections

corresponding to the required map projection, applying geometric

corrections, and screening for cloud cover on imagery during

processing.

10.2.2  Confidence Level/Accuracy Judgment.

The precision of satellite remote sensing estimates of surface

reflectance (Hall et al., 1992), calibrated and corrected for

atmospheric effects, was no worse than about 1 percent absolute.

The errors may actually be smaller, but an upper bound of 1

percent results from sampling variance caused by differences

among the satellite and ground sensors in spatial resolution,

atmospheric effects, and calibration.

10.2.3  Measurement Error for Parameters and Variables.

No quantitative assessment was made, see Section 10.2.2.

Other errors mentioned in Section 10.1 were not assessed.

10.2.4  Additional Quality Assessment Applied.

FIS staff applied a general Quality Assessment (QA) procedure to

the data to identify inconsistencies and problems for potential

users. As a general procedure, the FIS QA consisted of examining

the maximum, minimum, average, and standard deviation for each

numerical field in the data table. An attempt was made to find an

explanation for unexpected high or low values, values outside of

the normal physical range for a variable, or standard deviations

that appeared inconsistent with the mean. In some cases, histog-

rams were examined to determine whether outliers were consistent

with the shape of the data distribution.

The discrepancies, that were identified, are reported as problems

in Section 11.1.

11.  NOTES

11.1  Known Problems With The Data.

As of the revision date of this document, the following discrepancies or

errors in the data have been reported:

o  Although bands 4 and 5 of NOAA-10 images are supposedly duplicates,

average band counts have shown differences of 2 to 3 counts in some

images.  The source of the difference is currently unknown.

o  For NOAA-10, if the mean space view values in the level-1 ASCII

header file differ by more than 1.0 counts from 37.0, all these data

should be treated as questionable.

o  For NOAA-11 data processed prior to June 8, 1990, the EDC used

incorrect central wave numbers and did not account for the nonlinear

response of channels 4 and 5 in their gain calculations for the gains

contained on the level-0 (NOAA 1b) tapes.  The values used by FIS to

derive the level-1a products were correct in that the basic

calibration information was used along with proper wave numbers and

nonlinear terms to perform the calculations.

o  Channel 3 data was not atmospherically corrected due to inherent

noise problems, the inability to accurately characterize the

atmospheric properties in that channel, and the inherent complexity

in interpreting daytime data in this channel.

o  For the data collected from NOAA-9 the following problems were

found in the data for the parameters listed:

Parameter/Variable                Comments

------------------     ----------------------------------

BAND1_AVG_REFL         2 values >  20

BAND1_AVG_RADNC        4 values >  70  (MAX 103)

BAND2_AVG_RADNC        6 values >  70  (MAX 89)

BAND1_EXOATMOSIC_REFL  2 values >  21  (25.2, 28.9)

o  For the data collected from NOAA-10 the following problems were

found in the data for the parameters listed:

Parameter/Variable                Comments

------------------     ----------------------------------

BAND1_AVG_REFL         3 values >  25, 10 values < 0

BAND2_AVG_REFL         3 values >  60  (MAX 103)

BAND1_EXOATMOSIC_REFL  4 values >  20  (MAX 50.1)

BAND2_EXOATMOSIC_REFL  1 value  >  40  (48.5)

11.2  Usage Guidance.

AVHRR data are used in the preparation of a number of products

required for meteorological, climate, and land research, for the NOAA

operational weather forecasting system, and for the international weather

forecast community. Other applications include geographical and ecolog-

ical research. In addition, AVHRR data are used to determine vegetation

cover for large regions. These products include temperature and water

vapor soundings, oceanographic and hydrological products, radiation

budget products, and a variety of mapped and gridded imagery.

11.3  Other Relevant Information.

None.

12.  REFERENCES

12.1  Satellite/Instrument/Data Processing Documentation.

Newcomer, J.A., S.J. Goetz, D.E. Strebel, and F.G. Hall. 1989.

Image processing software for providing radiometric inputs to

land surface climatology models. IGARSS '89. 12th Can. Symp.

on Remote Sensing. pp. 1779-1782.

Fraser, R.S., R.A. Ferrare, Y.J. Kaufman, and S. Mattoo. 1989.

Algorithm for atmospheric corrections of aircraft and satellite

imagery. NASA Technical Memorandum 100751. 106 pp.

Kidwell, K. 1991. NOAA Polar Orbiter Data User's Guide. NCDC/SDSD.

National Climatic Data Center, Washington.

Kneizys, F.X., E.P. Shettle, W.O. Gallery, J.H. Chetwynd, L.W.

Abreu, J.E.A. Selby, S.A. Clough, and R.W. Fenn. 1988.

Atmospheric transmittance/radiance: computer code LOWTRAN-7,

AFGL-TR-88-0177. Air Force Geophysics Lab. Hanscomb AFB,

Massachusetts.

Markham, B.L., and J.L. Baker. 1986. Landsat MSS and TM post-calibration

dynamic ranges, exoatmospheric reflectances and at-satellite

temperatures. EOSAT Landsat Tech. Notes 1:3-7. Lanham, Maryland.

Price, J.C. 1983. Estimating surface temperatures from satellite

thermal infrared data -- a simple formulation for the atmospheric

effect. Remote Sensing Environment. 13:353-361.

12.2  Journal Articles and Study Reports.

Duggin, M.J., D. Piwinshi, V. Whitehead and G. Tyland. 1982. Evaluation

of NOAA-AVHRR Data for Crop Assessment. Appl. Opt. 21:1873-1875.

Justice, C.O., B.N. Holben, and M.D. Gwynne. 1986. Monitoring East

African Vegetation Using AVHRR Data. Int. J. Remote Sens.

7:1453-1474.

Ormsby, J.P. 1982. Classification of Simulated and Actual NOAA-6 AVHRR

Data for Hydrological Land-Surface Feature Definition. IEEE

Transactions on Geoscience and Remote Sensing. GE-20:262-268.

Strebel, D.E., J.A. Newcomer, J.P. Ormsby, F.G. Hall, and P.J. Sellers.

1990. The FIFE information system. IEEE Transactions on Geoscience

and Remote Sensing. 28(4):703-710.

Taconet, O., R. Bernard and D. Vidal-Madja. 1986. Evapotranspiration Over

an Agricultural Region Using a Surface Flux/Temperature Model Based

on NOAA-AVHRR Data. J. Climate Appl. Meteor. 25:284-307.

Tucker, C.J., J.A. Gatlin, and S.P. Sneider. 1984. Monitoring Vegetation

in the Nile Delta with NOAA-6 Imagery. Photogram. Engr. Remote Sens.

50:53-62.

Yates, H.W. and J.D. Tarpley. 1982. The Role of Meteorological Satellites

in Agricultural Remote Sensing."  Proc. of the 8th International

Symposium on Machine Processing of Remotely Sensed Data. Purdue

University, Indiana. pp. 23-32.

12.3  Archive/DBMS Usage Documentation.

The Collected Data of the First ISLSCP Field Experiment is archived at

the EOS Distributed Active Archive Center (DAAC) at Oak Ridge National

Laboratory (ORNL), Oak Ridge, Tennessee (Section 13 below).

Documentation about using the archive and/or online access to the data

at the ORNL DAAC is not available at this revision.

13.  DATA ACCESS

13.1  Contacts for Archive/Data Access Information.

ORNL DAAC User Services

P.O. Box 2008

Mail Stop 6490

Oak Ridge National Laboratory

Oak Ridge, TN 37831-6490

(615) 241-3952

13.2  Archive Identification.

EOSDIS Distributed Active Archive Center

P.O. Box 2008

Mail Stop 6490

Oak Ridge National Laboratory

Oak Ridge, TN 37831-6490

USA

Telephone:       (615) 241-3952

FAX:             (615) 574-4665

Internet Email:   ornldaac@onrl.gov

13.3  Procedures for Obtaining Data.

Users may place requests by letter, telephone, electronic mail, FAX, or

personal visit.

13.4  ORNL DAAC Status/Plans.

FIFE data will be available from the ORNL DAAC.  Please contact the

ORNL DAAC User Services Office for the most current information about

these data.

14.  OUTPUT PRODUCTS AND AVAILABILITY

14.1  Tape Products.

None.

14.2  Film Products.

None.

14.3  Other Products.

The Site Average Reflectances Extracted from AVHRR-LAC Imagery are

available on FIFE CD-ROM Volume 1.

15.  GLOSSARY OF ACRONYMS

AVHRR             Advanced Very High Resolution Radiometer

CD-ROM            Compact Disk-Read Only Memory

DAAC              Distributed Active Archive Center

EOSDIS            Earth Observing System Data and Information System

EROS              Earth Resources Observation System

FIFE              First ISLSCP Field Experiment

FIS               FIFE Information System

HRPT              High Resolution Picture Transmission

IFC               Intensive Field Campaign

ISLSCP            International Satellite Land Surface Climatology Project

LAC               Local Area Coverage

NOAA              National Oceanic and Atmospheric Administration

NRL               Naval Research Laboratory

ORNL              Oak Ridge National Laboratory

UTM               Universal Transverse Mercator