1.  TITLE

1.1  Data Set Identification.

Site Reflectances Extracted from SPOT HRV Imagery.

1.2  FIS Data Base Table Name.

SATELLITE_EXTRACT_SPOT_DATA.

1.3  CD-ROM File Name.

\DATA\SAT_OBS\Yyyyy\ydddFIFE.SPT

Where yyyy are the four digits of the year (e.g., 1987). Note: capital

letters indicate fixed values that appear on the CD-ROM exactly as shown

here, lower case indicates characters (values) that change for each path

and file.

The format used for the filenames is: ydddFIFE.sfx, y is the last digit of

the year (e.g., 7=1987, and 9=1989), and ddd is the day of the year

(061=sixty-first day of the year). The filename extension (.sfx),

identifies the data set content for the file (see Section 8.2) and is

equal to .SPT for this data set.

1.4  Revision Date Of This Document.

April 26, 1994.

This document has been reviewed to eliminate technical and editorial

inaccuracies by the FIFE Information Scientist. Previous versions of this

document have been reviewed by the Principal Investigator, the person who

transmitted the data to FIS, a FIS staff member, or a FIFE scientist

generally familiar with the data. It is believed that the document

accurately describes the data as collected and as archived on the FIFE

CD-ROM series.

2.  INVESTIGATOR(S)

2.1  Investigator(s) Name And Title.

Staff Science.

2.2  Title Of Investigation.

Staff Science Satellite Data Acquisition Program.

2.3  Contacts (For Data Production Information).

__________________________________________

|              |         Contact 1         |

|______________|___________________________|

|2.3.1 Name    |Jeffrey A. Newcomer        |

|2.3.2 Address |Code 923                   |

|              |NASA/Goddard Sp. Fl. Ctr.  |

|      City/St.|Greenbelt, MD.             |

|      Zip Code|20771                      |

|2.3.3 Tel.    |(301) 286-7858             |

|2.3.4 Email   |newcomer@ltp.gsfc.nasa.gov |

|______________|___________________________|

2.4  Requested Form of Acknowledgment.

The Site Reflectance from SPOT HRV Imagery were extracted by the FIFE

Information System staff. The dedicated work of Scott Goetz and Jeff

Newcomer is especially appreciated.

3.  INTRODUCTION

3.1  Objective/Purpose.

The FIFE Staff Science effort covered those activities that were FIFE

community-level activities, or required uniform data collection procedures

across sites and time. These activities included acquiring and processing

data from the High Resolution Visible (HRV) instruments on the SPOT

satellites.

As part of the FIFE staff science data collection effort, the FIFE Inform-

ation System (FIS) extracted site reflectances from the level-1 SPOT

multispectral (XS) and panchromatic (PAN) data.

3.2  Summary of Parameters.

Site specific radiance, exoatmospheric reflectance, and surface

reflectance for the three multispectral (XS) and one panchromatic bands.

3.3  Discussion.

The site reflectances extracted from SPOT HRV data contain the average

instrument corrected spectral radiances (in [Watts][meter^-2][steradian^-

1][micrometer^-1]) for each of the spectral bands (3 in XS and 1 in PAN).

In addition, the associated view angles and solar angles are available

for each of 39 FIFE ground measurement sites. The data set also contains

reflectances and exoatmospheric reflectances for these spectral bands.

These reflectances were derived using the sensor calibrated radiance

values corrected using atmospheric aerosol optical thickness and gaseous

absorption profile measurements, when available. The atmospheric

correction algorithm of Fraser et al. (1989) was used to calculate

reflectance in the visible and infrared channels. The extracts are

available about once every days to 10 two weeks during the growing

seasons of 1987, 1988, and 1989.

4.  THEORY OF MEASUREMENTS

The SPOT (Systeme Pour l'Observation de la Terre) satellite system provides

capability of applying high resolution multispectral imagery to a range of land

use and land cover analyses. The SPOT satellite is equipped with two HRV (High

Resolution Visible) linear array (pushbroom) sensors capable of operating in a

panchromatic (PAN) mode with 10-m resolution, or a three-band multispectral

(XS) mode with 20-m resolution.

Thematic considerations dictated, within technical constraints, the choice of

spectral band position and width in the multispectral mode. Three bands were

selected for the multispectral mode: 1) A green (500 to 590 nm) band centered

at the 550 nm maximum in the chlorophyll reflectance curve. This band is on the

long wavelength side of the broad attenuation minimum  of water, thus giving

access to turbidity assessment and bathymetric evaluation in the first 10 to

20 m of surface water; 2) A red band (610 to 680 nm), similar to the channel on

Landsat Thematic Mapper, provides much information on crop identification, bare

soil, and rocky surfaces. Atmospheric transmittance on a clear day is about 90

percent while water penetration is about 2 m with surface reflectance of 4%

(attenuation coefficient: 5x10E-1 [m^-1]). This band corresponds to a chloro-

phyll absorption, i.e., low vegetation reflectance. 3) The near infrared band

(790 to 890) is one that penetrates best through the atmosphere (Transmittance

is about 95 percent for a clear atmosphere model) and light haze. Vegetation

stands out brightly and water surfaces appear very dark (1% reflectance with a

high attenuation coefficient: 10 to 50 per m). Although silicon spectral

sensitivity extends out to 1100 nm, the band was not extended beyond 900 nm in

order to avoid response modulation by atmospheric water vapor and limit the

smearing effect of electron diffusion within the detectors (Chevrel et al.,

1981).

For the higher ground resolution black-and-white (so called panchromatic) mode,

a broader spectral band was required. In order to retain the capability for

texture analysis in support of the color mode and a high information content

over vegetated areas, the interval 510 to 730 nm was chosen for the broad band.

5.  EQUIPMENT

5.1  Instrument Description.

The High Resolution Visible sensor system used to collect the original

data from which this data set was produced has been described in detail

in the document describing the SPOT High Resolution Visible (HRV)

Averages. See that document for a detailed description of this

sensor system.

5.1.1  Platform.

See Section 5.1.

5.1.2  Mission Objectives.

See Section 5.1.

5.1.3  Key Parameters.

See Section 5.1.

5.1.4  Principles of Operation.

See Section 5.1.

5.1.5  Instrument Measurement Geometry.

See Section 5.1.

5.1.6  Manufacturer of Instrument.

See Section 5.1.

5.2  Calibration.

See Section 5.1.

5.2.1  Specifications.

See Section 5.1.

5.2.1.1  Tolerance.

See Section 5.1.

5.2.2  Frequency of Calibration.

See Section 5.1.

5.2.3  Other Calibration Information.

See Section 5.1.

6.  PROCEDURE

6.1  Data Acquisition Methods.

The SPOT site reflectances were extracted from the FIFE level-1 HRV data

by FIFE staff at Goddard Space Flight Center. Members of the staff

selected  pixels from SPOT HRV Level-1 Images that overlaid specific

stations within the FIFE study area. The original imagery from the

High Resolution Visible instruments on the SPOT satellites were

acquired from the SPOT Image Corporation, Reston, Virginia.

6.2  Spatial Characteristics.

The FIFE study area, with areal extent of 15 km by 15 km, is located south

of the Tuttle Reservoir and Kansas River, and about 10 km from Manhattan,

Kansas, USA. The northwest corner of the area has UTM coordinates of

4,334,000 Northing and 705,000 Easting in UTM Zone 14.

6.2.1  Spatial Coverage.

The data were extracted from the images at 40 different locations

scattered throughout the FIFE study area. The exact locations of

the extracted pixels are given below:

SITEGRID NORTHING EASTING  LATITUDE  LONGITUDE  ELEV  SLOPE ASPECT

-------- -------- -------  --------- ---------- ---- ------ ------

0847-SPT  4332344  714439  39 06 57  -96 31 11  418       1 TOP

1246-SPT  4331625  714200  39 06 34  -96 31 22  410      12 S

1445-SPT  4331160  714090  39 06 19  -96 31 27  400

1478-SPT  4331223  720664  39 06 15  -96 26 53  375       2 N

1563-SPT  4331100  717610  39 06 14  -96 29 01  366      18 W

1916-SPT  4330282  708259  39 05 55  -96 35 30  351       2 N

1935-SPT  4330195  711927  39 05 49  -96 32 58  425      20 N

1942-SPT  4330133  713414  39 05 46  -96 31 56  422       1 TOP

2043-SPT  4329952  713679  39 05 40  -96 31 45  415

2123-SPT  4329866  709506  39 05 41  -96 34 39  405       1 TOP

2133-SPT  4329706  711577  39 05 34  -96 33 13  443       1 TOP

2139-SPT  4329843  712789  39 05 37  -96 32 23  385

2330-SPT  4329314  711066  39 05 22  -96 33 35  424       5 E

2428-SPT  4329265  710635  39 05 20  -96 33 53  415

2516-SPT  4328956  708102  39 05 12  -96 35 38  405

2655-SPT  4328787  716070  39 05 00  -96 30 07  367       4 E

2731-SPT  4328678  711110  39 05 01  -96 33 34  446

2915-SPT  4328167  708028  39 04 47  -96 35 42  415

3021-SPT  4328000  709250  39 04 40  -96 34 52  410      11 NW

3129-SPT  4327822  710820  39 04 33  -96 33 47  431      14 E

3221-SPT  4327682  709112  39 04 30  -96 34 58  410

3317-SPT  4327395  708485  39 04 22  -96 35 24  427      15 W

3409-SPT  4327244  706850  39 04 18  -96 36 32  420      12 E

3414-SPT  4327286  707854  39 04 19  -96 35 51  410

3479-SPT  4327134  720890  39 04 02  -96 26 49  420

3921-SPT  4326116  709185  39 03 39  -96 34 57  415

4139-SPT  4325850  712780  39 03 28  -96 32 27  385       3 W

4268-SPT  4325630  718500  39 03 16  -96 28 30  420       1 TOP

4439-SPT  4325193  712773  39 03 06  -96 32 28  443       2 N

4509-SPT  4324960  706850  39 03 04  -96 36 35  390       3 SE

4609-SPT  4324890  706705  39 03 02  -96 36 41  390

5926-SPT  4322227  710270  39 01 32  -96 34 16  370

6221-SPT  4321583  709247  39 01 12  -96 34 59  410

6340-SPT  4321500  713000  39 01 07  -96 32 23  410       4 SW

6469-SPT  4321189  718752  39 00 51  -96 28 25  440       3 NE

6735-SPT  4320652  712073  39 00 40  -96 33 03  385       1 BOTTOM

6833-SPT  4320346  711660  39 00 30  -96 33 20  410

6912-SPT  4320111  707336  39 00 26  -96 36 20  397       2 N

6943-SPT  4320147  713500  39 00 22  -96 32 04  415

8739-SPT  4316699  712845  38 58 31  -96 32 35  442       1 TOP

6.2.2  Spatial Resolution.

The multispectral data have a nadir IFOV of 20 m. The panchromatic

data have a nadir IFOV of 10 m.

6.3  Temporal Characteristics.

6.3.1  Temporal Coverage.

FIFE SPOT HRV image acquisition used for extract products covers

the period from March 20, 1987 through August 9, 1989. There are

no data for December 1987 - March 1988, and from November 1988 -

May 1989.

6.3.2  Temporal Resolution.

The SPOT satellite orbit and pointable sensors make it possible to

acquire images as little as one day apart. During the FIFE IFCs, an

attempt was made to obtain as many cloud free images as possible.

In addition, monitoring data were obtained at least monthly through

the growing season of all 3 years of FIFE. Extracts are available

about once every 10 days to two weeks during the overall coverage

given above.

7.  OBSERVATIONS

7.1  Field Notes.

None.

8.  DATA DESCRIPTION

8.1  Table Definition With Comments.

The SQL definition for this table is found in the SAT_SPOT.TDF file

found on FIFE CD-ROM Volume 1.

8.2  Type of Data.

--------------------------------------------------------------------------------

|                 8.2.1                  |               |          |          |

|Parameter/Variable Name                 |               |          |          |

--------------------------------------------------------------------------------

|    |               8.2.2               |     8.2.3     |  8.2.4   |  8.2.5   |

|    |Parameter/Variable Description     |Range          |Units     |Source    |

--------------------------------------------------------------------------------

|SITEGRID_ID                             |               |          |          |

|    |This is a FIS grid location code.  |               |          |          |

|    |Site grid codes (SSEE-III) give    |               |          |          |

|    |the south (SS) and the east (EE)   |               |          |          |

|    |cell number in a 100x100 array of  |               |          |          |

|    |200m square cells.  The last 3     |               |          |          |

|    |characters (III) are an instrument |               |          |          |

--------------------------------------------------------------------------------

|STATION_ID                              |               |          |          |

|    |The station ID designating the     |               |          |          |

|    |location of the observations.      |               |          |          |

--------------------------------------------------------------------------------

|OBS_DATE                                |               |          |          |

|    |The date (expressed as             |               |          |          |

|    |DD-MMM-YY), on which the image     |               |          |          |

|    |data was recorded.                 |               |          |          |

--------------------------------------------------------------------------------

|OBS_TIME                                |               |          |          |

|    |The time (GMT) when the data at    |               |          |          |

|    |the center of the level-1 image    |               |          |          |

|    |were collected.                    |               |          |          |

--------------------------------------------------------------------------------

|IMAGE_ID                                |               |          |          |

|    |The FIS image identification code  |               |          |          |

|    |for the level-1 satellite image    |               |          |          |

|    |from which the site statistics     |               |          |          |

|    |were derived.                      |               |          |          |

--------------------------------------------------------------------------------

|PLATFORM                                |               |          |          |

|    |The satellite platform on which    |               |          |          |

|    |the data collecting instrument is  |               |          |          |

|    |mounted.                           |               |          |          |

--------------------------------------------------------------------------------

|INSTR_ID                                |               |          |          |

|    |The instrument which collected     |               |          |          |

|    |the image data.                    |               |          |          |

--------------------------------------------------------------------------------

|NUM_OBS                                 |               |          |          |

|    |The number of observations         |               |          |          |

|    |(pixels) found within the site     |               |          |          |

|    |coordinate boundaries and used in  |               |          |          |

|    |the statistics calculations.       |               |          |          |

--------------------------------------------------------------------------------

|MIN_LAT                                 |               |          |          |

|    |The minimum latitude of all the    |               |          |          |

|    |pixels extracted from the level-1  |               |          |          |

|    |image and used to derive the site  |               |          |          |

|    |statistics (expressed as DD MM     |               |          |          |

|    |SS.SS).                            |               |          |          |

--------------------------------------------------------------------------------

|MAX_LAT                                 |               |          |          |

|    |The maximum latitude of all the    |               |          |          |

|    |pixels extracted from the level-1  |               |          |          |

|    |image and used to derive the site  |               |          |          |

|    |statistics (expressed as DD MM     |               |          |          |

|    |SS.SS).                            |               |          |          |

--------------------------------------------------------------------------------

|MIN_LON                                 |               |          |          |

|    |The minimum longitude of all the   |               |          |          |

|    |pixels extracted from the level-1  |               |          |          |

|    |image and used to derive the site  |               |          |          |

|    |statistics (expressed as DDD MM    |               |          |          |

|    |SS.SS).                            |               |          |          |

--------------------------------------------------------------------------------

|MAX_LON                                 |               |          |          |

|    |The maximum longitude of all the   |               |          |          |

|    |pixels extracted from the level-1  |               |          |          |

|    |image and used to derive the site  |               |          |          |

|    |statistics (expressed as DDD MM    |               |          |          |

|    |SS.SS).                            |               |          |          |

--------------------------------------------------------------------------------

|VIEW_ZEN_ANG                            |               |          |          |

|    |The view zenith at the center of   |               |[degrees] |          |

|    |the site.                          |               |          |          |

--------------------------------------------------------------------------------

|VIEW_AZIM_ANG                           |               |          |          |

|    |The view azimuth at the center of  |               |[degrees f|          |

|    |the site (North = 0, East =        |               |rom North]|          |

|    |90, South = 180, West = 270).      |               |          |          |

--------------------------------------------------------------------------------

|SOLAR_ZEN_ANG                           |               |          |          |

|    |The solar zenith at the center of  |               |[degrees] |          |

|    |the site.                          |               |          |          |

--------------------------------------------------------------------------------

|SOLAR_AZIM_ANG                          |               |          |          |

|    |The solar azimuth at the center    |               |[degrees f|          |

|    |of the site (North = 0, East       |               |rom North]|          |

|    |= 90, South = 180, West = 270).    |               |          |          |

--------------------------------------------------------------------------------

|BAND1_AVG_RADNC                         |      $$       |          |          |

|    |The average radiance over the      |               |[Watts][me|          |

|    |site for band 1 of the designated  |               |ter^-2]   |          |

|    |sensor.                            |               |          |          |

--------------------------------------------------------------------------------

|BAND1_SDEV_RADNC                        |    *, $$      |          |          |

|    |The standard deviation of the      |               |[Watts][me|          |

|    |radiance values over the site for  |               |ter^-2]   |          |

|    |band 1 of the designated sensor.   |               |          |          |

--------------------------------------------------------------------------------

|BAND2_AVG_RADNC                         |      $$       |          |          |

|    |The average radiance over the      |               |[Watts][me|          |

|    |site for band 2 of the designated  |               |ter^-2]   |          |

|    |sensor.                            |               |          |          |

--------------------------------------------------------------------------------

|BAND2_SDEV_RADNC                        |     *, $$     |          |          |

|    |The standard deviation of the      |               |[Watts][me|          |

|    |radiance values over the site for  |               |ter^-2]   |          |

|    |band 2 of the designated sensor.   |               |          |          |

--------------------------------------------------------------------------------

|BAND3_AVG_RADNC                         |      $$       |          |          |

|    |The average radiance over the      |               |[Watts][me|          |

|    |site for band 3 of the designated  |               |ter^-2]   |          |

|    |sensor.                            |               |          |          |

--------------------------------------------------------------------------------

|BAND3_SDEV_RADNC                        |    *, $$      |          |          |

|    |The standard deviation of the      |               |[Watts][me|          |

|    |radiance values over the site for  |               |ter^-2]   |          |

|    |band 3 of the designated sensor.   |               |          |          |

--------------------------------------------------------------------------------

|BAND1_AVG_REFL                          |      $$       |          |          |

|    |The average reflectance over the   |               |[percent] |          |

|    |site for band 1 of the designated  |               |          |          |

|    |sensor.                            |               |          |          |

--------------------------------------------------------------------------------

|BAND2_AVG_REFL                          |      $$       |          |          |

|    |The average reflectance over the   |               |[percent] |          |

|    |site for band 2 of the designated  |               |          |          |

|    |sensor.                            |               |          |          |

--------------------------------------------------------------------------------

|BAND1_EXOATMOSIC_REFL                   |      $$       |          |          |

|    |The at-satellite reflectance for   |               |[percent] |          |

|    |band 1 of the sensor, calculated   |               |          |          |

|    |as observed reflected radiance     |               |          |          |

|    |divided by the solar irradiance at |               |          |          |

|    |the top of the atmosphere.         |               |          |          |

--------------------------------------------------------------------------------

|BAND2_EXOATMOSIC_REFL                   |      $$       |          |          |

|    |The at-satellite reflectance for   |               |[percent] |          |

|    |band 2 of the sensor, calculated   |               |          |          |

|    |as observed reflected radiance     |               |          |          |

|    |divided by the solar irradiance at |               |          |          |

|    |the top of the atmosphere.         |               |          |          |

--------------------------------------------------------------------------------

|BAND3_EXOATMOSIC_REFL                   |      $$       |          |          |

|    |The at-satellite reflectance for   |               |[percent] |          |

|    |band 3 of the sensor, calculated   |               |          |          |

|    |as observed reflected radiance     |               |          |          |

|    |divided by the solar irradiance at |               |          |          |

|    |the top of the atmosphere.         |               |          |          |

--------------------------------------------------------------------------------

|FIFE_DATA_CRTFCN_CODE                   |               |          |          |

|    |The FIFE Certification Code for    |      **       |          |          |

|    |the data, in the following format: |               |          |          |

|    |CPI (Certified by PI), CPI-???     |               |          |          |

|    |(CPI - questionable data).         |               |          |          |

--------------------------------------------------------------------------------

|LAST_REVISION_DATE                      |               |          |          |

|    |The last revision date for the     |               |          |          |

|    |data, in the format (DD-mmm-YY).   |               |          |          |

--------------------------------------------------------------------------------

Footnotes:

$$ When the IMAGE_ID begins with "SX", Bands 1, 2, and 3 are used for

multispectral data. When the IMAGE_ID begins with "SP", only Band 1

has data. Bands 2 and 3 are null.

* The missing value indicator is -99.

** Valid levels

The primary certification codes are:

EXM - Example or Test data (not for release)

PRE - Preliminary (unchecked, use at your own risk)

CPI - Checked by Principal Investigator (reviewed for quality)

CGR - Checked by a group and reconciled (data comparisons and

cross checks)

The certification code modifiers are:

PRE-NFP  - Preliminary - Not for publication, at the request of

investigator.

CPI-MRG  - PAMS data which is "merged" from two separate receiving

stations to eliminate transmission errors.

CPI-???  - Investigator thinks data item may be questionable.

8.3  Sample Data Base Data Record.

SITEGRID_ID STATION_ID OBS_DATE  OBS_TIME IMAGE_ID         PLATFORM

----------- ---------- --------- -------- ---------------- ----------

4509-SPT           817 08-JUN-89     1729 SX043-1          SPOT1

4609-SPT            22 08-JUN-89     1729 SX043-1          SPOT1

5926-SPT            15 08-JUN-89     1729 SX043-1          SPOT1

8739-SPT            26 08-JUN-89     1729 SX043-1          SPOT1

INSTR_ID     NUM_OBS MIN_LAT      MAX_LAT      MIN_LON

------------ ------- ------------ ------------ --------------

HRV1               4  39 03 04.45  39 03 05.22   -96 36 36.00

HRV1               5  39 03 01.44  39 03 02.34   -96 36 38.75

HRV1               6  39 01 31.75  39 01 33.14   -96 34 11.59

HRV1               5  38 58 20.09  38 58 21.50   -96 32 26.75

MAX_LON        VIEW_ZEN_ANG VIEW_AZIM_ANG SOLAR_ZEN_ANG SOLAR_AZIM_ANG

-------------- ------------ ------------- ------------- --------------

-96 36 34.97          8.6         281.9          20.1          139.7

-96 36 37.13          8.6         281.9          20.1          139.7

-96 34 10.31          8.9         281.9          20.1          139.7

-96 32 25.72          9.3         281.9          20.0          139.7

BAND1_AVG_RADNC BAND1_SDEV_RADNC BAND2_AVG_RADNC BAND2_SDEV_RADNC

--------------- ---------------- --------------- ----------------

65.342            .4888          53.139            .5235

65.782           1.0726          53.610            .8757

62.813           1.4409          53.401           1.7521

54.035            .8189          38.742           1.0472

BAND3_AVG_RADNC BAND3_SDEV_RADNC BAND1_AVG_REFL BAND2_AVG_REFL

--------------- ---------------- -------------- --------------

69.893           1.1007            9.4           10.4

74.846            .7778            9.5           10.5

58.520           1.8957            8.9           10.5

71.544           1.3476            6.9            6.8

BAND1_EXOATMOSIC_REFL BAND2_EXOATMOSIC_REFL BAND3_EXOATMOSIC_REFL

--------------------- --------------------- ---------------------

11.9                  11.2                  22.2

12.0                  11.3                  23.8

11.5                  11.3                  18.6

9.9                   8.2                  22.7

FIFE_DATA_CERTFN_CODE LAST_REVISION_DATE

--------------------- ------------------

CPI                   09-JAN-91

CPI                   09-JAN-91

CPI                   09-JAN-91

CPI                   09-JAN-91

8.4  Data Format.

The CD-ROM file format consists of numerical and character fields of

varying length separated by commas. The character fields are enclosed with

a single apostrophe. There are no spaces between the fields.  Each file

begins with five header records. Header records contain the following

information:

Record 1    Name of this file, its table name, number of records in

this file, path and name of the document that describes

the data in this file, and name of principal investigator

for these data.

Record 2    Path and filename of the previous data set, and path and

filename of the next data set. (Path and filenames for files

that contain another set of data taken at the same site on

the same day.)

Record 3    Path and filename of the previous site, and path and filename

of the next site. (Path and filenames for files of the same

data set taken on the same day for the previous and next

sites (sequentially numbered by SITEGRID_ID)).

Record 4    Path and filename of the previous date, and path and filename

of the next date. (Path and filenames for files of the same

data set taken at the same site for the previous and next

date.)

Record 5    Column names for the data within the file, delimited by

commas.

Record 6    Data records begin.

Each field represents one of the attributes listed in the chart in

Section 8.2 and described in detail in the TDF file (see Section 8.1).

These fields are in the same order as in the chart.

8.5  Related Data Sets.

Level 3 Normalized Difference Vegetation Index (NDVI) Images from SPOT.

SPOT High Resolution Visible (HRV) Averages.

Site Reflectances Extracted from Landsat TM Imagery.

Site Average Reflectances Extracted from AVHRR-LAC Imagery.

Satellite Image Value Conversion Coefficients.

Daily and Site Radiation Flux Averages Derived from GOES.

9.  DATA MANIPULATIONS

9.1  Formulas.

9.1.1  Derivation Techniques/Algorithms.

The procedure used to calculate SPOT atmospherically corrected

reflectance was developed at GSFC as part of the FIFE staff

science effort. The images were calculated from FIFE level-1

data and the reflectance and atmospheric correction calculations

were performed using data and information available in the FIFE

data collection.

Sensor calibrated radiances for the SPOT HRV images were converted

to surface reflectance using the following procedures and informat-

ion available in the FIFE satellite extracts coefficients data set.

1) derive at-satellite radiance:

The radiance values for the respective bands were calculated as

follows:

Radiance = (DN(I) - OFFSET(I))/GAIN(I)

where:

Radiance = spectral radiance for channel I

[Watt][m^-2][sr^-1][um^-1]

DN(I) = digital number for channel I [counts]

OFFSET(I) = sensor offset for channel I [counts]

GAIN(I) = sensor gain for channel I

[counts][radiance unit^-1]

a) OFFSET Determination

The offset variable values used were those supplied on the

data tapes from SPOT Image Corporation. These values to date

on all tapes acquired for FIFE, have been equal to 0.0

(zero).

b) GAIN Determination

The gain variable values used are calculated from the gain

settings supplied on the data tape, a set of absolute calibr-

ation coefficients obtained from SPOT Image Corporation, and

the following equation :

GAIN(i) = CC(i) * [1.3**(Gs(i) - 3)]

where,

GAIN(i) is the gain value for band i.

CC(i) is the absolute calibration coefficient for band i

supplied by SPOT Image Corporation.

Gs(i) is the gain setting obtained from the data tape.

The following table contains the values for CC(i) for several dates

since launch (February 1986) for the SPOT 1 satellite. CC(i) values

for images acquired between these dates are linearly interpolated

from this table.

HRV1                          HRV2

DATE      PAN    XS1    XS2    XS3     PAN    XS1    XS2    XS3

---------  -----  -----  -----  -----   -----  -----  -----  -----

24-FEB-86  0.604  0.558  0.422  0.629   0.604  0.568  0.463  0.644

20-MAR-86  0.605  0.550  0.405  0.608   0.605  0.560  0.445  0.623

20-JUN-86  0.598  0.536  0.388  0.589   0.598  0.541  0.428  0.606

20-MAR-87  0.592  0.510  0.363  0.563   0.582  0.512  0.407  0.586

20-SEP-87  0.583  0.496  0.353  0.554   0.574  0.498  0.400  0.580

15-MAR-88  0.577  0.485  0.344  0.547   0.567  0.484  0.392  0.574

20-AUG-88  0.572  0.479  0.342  0.544   0.563  0.480  0.389  0.572

20-DEC-88  0.568  0.473  0.338  0.541   0.558  0.473  0.385  0.569

20-APR-89  0.564  0.466  0.335  0.538   0.553  0.468  0.381  0.567

20-AUG-89  0.561  0.461  0.332  0.536   0.549  0.462  0.378  0.566

20-DEC-89  0.556  0.454  0.329  0.534   0.543  0.454  0.375  0.566

2) convert to exoatmospheric reflectance (L'm) for each band:

L'm = (PI * Lm) / (F'o * cos(szen)

where (an apostrophe indicates a subscript):

Lm = apparent at-satellite calibrated spectral

radiance [mWatt][m^-2][sr^-1][um^-1]

F'o = Fo / (R^2)

R = d/d'

d = earth-sun distance (astronomical units)

d' = average earth-sun distance =1 astronomical unit

szen = solar zenith angle (degrees)

Fo = extraterrestrial solar irradiance in the HRV and PAN

bandpasses as listed below:

HRV1      HRV2      PAN

Band1   1884.0    1873.0    1689.0 [Watt][m^-2][um^-1]

Band2   1635.0    1590.0      -

Band3   1084.0    1039.0      -

3) calculate surface reflectance (rho) for each band:

rho = f / (1 + s * f)

where:

s = backscattering ratio (column BSCAT in the satellite

extract coefficients data set)

f = (L'm - L'o) / (F'd * T)

L'm = from (2) above

F'd = normalized surface flux (column IRRAD in the satellite

extract coefficients data set)

L'o = normalized path radiance (column PATH_RAD in the

satellite extract coefficients data set)

T = path transmission (column TRANS in the satellite

extract coefficients data set)

The final site radiances and reflectances are averaged over the site

and the mean and standard deviation for each channel is reported.

9.2  Data Processing Sequence.

9.2.1  Processing Steps and Data Sets.

FIFE staff creates the average instrument corrected spectral

radiance data by :

1) Enumerating the pixel and line coordinates of the pixels

that fall within each of the 39 sites.

2) Extracting the spectral, geographic, viewing, and solar

information for the enumerated pixels.

3) Converting and correcting the digital spectral counts to

radiance.

FIFE staff creates the reflectance data by :

1) Extracting the date, time, station_id, level-1a sensor

calibrated radiances, solar zenith angles, view zenith angles,

and observation height information from the FIS, for all dates,

sites, and SPOT visible and IR channels.

2) Correcting the apparent at-sensor radiances for Earth-Sun

distance and solar zenith angle by converting them to

exoatmospheric (top of the atmosphere) reflectances, calculated

as described in Fraser et al., (1989), and in Markham and Barker

(1986).

3) Extracting optical thickness measurements from the FIS for the

variety of instruments that collected such data, and calculating

the aerosol optical thickness for the SPOT-channel wavelengths.

Optical thickness measurements are interpolated to the SPOT

channels using a linear fit of the log of wavelength regressed

against the log of optical thickness. SPOT-channel model

wavelengths are calculated as described in Fraser et al.,

(1989). For the acquisitions in which no optical thickness

measurements were available, a default value calculated from

the median of all such measurements taken in 1988 and 1989

is used. The default values used are 0.14, 0.12, 0.08, for

SPOT channels 1-3 respectively, and 0.12 for the panchromatic

channel.

4) Calculating default extinction coefficients (optical thicknesses)

due to water vapor and other gas (carbon dioxide and ozone)

absorption from the Lowtran7 mid-latitude summer model. The

default values calculated are listed below (the quantities are

dimensionless):

HRV1                HRV2                PAN

Water    Other      Water    Other     Water    Other

-------  -------    -------  -------   -------  -------

Band1   0.00740  0.02526    0.00451  0.02732   0.01610  0.02738

Band2   0.01319  0.02881    0.01720  0.02456      -        -

Band3   0.04908  0.00849    0.05362  0.00127      -        -

5) Calculating ground level downward irradiance, upward radiance,

and surface reflectance in the SPOT channels for each of the

sites in each acquisition using the algorithm of Fraser et al.,

(1989) and all the atmospheric, geometric, and radiometric data

from above. The algorithm also outputs coefficients that allow

conversion of any other portion of the scene to be corrected to

surface reflectance. These coefficients are included in the

satellite coefficients data set.

9.2.2  Processing Changes.

None.

9.3  Calculations.

9.3.1  Special Corrections/Adjustments.

None.

9.4  Graphs and Plots.

None.

10.  ERRORS

10.1  Sources of Error.

Errors could arise in the acquired imagery due to location accuracy,

distortion of lengths, anisomorphism, the instrument's local coherence,

the ability to register multispectral data, and relief plotting

accuracy. Other errors could arise from inherent radiometric

imperfections of the sensors. Spectral errors arise due to image

wide signal-to-noise ratio, saturation, cross-talk, spikes,

response normalization due to change in gain.

Whatever the processing level, the geometric quality of the image depends

on, a) the accuracy of the viewing geometry, and b) the ground control

points as required to adjust the viewing model. Errors arise from the

processing of the data to level-1A due to calibration inaccuracies (gains

and offsets/drift of sensor characteristics/optical degradation), and

resampling/positioning in geometric corrections.

10.2  Quality Assessment.

10.2.1  Data Validation by Source.

The FIFE staff was responsible for screening imagery for quality,

applying radiometric corrections, computing geometric corrections

corresponding to the required map projection, applying geometric

corrections, and screening for cloud cover on imagery during

processing.

10.2.2  Confidence Level/Accuracy Judgment.

The precision of satellite remote sensing estimates of surface

reflectance (Hall et al., 1992), calibrated and corrected for

atmospheric effects, was no worse than about 1 percent absolute.

The errors may actually be smaller, but an upper bound of 1

percent results from sampling variance caused by differences

among the satellite and ground sensors in spatial resolution,

atmospheric effects, and calibration.

10.2.3  Measurement Error for Parameters and Variables.

The magnitude of the errors described in Section 10.1 is unknown.

10.2.4  Additional Quality Assessment Applied.

FIS staff applied a general Quality Assessment (QA) procedure to

the data to identify inconsistencies and problems for potential

users. As a general procedure, the FIS QA consisted of examining

the maximum, minimum, average, and standard deviation for each

numerical field in the data table. An attempt was made to find an

explanation for unexpected high or low values, values outside of

the normal physical range for a variable, or standard deviations

that appeared inconsistent with the mean. In some cases, histog-

rams were examined to determine whether outliers were consistent

with the shape of the data distribution.

The discrepancies, which were identified, are reported as

problems in Section 11.1.

11.  NOTES

11.1  Known Problems With The Data.

As of the revision date of this document, the several discrepancies or

errors in the data have been reported. For each band a number of

unusually high values have been identified in the average reflectances,

average radiances and exoatmospheric reflectances.  Those values above

the indicated threshold (given in the table below) should be used with

caution.

Number                Maximum

Band      Parameter        of      Threshold    Value

Values               Encountered

-----   ---------------   ------   ----------  -----------

BAND1   AVG_REFL            24        20

BAND1   AVG_RADNC           28        100       MAX 280

BAND1   EXOATMOSIC_REFL     25        20        MAX 50+

BAND2   AVG_REFL            21        20

BAND2   AVG_RADNC           20        90        MAX 239

BAND2   EXOATMOSIC_REFL     15        25        MAX 50+

BAND3   AVG_REFL            14        50

BAND3   AVG_RADNC            5        150       MAX 185

BAND3   EXOATMOSIC_REFL      5        50

BAND4   AVG_REFL             1        12

Only a few of the SPOT images caused the majority of the spikes noted

above.  Those images are listed below and of those, three (marked with

an '&' below) are associated with the highest spikes.

IMAGE_ID         OBS_DATE

----------       ---------

SX006-1  &       01-JUN-87

SX009-1  &       11-JUN-87

SX012-1          18-JUL-87

SX013-1          18-JUL-87

SX014-1          23-JUL-87

SX029-1          13-APR-88

SX033-1  &       05-JUL-88

11.2  Usage Guidance.

The SPOT extract data collected for FIFE may be useful for establishing

relationships with bio- and geo-physical variables on the Earth's

surface, and for comparison with other prairie landscapes.

11.3  Other Relevant Information.

None.

12.  REFERENCES

12.1  Satellite/Instrument/Data Processing Documentation.

Kneizys, F.X., E.P. Shettle, W.O. Gallery, J.H. Chetwynd, L.W.

Abreu, J.E.A. Selby, S.A. Clough, and R.W. Fenn. 1988. Atmospheric

transmittance/radiance: computer code LOWTRAN-7, AFGL-TR-88-0177. Air

Force Geophysics Lab. Hanscomb AFB, Massachusetts.

Newcomer, J.A., S.J. Goetz, D.E. Strebel, and F.G. Hall. 1989. Image

processing software for providing radiometric inputs to land surface

climatology models. IGARSS '89, 12th Can. Symp. on Remote Sensing.

pp. 1779-1782.

12.2  Journal Articles and Study Reports.

Asrar, G., R Murphy, F. Hall, and P Sellers. 1991. Use of multitemporal

SPOT data in First ISLSCP Field Experiment. Proc. 5th Intl.

Colloquium. Courchevel, France. 14-18 January.

Boissin, B., and J. Perbos. 1985. SPOT image quality and post launch

assessment. Advanced Space Res. 5:51-60.

Carper, J.W., T.M. Lillesand, and R.W. Kiefer. 1990. The use of

intensity-hue-saturation transformations for merging SPOT panchromatic

and multispectral image data. Photogr. Engr. and Rem. Sen. 56:459-467.

Chavez, P.S.,Jr., and J.A. Bowell. 1988. Comparison of the spectral

information content of Landsat Thematic Mapper and SPOT for three

different sites in the Phoenix, Arizona, Region. Photogr. Engr. and

Rem. Sen. 54:1699-1708.

Chevrel, M., M. Courtois, and G. Weill. 1981. The SPOT satellite remote

sensing mission. Photogr. Engr. and Rem. Sens. 47:1163-1171.

Fraser, R. S., R. A. Ferrare, Y. J. Kaufman, and S. Mattoo. 1989.

Algorithm for atmospheric corrections of aircraft and satellite

imagery. NASA Technical Memorandum 100751. 106p.

Hall, F.G., D.E. Strebel, J.E. Nickeson, and S.J. Goetz. 1991.

Radiometric Rectification: Toward a Common Radiometric Response Among

Multidate, Multisensor Images. Remote Sens. Environ. 35:11-27.

Hall, F.G., K.F. Huemmrich, S.J. Goetz, P.J. Sellers, and J.E. Nickeson.

1992. Satellite remote sensing of surface energy balance: success,

failures, and unresolved issues in FIFE. J. Geophys. Res.

97:19061-19089.

Markham, B.L., and J.L. Baker. 1986. Landsat MSS and TM post-calibration

dynamic ranges, exoatmospheric reflectances and at-satellite temp-

eratures. EOSAT Landsat Tech. Notes 1:3-7. Lanham, Maryland.

Welch, R., and M. Ehlers. 1987. Merging multiresolution SPOT HRV and

Landsat TM data. Photogr. Engr. and Rem. Sen. 53:301-305.

12.3  Archive/DBMS Usage Documentation.

12.3  Archive/DBMS Usage Documentation.

The Collected Data of the First ISLSCP Field Experiment is archived at

the EOS Distributed Active Archive Center (DAAC) at Oak Ridge National

Laboratory (ORNL), Oak Ridge, Tennessee (Section 13 below).

Documentation about using the archive and/or online access to the data

at the ORNL DAAC is not available at this revision.

13.  DATA ACCESS

13.1  Contacts for Archive/Data Access Information.

ORNL DAAC User Services

P.O. Box 2008

Mail Stop 6490

Oak Ridge National Laboratory

Oak Ridge, TN 37831-6490

(615) 241-3952

13.2  Archive Identification.

EOSDIS Distributed Active Archive Center

P.O. Box 2008

Mail Stop 6490

Oak Ridge National Laboratory

Oak Ridge, TN 37831-6490

USA

Telephone:       (615) 241-3952

FAX:             (615) 574-4665

Internet Email:   ornldaac@onrl.gov

13.3  Procedures for Obtaining Data.

Users may place requests by letter, telephone, electronic mail, FAX, or

personal visit.

13.4  ORNL DAAC Status/Plans.

FIFE data will be available from the ORNL DAAC.  Please contact the

ORNL DAAC User Services Office for the most current information about

these data.

14.  OUTPUT PRODUCTS AND AVAILABILITY

14.1  Tape Products.

None.

14.2  Film Products.

None.

14.3  Other Products.

Site Reflectances Extracted from SPOT HRV Imagery are available on FIFE

CD-ROM Volume 1.

15.  GLOSSARY OF ACRONYMS

ASCII             American Standard Coded Information Interchange

BPI               Byte per inch

CCT               Computer Compatible Tape

CD-ROM            Compact Disk-Read Only Memory

DAAC              Distributed Active Archive Center

EOSDIS            Earth Observing System Data and Information System

FIFE              First ISLSCP Field Experiment

FIS               FIFE Information System

GMT               Greenwich Mean Time

HRV               High Resolution Visible

ISLSCP            International Satellite Land Surface Climatology Project

ORNL              Oak Ridge National Laboratory

SPOT              Systeme Pour l'Observation de la Terre

UTM               Universal Transverse Mercator