1.  TITLE

1.1  Data Set Identification.

Soil Moisture Transect.

1.2  FIS Data Base Table Name.

SOIL_MOIST_TRANSECT_DATA.

1.3  CD-ROM File Name.

\DATA\SOILMSTR\SM_TRAN\GRIDxxxx\ydddgrid.SMT

Where xxxx is the four digit code for the location within the FIFE site

grid. Note: capital letters indicate fixed values that appear on the CD-

ROM exactly as shown here, lower case indicates characters (values) that

change for each path and file.

The format used for the filenames is: ydddgrid.sfx, where grid is the

four-number code for the location within the FIFE site grid, y is the

last digit of the year (e.g., 7=1987, and 9=1989), and ddd is the day of

the year (e.g., 061=sixty-first day in the year). The filename extension

(.sfx), identifies the data set content for the file (see Section 8.2) and

is equal to .SMT for this data set.

1.4  Revision Date Of This Document.

May 6, 1994.

Warning:  This document has not been checked for technical or editorial

accuracy by the FIFE Information Scientist. There may be

inconsistencies with other documents, technical or editorial

errors that were inadvertently introduced when the document was

compiled or references to preliminary data that were not

included on the final CD-ROM.

Previous versions of this document have been reviewed by the

Principal Investigator, the person who transmitted the data to

FIS, a FIS staff member, or a FIFE scientist generally familiar

with the data.

2.  INVESTIGATOR(S)

2.1  Investigator(s) Name And Title.

James Wang

Goddard Space Flight Center

2.2  Title Of Investigation.

Estimation of Soil Moisture and other Surface Parameters from Airborne

and Satellite Visible, Infrared and Microwave Data.

2.3  Contacts (For Data Production Information.

________________________________________________________________________

|              |     Contact 1              |        Contact 2           |

|______________|____________________________|____________________________|

|2.3.1 Name    |Dr. E.T. Engman             |Dr. S.G. Ungar              |

|2.3.2 Address |Code 974                    |Code 923                    |

|              |NASA/Goddard Sp. Fl. Ctr.   |NASA/Goddard Sp. Fl. Ctr.   |

|      City/St.|Greenbelt, MD               |Greenbelt, MD               |

|      Zip Code|20771                       |20771                       |

|2.3.3 Tel.    |(301) 286-5355              |(301) 286-4007              |

|2.3.4 Email   |tengman@gsfcmail.nasa.gov   |ungar@higwire.gsfc.nasa.gov |

|______________|____________________________|____________________________|

2.4  Requested Form of Acknowledgment.

The Soil Moisture Transect data were collected for FIFE by E.T. Engman and

his colleagues at the Goddard Space Flight Center, and Barc at USDA.

3.  INTRODUCTION

3.1  Objective/Purpose.

The aim of the FIFE soil moisture transect work was to characterize

spatial and temporal patterns of soil moisture along selected transects at

the FIFE study area, so as to help validate and calibrate remote sensing

measurements of soil moisture, and to evaluate alternative methods of

measuring soil moisture both from the air and on the ground.

3.2  Summary of Parameters.

Gravimetric soil moisture, volumetric soil moisture, bulk density, wet and

dry soil weight, and soil temperature.

3.3  Discussion.

Ground data were collected to support the passive microwave (PBMR)

flights over the Konza experimental area. There were two levels of ground

data collection. The top level consisted of three transects of 13 to 17

samples each that traversed watersheds 1D and 2D. These transects were

also the flight lines for the low level PBMR flights (aircraft elevation

approximately 300 m above ground surface). The three top level transects

were sampled for each CMP-3 mission by taking gravimetric samples and soil

impedance measurements at each pre-marked sampling site. Sample sites

were marked by flags and the actual samples were taken from a circular

area with a radius of about 5 m. Other transects were measured less frequ-

ently depending on the need to support specific instruments (radar or

gamma) or scientists' needs.

4.  THEORY OF MEASUREMENTS

Soil moisture measured along a transect is necessary to calibrate airborne

moisture instruments or compare data obtained from them. Soil units in a

landscape are inherently heterogeneous, which leads to variations in moisture

content along an aircraft flight path on the ground. In order to reduce errors

that arise due to use of average values covering large areas, it is more

prudent to use values on the flight path, sampled at closer intervals.

Water content measurements by gravimetric methods involve weighing the wet

sample, drying the sample in an oven, and reweighing the sample to determine

the amount of water removed. Water content then is obtained by dividing the

difference between wet and dry masses by the mass of the dry sample to obtain

the ratio of the mass of water to the mass of dry soil. When multiplied by 100,

this becomes the percentage of water in the sample on a dry-mass (or, as often

expressed, on a dry-weight) basis.

5.  EQUIPMENT

5.1  Instrument Description.

Soils were sampled with an auger, sampling tube or other suitable device

to take a soil sample. Samples were stored in soil containers with tight-

fitting lids and/or tape, dried in an oven with means for controlling the

temperature between 100 and 110 degrees Celsius. Samples were weighed with

a balance.

5.1.1  Platform.

Ground.

5.1.2  Mission Objectives.

The aim was to determine soil moisture and bulk density along

established transects, in support of airborne soil moisture

measurements.

5.1.3  Key Parameters.

Soil moisture, bulk density, dry weight, and wet weight.

5.1.4  Principles of Operation.

Previously weighed soil samples are dried to a constant weight,

usually at about 105 degrees C, and then weighed again. The differ-

ence in dry and wet weight of soil is expressed as a percentage of

the dry weight. The bulk density is determined after drying the

sample to constant weight, and using the volume of the sample as

taken in the field, to find the ratio of mass of dry solids to the

bulk volume of the sample.

5.1.5  Instrument Measurement Geometry .

Soil samples were obtained with a 2 cm diameter coring tube

or a 5 x 5 x 5 cm sample tool.

5.1.6  Manufacturer of Instrument.

Assorted vendors for gravimetric moisture components.

5.2  Calibration.

The weighing balance did not require calibration.

5.2.1  Specifications.

None.

5.2.1.1  Tolerance.

Not applicable.

5.2.2  Frequency of Calibration.

None.

5.2.3  Other Calibration Information.

None.

6.  PROCEDURE

6.1  Data Acquisition Methods.

Soil samples of the top 5 cm of soil were taken using the special sampling

tool, carving a sample with a putty knife, or taking several cores with a

2 cm diameter coring tube. The total sample (approximately 100-150 grams)

was put in a tin sample can and sealed with plastic tape. Each sample can

was identified by the sample site ID code. The cans were returned to the

laboratory, and the cans were opened and weighed to obtain the wet weight.

Open cans were placed in ovens at 105 degrees Celsius for a minimum of 24

hours for drying, then they were weighed again to obtain the dry weight.

Bulk density is determined by excavating a quantity of soil, drying and

weighing it, and determining the volume of the excavation. The volume was

determined by inserting a rubber-balloon into the excavation and filling

it with water until the excavation was just full. The volume of the excav-

ated soil sample is equal to the volume of the fluid dispensed from a

graduated cylinder.

Soil samples were collected from the following transects:

Transect 883: Approximate coordinates:  39 04' 53" N, 96 33' 42" W

39 04' 53" N, 96 33' 17" W

Transect 883 is numbered from West to East; Sample

sites are approximately 50 m apart.

Sample 2175: 10 m from road, on top.

Sample 2125: 50 m east of 2175 on top.

Sample 2075: 50 m east of 2125 on small terrace west of channel.

Sample 2000: 50 m east of 2075 near insect traps, stoney slope.

Sample 1925: 50 m east of 2000 on top, gentle slope.

Sample 1875: 50 m east of 1925 on top, 30 m west of flight marker.

Sample 1825: 50 m east of 1875 on top in unburned watershed 2D.

Sample 1750: 50 m east of 1825 on top.

Sample 1700: 50 m east of 1750 on tongue between gulleys.

Sample 1650: 35 m east of 1700 on tongue between gulleys.

Sample 1600: 28 m east of 1650 in gulley.

Sample 1575: 35 m east of 1600, half way up hill.

Sample 1525: 50 m east of 1575 on top.

Sample 1450: 50 m east of 1525 on top, near fence corner.

Transect 884: Approximate coordinates: 39 04' 43" N, 96 33' 50" W

39 04' 43" N, 96 33' 14" W.

Transect 884 is numbered from west to east; sample

sites are approximately 50 m apart.

Sample 2100: 10 m east of road on top.

Sample 2050: 42 m east of 2100 just before slope down but flat.

Sample 2000: 42 m east of 2050 at bottom of slope, alluvial soil.

Sample 1950: 50 m east of 2000 on tongue between channels, alluvial.

Sample 1900: 50 m east of 1950 on gentle slope before steep slope.

Sample 1850: 50 m east 0f 1900 on top of nob, stoney soil.

Sample 1800: 50 m east of 1850 25 m from flight marker, stoney soil.

Sample 1725: 50 m east of 1800 25 m east of marker, unburned, just

before drop-off.

Sample 1675: 50 m east of 1725 bottom of slope, just before gulley.

Sample 1625: 35 m east of 1675 between gulleys on high tongue.

Sample 1575: 50 m east of main channel

Sample 1525: 50 m east of 1575 parallel to large gulley up a

gentle slope

Sample 1475: 50 m east of 1525 parallel to large gulley on small

bench.

Sample 1400: 50 m east of 1475 parallel to gulley on hillside.

Sample 1325: 50 m east of 1400 parallel to gulley but on the south

side of it (crossed over).

Sample 1275: 50 m east of 1325 almost at head of gulley on gentle

slope near top.

Sample 1200: 50 m east of 1275 on top 10 m from fire road.

Transect 885: Approximate coordinates: 39 04' 36" N, 96 33' 20" W

39 04' 36" N, 96 33' 56" W.

Transect 885 in numbered from east to west; sample

sites are approximately 50 m apart.

Sample 1490: 10 m west of fire road on top of hill.

Sample 1540: 50 m west of 1490 half way down slope.

Sample 1590: 50 m west of 1540 middle of long gentle slope to stream

Sample 1640: 50 m west of 1590 near bottom of slope and parallel

to small gulley.

Sample 1690: 50 m west of 1640 on tongue between channels.

Sample 1740: 50 m west of 1690 mid-way up slope to marker.

Sample 1790: 50 m west of 1740 and 5 m west of marker, fire road

mid-way between 15-7 and 15-8.

Sample 1840: 50 m west of 1790 near bottom of stoney slope.

Sample 1883: 43 m west of 1840 near center of gradual slope to

channel and big tree (at 50 m).

Sample 1953: 70 m west of channel on stoney soil.

Sample 2003: 50 m west of 1953 at base of hill, stoney soil.

Sample 2048: 45 m west of 2003 & 5 m before drop into gulley.

Sample 2103: 55 m west of 2048 in gulley slump area.

Sample 2153: 50 m west of 2103 about 2/3 up slope.

Sample 2203: 50 m west of 2153 on top 20 m from fire road on

west end of transect.

Transect 886: Approximate coordinates: 39 04' 34" N, 96 33' 56" W

39 04' 34" N, 96 33' 21" W

Transect 886 is numbered from west to east; sample

sites are approximately 50 m apart.

Between IFC-1 and IFC-2, transect 885 was moved approximately 65 m south,

so that the transect 886 was in line with the southern-most dead tree of

the six trees (second tree from the south of the total of six trees)

along the channel in 2D. The transect 885 used in IFC-1 was in line with

the northern-most of the six trees. The new transect is numbered 886 and

the sample sites run from west to east (as do 883 and 884). Each sample

site is spaced about 50 to 55 m from the previous site. Site 885-1 is

located about 10 m east of the fire road on the western boundary of

watershed 1D, sample number 2164.

Transect 881: Approximate coordinates: 39 03'07 N, 96 32'35 W

39 03'07 N, 96 34'00 W.

Transect 881 is numbered from east to west; sample

sites are approximately 50 m apart.

Transect 881 was chosen to monitor the privately owned, burned cover, to

provide a calibration line for the NOAA gamma instrument, and to provide

a relatively level ground data track for the Kansas radar mounted on the

NASA helicopter. The 18 sites on transect 881 were located about 50 m

apart, starting about 50 m west of Shashi Verma's EC/S, PAM6 site (site-

grid=4439). The sample sites were not flagged so subsequent samples taken

on different days were not necessarily within 5 m of the

previous samples.

Transect 884 extended: Approximate coordinates: 39 04'43" N, 96 34'05" W

39 04'43" N, 96 34'45" W.

Transect 884 extended is a western extension of transect 884

into the unburned area of watershed N1B. Sample sites are

approximately 50 m apart.

Transect 884 extended was chosen to monitor soil moisture in an unburned area

that had been sampled in 1985 to provide comparable data. The sample sites

(7 sites numbered 2871 through 2421) started about 50 m west of the fire road

and extended at about 50 m intervals down a rough and steep hill. The sample

sites were flagged.

Transect 882:  Approximate coordinates: 39 04'30" N, 96 36'00" W

39 04'30" N, 96 36'30" W.

Transect 882 consisted of 6 unflagged sample sites approximately 50 m

apart starting 50 m west of the fire road and crossing the northern arm

of watershed 4B. Sample sites progressed East to West.

Locations of 1985 transects (station numbers 851 through 855) are included

in the Miscellaneous Geographic Reference data on FIFE CD-ROM Volume 5.

6.2  Spatial Characteristics.

The FIFE study area, with areal extent of 15 km by 15 km, is located south

of the Tuttle Reservoir and Kansas River, and about 10 km from Manhattan,

Kansas, USA. The northwest corner of the area has UTM coordinates of

4,334,000 Northing and 705,000 Easting in UTM Zone 14.

6.2.1  Spatial Coverage.

Transect data were gathered mostly from watershed 1D in the

northwest quadrant of the FIFE study area (a diagram of watershed

1D is on FIFE CD-ROM Volume 5 in the Miscellaneous GIS Raster

Images data grouping). The stations and sitegrids for the starting

point for each of the soil moisture transects are shown below:

SITEGRID  STN  NORTHING  EASTING  LATITUDE  LONGITUDE

--------  ---  --------  -------  --------  ---------

2829-SMT  855  4328480   710850   39 04 55  -96 33 45

2840-FTR  883  4328485   712935   39 04 53  -96 32 18

2940-FTR  884  4328177   712943   39 04 43  -96 32 18

3040-FTR  885  4327961   712949   39 04 36  -96 32 18

3126-SMT  853  4327790   710280   39 04 33  -96 34 09

3140-FTR  886  4327899   712951   39 04 34  -96 32 18

3140-FTR  882  4327776   712954   39 04 30  -96 32 18

3228-SMT  852  4327700   710640   39 04 30  -96 33 54

3229-SMT  851  4327700   710700   39 04 30  -96 33 52

4440-FTR  881  4325217   713023   39 03 07  -96 32 18

6.2.2  Spatial Resolution.

These were point data, collected approximately 50 m apart along

transect lines. FIS staff converted site numbers to estimates of

distance from the road (U.S. Rt. 177) in meters. The distance

estimate is probably +/-20 m at most, see Section 6.1.

6.3  Temporal Characteristics.

6.3.1  Temporal Coverage.

Prior to FIFE 1987 IFC, data were collected from June 13 through

July 11, 1985. Soil samples were collected from late spring through

the fall of 1987, during the Intensive Field Campaigns (IFC) of

approximately 2 weeks each. Specifically, 1987 data were collected

from May 28 through October 15, 1987.

6.3.2  Temporal Resolution.

Maximum of one set (one transect) of measurements per day during

the IFCs.

7.  OBSERVATIONS

7.1  Field Notes.

None.

8.  DATA DESCRIPTION

8.1  Table Definition With Comments.

The SQL definition for this table is found in the SM_TRAN.TDF file

found on FIFE CD-ROM Volume 1.

8.2  Type of Data.

--------------------------------------------------------------------------------

|                 8.2.1                  |               |          |          |

|Parameter/Variable Name                 |               |          |          |

--------------------------------------------------------------------------------

|    |               8.2.2               |     8.2.3     |  8.2.4   |  8.2.5   |

|    |Parameter/Variable Description     |Range          |Units     |Source    |

--------------------------------------------------------------------------------

|SITEGRID_ID                             |               |          |          |

|    |This is a FIS grid location code.  |               |          |FIS       |

|    |Site grid codes (SSEE-III) give    |               |          |          |

|    |the south (SS) and east (EE) cell  |               |          |          |

|    |number in a 100x100 array of 200m  |               |          |          |

|    |square cells.  The last 3          |               |          |          |

|    |characters (III) are an instrument |               |          |          |

|    |identifier.                        |               |          |          |

--------------------------------------------------------------------------------

|STATION_ID                              |               |          |          |

|    |The three-digit FIFE site          |min = 851,     |          |FIS       |

|    |identification number for the site |max = 886      |          |          |

|    |where the data were collected.     |               |          |          |

--------------------------------------------------------------------------------

|OBS_DATE                                |               |          |          |

|    |The date on which the observation  |min = 13-JUN-85|          |GSFC      |

|    |was made.                          |,              |          |          |

|    |                                   |max = 15-OCT-87|          |          |

--------------------------------------------------------------------------------

|OBS_TIME                                |               |          |          |

|    |The time of day that the data      |min = 608,     |[GMT]     |GSFC      |

|    |were collected, given as the       |max = 2157,    |          |          |

|    |midpoint of 30-minute average.     |missing = -999 |          |          |

--------------------------------------------------------------------------------

|SAMPLE_NUM                              |               |          |          |

|    |The sample number, an              |min = 0,       |          |GSFC      |

|    |approximation of the distance      |max = 5885     |          |          |

|    |along the transect in meters.      |               |          |          |

--------------------------------------------------------------------------------

|SAMPLE_DEPTH                            |               |          |          |

|    |The depth of the sample.           |min = 5,       |[cm]      |SCALE MEAS|

|    |                                   |max = 5        |          |URE       |

--------------------------------------------------------------------------------

|SOIL_TEMP                               |               |          |          |

|    |The temperature of the soil at     |min = 10,      |[degrees C|THERMOMETE|

|    |the sample depth.                  |max = 38,      |elsius]   |R         |

|    |                                   |missing = -99.9|          |          |

--------------------------------------------------------------------------------

|WET_WEIGHT                              |               |          |          |

|    |The weight of the soil sample      |min = 37.29,   |[grams]   |WEIGHING B|

|    |before drying.                     |max = 196.65,  |          |ALANCE    |

|    |                                   |missing = -9.99|          |          |

--------------------------------------------------------------------------------

|DRY_WEIGHT                              |               |          |          |

|    |The weight of the soil sample      |min = 26.37,   |[grams]   |WEIGHING B|

|    |after drying.                      |max = 168.4,   |          |ALANCE    |

|    |                                   |missing = -9.99|          |          |

--------------------------------------------------------------------------------

|BULK_DENSITY                            |               |          |          |

|    |The bulk density of the soil.      |min = .53,     |[grams][cm|FIXED RING|

|    |                                   |max = 1.29,    |^-3]      |          |

|    |                                   |missing = -9.99|          |          |

--------------------------------------------------------------------------------

|GRAVMTRC_SOIL_MOISTURE                  |               |          |          |

|    |The gravimetric soil moisture.     |min = 4.4,     |[percent] |SOIL CORE |

|    |                                   |max = 63.68,   |          |          |

|    |                                   |missing = -9.99|          |          |

--------------------------------------------------------------------------------

|VOLUMTRC_SOIL_MOISTURE                  |               |          |          |

|    |The volumetric soil moisture.      |min = 4,       |[percent] |SOIL CORE |

|    |                                   |max = 72.6,    |          |          |

|    |                                   |missing = -9.99|          |          |

--------------------------------------------------------------------------------

|FIFE_DATA_CRTFCN_CODE                   |      **       |          |          |

|    |The FIFE Certification Code for    |CPI=checked by |          |FIS       |

|    |the data, in the following format: |principal inves|          |          |

|    |CPI (Certified by PI), CPI-???     |tigator,       |          |          |

|    |(CPI - questionable data).         |CPI-UPS= checke|          |          |

|    |                                   |d by principal |          |          |

|    |                                   |investigator an|          |          |

|    |                                   |d updated posit|          |          |

|    |                                   |ion of sample  |          |          |

--------------------------------------------------------------------------------

|LAST_REVISION_DATE                      |               |          |          |

|    |The last revision date for the     |min = 24-DEC-86|          |FIS       |

|    |data, in the format (DD-MMM-YY).   |,              |          |          |

|    |                                   |max = 11-MAY-90|          |          |

--------------------------------------------------------------------------------

Footnote:

** Decode the FIFE_DATA_CRTFCN_CODE field as follows:

The primary certification codes are:

EXM - Example or Test data (not for release).

PRE - Preliminary (unchecked, use at your own risk).

CPI - Checked by Principal Investigator (reviewed for quality).

CGR - Checked by a group and reconciled (data comparisons and

cross checks).

The certification code modifiers are:

PRE-NFP  - Preliminary - Not for publication, at the request of

investigator.

CPI-MRG  - PAMS data that is "merged" from two separate receiving

stations to eliminate transmission errors.

CPI-???  - Investigator thinks data item may be questionable.

8.3  Sample Data Record.

SITEGRID_ID STATION_ID OBS_DATE  OBS_TIME SAMPLE_NUM SAMPLE_DEPTH SOIL_TEMP

----------- ---------- --------- -------- ---------- ------------ ---------

4440-FTR           881 30-JUN-87     1935        408            5     22.00

4440-FTR           881 30-JUN-87     1938        450            5     21.00

4440-FTR           881 30-JUN-87     1942        508            5     21.00

4440-FTR           881 30-JUN-87     1945        558            5     21.00

WET_WEIGHT DRY_WEIGHT BULK_DENSITY GRAVMTRC_SOIL_MOISTURE

---------- ---------- ------------ ----------------------

122.280     97.610        .9000                 41.900

117.750     95.950        .9000                 38.100

126.140    104.830        .9000                 32.200

102.140     83.100        .9000                 42.900

VOLUMTRC_SOIL_MOISTURE FIFE_DATA_CRTFCN_CODE LAST_REVISION_DATE

---------------------- --------------------- ------------------

37.700 CPI                   11-MAY-90

34.300 CPI                   11-MAY-90

29.000 CPI                   11-MAY-90

38.600 CPI                   11-MAY-90

8.4  Data Format.

The CD-ROM file format consists of numerical and character fields of

varying length separated by commas. The character fields are enclosed with

a single apostrophe. There are no spaces between the fields.  Each file

begins with five header records.  Header records contain the following

information:

Record 1    Name of this file, its table name, number of records in

this file, path and name of the document that describes

the data in this file, and name of principal investigator

for these data.

Record 2    Path and filename of the previous data set, and path and

filename of the next data set.  (Path and filenames for files

that contain another set of data taken at the same site on

the same day.)

Record 3    Path and filename of the previous site, and path and filename

of the next site.  (Path and filenames for files of the same

data set taken on the same day for the previous and next

sites (sequentially numbered by SITEGRID_ID)).

Record 4    Path and filename of the previous date, and path and filename

of the next date.  (Path and filenames for files of the same

data set taken at the same site for the previous and next

date.)

Record 5    Column names for the data within the file, delimited by

commas.

Record 6    Data records begin.

Each field represents one of the attributes listed in the chart in

Section 8.2 and described in detail in the TDF file (see Section 8.1).

These fields are in the same order as in the chart.

8.5  Related Data Sets.

Soil Moisture Release.

Soil Water Properties.

Gravimetric Soil Moisture.

Soil Properties Reference Information.

Neutron Probe Soil Moisture.

Soil Moisture Contours.

Peck Airborne Gamma Ray Soil Moisture.

FIFE Level-3 Pushbroom Microwave Radiometer (PBMR) Soil Moisture Imagery.

9.  DATA MANIPULATIONS

9.1  Formulas.

9.1.1  Derivation Techniques/Algorithms.

Percent soil moisture on a weight basis was calculated according

to the following formula:

Gravimetric soil moisture = ((Wet wt. - Dry wt.)/(Dry wt.)) x 100

Percent soil moisture on a volume basis was calculated according

to the following formula.

Volumetric moisture (soil moisture on weight basis) x Bulk density

Calculate the bulk density according to:

Dry soil wt. (grams)

Bulk density =   ---------------------

(Volume 1 - Volume 2)

9.2  Data Processing Sequence.

9.2.1  Processing Steps and Data Sets.

1) Assemble pre-drying weight values by can number.

2) Assemble post-drying weight values by can number.

3) Determine tare weight values for each can.

4) Calculate soil moisture and bulk density.

9.2.2  Processing Changes.

None.

9.3  Calculations.

9.3.1  Special Corrections/Adjustments.

None.

9.4  Graphs and Plots.

None.

10.  ERRORS

10.1  Sources of Error.

Minor errors could arise in water content measurements depending on the

technique used to avoid absorption of water from the air during cooling

and prior to weighing. Also, the time necessary to reach constant weight

will depend upon the type of oven used (e.g., convection type forced

draft), the size or depth of the sample, the nature of the soil, and if

the oven is over loaded. Major errors could result from mistakenly

spilling soil samples.

Water content values for stony or gravelly soils can be misleading. Large

rocks can occupy appreciable volume in a soil sample and contribute appr-

eciably to the mass without a commensurate contribution to the porosity

or water capacity of the soil.

10.2  Quality Assessment.

10.2.1  Data Validation by Source.

Consistency checks (spatial and temporal) were made by the

Principal Investigator and no unusual values were found.

10.2.2  Confidence Level/Accuracy Judgment.

The Principal Investigator is confident that the transect data

were good.

10.2.3  Measurement Error for Parameters and Variables.

No quantitative assessment was made, see Section 10.2.2.

Other errors mentioned in Section 10.1 above were not quantified.

10.2.4  Additional Quality Assessment Applied.

FIS staff applied a general Quality Assessment (QA) procedure to

the data to identify inconsistencies and problems for potential

users. As a general procedure, the FIS QA consisted of examining

the maximum, minimum, average, and standard deviation for each

numerical field in the data table. An attempt was made to find an

explanation for unexpected high or low values, values outside of

the normal physical range for a variable, or standard deviations

that appeared inconsistent with the mean. In some cases, histog-

rams were examined to determine whether outliers were consistent

with the shape of the data distribution.

The discrepancies, that were identified, are reported as problems

in Section 11.1.

11.  NOTES

11.1  Known Problems With The Data.

None.

11.2  Usage Guidance.

This data set can be used in conjunction with other soil moisture data to

validate the soil moisture values predicted by the airborne remote

sensing instruments during FIFE (i.e., Peck Airborne Gamma Ray Soil

Moisture and FIFE Level-3 Pushbroom Microwave Radiometer (PBMR)

Soil Moisture Imagery).

11.3  Other Relevant Information.

None.

12.  REFERENCES

12.1  Satellite/Instrument/Data Processing Documentation.

Gardner, W.H. 1986. Water content. pp.635-662. In: A. Klute (ed.) Methods

of Soil Analysis. Part 1. Physical and mineralogical methods. 2nd ed.

Agronomy Monogr. 9. ASA and SSSA. Madison, WI.

12.2  Journal Articles and Study Reports.

Carroll, T.R. 1981. Airborne soil moisture measurement using natural

terrestrial gamma radiation. Soil Science. 132:358-366.

Carroll, T.R., and E.L. Peck. 1988. Airborne time-series measurements of

soil moisture using terrestrial gamma radiation. Proc. Am. Congr. Sur.

Map. and Am. Soc. for Photogram. &  Rem. Sens. St. Louis, MO.

Engman, E.T., W. Kustas, T.J. Schmugge, and J.R. Wang. 1987. Relationship

among the remotely sensed soil moisture, streamflow, and evapotrans-

piration. AGU Fall Meeting. San Francisco.

Engman, E.T., G. Angus, and W. Kustas. 1989. Relationships between the

hydrologic balance of a small watershed and remotely sensed soil

moisture. Remote Sensing and Large Scale Processes. IAHS Publ. No.

186. Proc. IAHS. 3rd Int. Asso. Baltimore, MD.

Engman, E.T., J.R. Wang. 1989. Remotely Sensed Soil Moisture Input to

a Hydrologic Model. Proc. IGARSS. 89. 12th Canadian Symposium or

Remote Sensing July 10-14. Vancouver, Canada. 4:2150-2153.

Engman, E.T., W.P. Kustas, J.R. Wang. 1990. Remotely Sensed Soil

Moisture Input to a Hydrological Model for Simulating the Water

Balance of a Small Basin. Regionalization in Hydrology. 2:4-16.

M.A. Beran, M. Brilly, A. Becker and O. Bonacci (Eds.).

Gogineni, S. 1990. Radar measurements of soil moisture over the Konza

prairie. AMS Symposium on the First ISLSCP Field Expt. Anaheim, CA.

Peck, E.L., T.R. Carroll, and D.M. Lipinski. 1990. Airborne gamma radiat-

ion soil moisture measurements over short flight lines. AMS Symposium

on the First ISLSCP Field Expt. Anaheim, CA.

Savabi, M.R., E.T. Engman, W.P. Kustas, and W.J. Rawls. 1989. Evaluation

of Wepp Water Balance Model for Watershed 1D in the Konza Prairie,

Kansas. 19th Conference Agricultural and Forest Meteorology and 9th

Conference Biometeorology and Aerdrology. AMS. Charleston, SC.

pp.147-150.

Schmugge,T.J. 1978. Remote sensing of surface soil moisture. J. Appl.

Meteorol. 17:1547-1557.

Schmugge, T.J., J.M. Meneely, A. Rango, and R. Neff. 1977. Satellite

microwave observations of soil moisture variations. Bull. of Water

Resources. 13:265.

Schmugge, T.J., T.J. Jackson, and H.L. McKim. 1980. Survey of methods of

soil moisture determination. Water Resources Res. 16(16):961-979.

Wang, J.R., J.C. Shiue, E.T. Engman, and T.J. Schmugge. 1988. The soil

moisture variations of two small watersheds in Konza prairie as esti-

mated from the L-band radiometric measurements. Geophys. Res. Lett.

(in review).

Wang, J.R., and J.C. Shieu. 1989. Remote sensing of soil moisture variat-

ions with PBMR. AGU EOS. 70(issue 15). No. 347.

Wang, J.R., T.J. Schmugge, J.C. Shiue, E.T. Engman. 1989. Mapping

Surface Soil Moisture with L-Band Radiometric Measurements.

Remote Sens. Environ. 27:305-312.

Wang, J.R., J.C. Shiue, T.J. Schmugge, E.T. Engman. 1989. Results of

PBMR Flights During the Four Intensive Field Campaigns of FIFE.

MicrowaveI Radiomet. Remote Sens. Appl. 29-38 P. Pampaloni (Ed.)

Wetzel, P.J., and J.T. Chang. 1987. Concerning the relationship between

evapotranspiration and soil moisture. J. Clim. Appl. Meteor.

12.3  Archive/DBMS Usage Documentation.

The Collected Data of the First ISLSCP Field Experiment is archived at

the EOS Distributed Active Archive Center (DAAC) at Oak Ridge National

Laboratory (ORNL), Oak Ridge, Tennessee (Section 13 below).

Documentation about using the archive and/or online access to the data

at the ORNL DAAC is not available at this revision.

13.  DATA ACCESS

13.1  Contacts for Archive/Data Access Information.

ORNL DAAC User Services

P.O. Box 2008

Mail Stop 6490

Oak Ridge National Laboratory

Oak Ridge, TN 37831-6490

(615) 241-3952

13.2  Archive Identification.

EOSDIS Distributed Active Archive Center

P.O. Box 2008

Mail Stop 6490

Oak Ridge National Laboratory

Oak Ridge, TN 37831-6490

USA

Telephone:       (615) 241-3952

FAX:             (615) 574-4665

Internet Email:   ornldaac@onrl.gov

13.3  Procedures for Obtaining Data.

Users may place requests by letter, telephone, electronic mail, FAX, or

personal visit.

13.4  ORNL DAAC Status/Plans.

FIFE data will be available from the ORNL DAAC. Please contact the

ORNL DAAC User Services Office for the most current information about

these data.

14.  OUTPUT PRODUCTS AND AVAILABILITY

14.1  Tape Products.

None.

14.2  Film Products.

None.

14.3  Other Products.

The Soil Moisture Transect data are available on FIFE CD-ROM Volume 1.

15.  GLOSSARY OF ACRONYMS

CD-ROM            Compact Disk-Read Only Memory

CMP               Coordinated Mission Plan

DAAC              Distributed Active Archive Center

EOSDIS            Earth Observing System Data and Information System

FIFE              First ISLSCP Field Experiment

FIS               FIFE Information System

GMT               Greenwich Mean Time

GSFC              Goddard Space Flight Center

IFC               Intensive Field Campaign

ISLSCP            International Satellite Land Surface Climatology Project

ORNL              Oak Ridge National Laboratory

PBMR              Pushbroom Microwave Radiometer

UTM               Universal Transverse Mercator